SCHEDULE OF QUANTITIES

Name of work :	 Construction of Commonwealth Games Village .

Sub Head  :        	Providing and fixing barbed Wire fencing of land near Railway Line .

| S.No . | Description of Item | Qty . | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Earth work in excavation in foundation trenches or drain (not exceeding 1.5 m in width or 10 sqm. on plan ) including dressing of sides and ramming of bottoms, lift upto 1.5m including getting out the excavated soil and disposal of surplus excavated soil as directed within a lead of 50m . (a) All kinds of soil . | 102.00 | Cum . |  |  |
| 2. | Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering :- All work upto plinth level . a) 1:3:6 (1 cement : 3 coarse sand : 6 graded stone aggregate 20mm nominal size ) . | 82.00 | Cum . |  |  |
| 3. | Supplying at site 1.8m long R.C.C. standard posts with wooden plugs or 6mm bars nibs . | 484 Nos . | Each |  |  |
| 4. | Supplying at site R.C.C. standards struts 2.0 metre long . | 121 Nos . | Each |  |  |
| 5. | 1.2m high fencing with 1.8m R.C.C design placed every 3m apart, embedded in cement concrete blocks, every 15th post, last but one end post and corner post shall be strutted on both sides and end post one side only, provided with nine horizontal lines and two diagonals of barbed wire 9.38 Kg. Per 100 metres (min.) between the two posts fitted and fixed with G.I. staples on wooden plugs or G.I. binding wire tied to 6mm bar nibs fixed while casting the posts (cost of R.C.C. posts, struts , earth work and concrete to be paid separately ) . a) With G.I. barbed wire . | 1450.00 M | Metre |  |  |

Material :  Cement 18.04 MT @4600/- ( Rs. Four thousand six hundred only)

per MT will be issued from any DDA store at Delhi / New Delhi

Executive Engineer

CWG-Divn. No.I/ DDA

# SCHEDULE OF QUANTITIES

Name of work :-   		 Construction of  Common Wealth Games Village .

SH :        	 	P/L G.I. Pipes line  for Horticulture purpose along Akshar Dham Temple .

| S.No . | Description of Item | Qty . | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Cleaning Jungle including uprooting of rank vegetation, grass , brush , wood , trees and sapling of girth upto 30 cm measured at a height of 1m above ground level and removal of rubbish upto a distance of 50m outside the periphery of the area cleared. | 458 M2 | 100M2 |  |  |
| 2. | F/F G .I . pipes complete with G .I. fitting including trenching and refilling etc . (external work ) . - 25mm dia nominal bore - 40mm dia nominal bore | 156 M 2740 M | Mtr . Mtr . |  |  |
| 3. | P/F gun metal gate valve with C .I . wheel of approved quality (Screwed end ). - 25mm nominal bore - 40mm nominal bore | 4 Nos . 55 Nos . | Each Each |  |  |
| 4. | Painting G .I . pipes and fitting with two coats of anticorrosive bitumastic paint of approved quality |  |  |  |  |
| a) | 25mm dia meter pipe | 156 M | Mtr . |  |  |
| b) | 40mm dia meter pipe | 2740 M | Mtr . |  |  |
| 5 . | Constructing masonary chamber 30 X30 x50 cm . inside with 75 class designation brick work in cement mortar 1:5 (1 Cement : 5 fine sand ) for stop cock , with C.I. surface box 100x100x75mm (inside ) with hinged cover fixed in cement concrete slab 1:2:4 mix (1 cement : 2 C . Sand : 4 Graded stone aggregate 20mm nominal size ) necessary excavation foundation concrete 1: 5 :10 (1 Cement :5 fine sand : 10 graded stone aggregate 40mm nominal size ) & inside plastering with cement mortar 1:3 (1 cement : 3 Coarse sand ) . 12mm thick finished with a floating coat of neat cement complete as per standard design . a) with F . P . S . bricks . | 59 Nos . | Each |  |  |
| 6. (a) (b) | P /F sand of grading Zone-V or coaser grade alround the G .I .pipes in external work . 25mm dia pipe . 40mm dia pipe. | 120 M 2740 M | Mtr . Mtr . |  |  |
| 7. | Dismentling including stacking of serviceable material and disposal of unserviceable material within 50m lead. a) Bituminous road | 0.50 M 2 | Sqm . |  |  |
| 8. | P/F G.I. pipes complete with G.I fittings (external work) including trenching and refilling etc. - 40 mm dia nominal bore. (i) Deduct for trenching and refilling not required/done. | 40.50m | Mtr. |  |  |

Material :  Cement 1.53MT @4600/- ( Rs. Four thousand six hundred only)

per MT will be issued from any DDA store at Delhi / New Delhi

Executive Engineer

CWG-Divn. No.I/ DDA

SCHEDULE OF QUANTITIES

Name of work: 	Construction of Commonwealth Games Village.

SH: 	Demarcation of layout at site to facilitate handing over of site of utilities to DJB, BSES, Project Developer & 				Police etc.

| S.NO. | Description of item | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Demarcation of various proposed structures of Commonwealth Games Village site such as residential block, roads, swimming pool, indoor hall, transport mall, WTP, STP etc. all complete as per approved master/concept plan and as directed by Engineer-in-charge with total station surveying instrument. | 146.42 Acre | Acre |  |  |
| 2. | Providing and fixing 15x15x60 cm. boundary stone of hard stone with top 30 cm. Chiesel dressed on all four sides and on top (cost of excavation, refilling and concrete etc. to be paid separately). | 1500 Nos. | Each |  |  |
| 3. | Earth work in excavation in foundation trenches or drains (not exceeding 1.5m in width or 10 sqm. on plan) including dressing of sides and ramming of bottoms, lift upto 1.5m i/c getting out the excavated soil and disposal of surplus excavated soil as directed with-in a lead of 50m. a) All kinds of soil. | 18.00 Cum. | Cum. |  |  |
| 4. | Filling available excavated earth (excluding rock) in trenches plinth sides of foundations etc. in layers not exceeding 20 cm. in depth consolidating each deposited layer by ramming and watering, lead upto 50m. and lift upto 1.5m. | 7.87 Cum | Cum. |  |  |
| 5. | Painting with synthetic enamel paint of approved brand and manufacture to give an even shade. i) Two or more coats on new work. | 303.75 Sqm. | Sqm. |  |  |

Executive Engineer

CWG-Divn. No.I/ DDA