Estimated Cost : Rs. 29590/-	Date of sale : 22-06-2007

Earnest Money : Rs.  592/-		Date of Opening : 26-06-2007

SCHEDULE OF WORK

| Name of work | : | M/o Office Building Vikas Minar – R/M of Pumps, wet riser and Sprinkler System (SH : Repairing of water supply and dewatering pumps) |
| --- | --- | --- |

| S.NO. | DESCRIPTION OF ITEM |  | QTY. | UNIT | RATE | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Repairing of clear water centrifugal pump set no.4 coupled with 25 HP motor Kirloskar make model DB 50 / 26 including supplying and replacement of following parts including providing rubber packing and gland dori, dismentling from foundation, reinstallation, commissioning, testing etc. complete as required. |  |  |  |  |  |
|  | i) | Shaft SS – 1 No. |  |  |  |  |
|  | ii) | Ball Bearing 6309 – 2 Nos. |  |  |  |  |
|  | iii) | Neck Ring – 2 Nos. |  |  |  |  |
|  | iv) | Sleeve – 2 Nos. |  |  |  |  |
|  | v) | Coupling nut – 1 No. |  |  |  |  |
|  | vi) | Impeller nut – 1 No. |  |  |  |  |
|  | vii) | Turning after remetalling of impeller – 1 Job |  |  |  |  |
|  | viii) | Studs – 8 Nos. | 1 Job | Job |  |  |
| 2. | Repairing of clear water centrifugal pump set no. 2 Kirloskar make model DSM1 coupled with 10 HP motor including supplying and replacement of following parts including providing rubber packing and gland dori, dismentlling from foundation, reinstallation, testing, commissioning etc. complete as required. |  |  |  |  |  |
|  | i) | Shaft SS – 1 No. |  |  |  |  |
|  | ii) | Sleeve – 2 Nos. |  |  |  |  |
|  | iii) | Ball Bearing 6304 – 2 Nos. |  |  |  |  |
|  | iv) | Neck Ring – 2 Nos. |  |  |  |  |
|  | v) | Coupling nut – 1 No. |  |  |  |  |
|  | vi) | Studs – 9 Nos. |  |  |  |  |
|  | vii) | Turning after remetalling of impeller – 1 Job | 1 Job | Job |  |  |
| 3. | Repairing and rewinding of 1.5 KW three phase mody make portable dewatering submersible pump set no. 1 including supplying and replacement of following parts, dismentling, reassembling, commissioning, testing etc. complete as required. |  |  |  |  |  |
|  | i) | Rewinding kit – 1 Set |  |  |  |  |
|  | ii) | Mechanical Seal – 2 Set |  |  |  |  |
|  | iii) | ‘O’ Ring Set – 1 Set |  |  |  |  |
|  | iv) | Ball bearing 6302 – 2 Nos. |  |  |  |  |
|  | v) | Oil indo – 48 |  |  |  |  |
|  | vi) | Cable grommet – 1No. | 1 Job | Job |  |  |

ADDITIONAL CONDITIONS

1.	The work shall be carried out to the satisfaction of Engineer-incharge.

2.	No T&P shall be issued to contractor by the department.

3.	Ball bearing shall be of SKF/NBC make only.

4.	Guarantee letter for the six month for the smooth operation of pump will be submitted by the contractor and also for satisfactory performance for the work.

5.	The rates quoted shall be inclusive of all taxes and duties etc. Nothing extra shall be paid on this account to agency by the department.

6.	Dismentled material shall be returned back to the department.

7.	Nothing extra on account of cartage shall be paid to the agency by the department.

8.	Parts to be replaced shall be of same make as provided originally at site as per makes in the item.