OFFICE OF THE EXECUTIVE ENGINEER

NORTHERN DIVISION NO.11 DDA,

ASHOK VIHAR, PH-I, DELHI-110052

Date of Sale	: 31-07-07

Date of Opening	: 04-08-07

Tendered Cost	: 500/-

Estimated Cost	: Rs. 6,94,740.00

Earnest Money	: Rs. 13,889.00

Time Allowed	: Eighteen Months

SCHEDULE OF QUANTITY

Name of work:	D/o Land block C&D, Shalimar Bagh.

Sub-Head:	Appointment of Consultant for designing and planning of sewerage pumping station. Sump well & rising main i/c electrical and mechanical works/ installation at Block C&D, Shalimar Bagh.

| S. No. | Description of Item | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Appointment of Consultant for designing and planning of sewerage pumping station. Sump well & rising main i/c electrical and mechanical works/ installation at Block C&D, Shalimar Bagh. | 1 Job | Per Job |  |  |

Executive Engineer

ND-11/DDA

Delhi

OFFICE OF THE EXECUTIVE ENGINEER

NORTHERN DIVISION NO.11 DDA,

ASHOK VIHAR, PH-I, DELHI-110052

Date of Sale	: 31-07-07

Date of Opening	: 04-08-07

Tender cost	: Rs. 500/-

Estimated Cost	: Rs. 3,77,791.00

Earnest Money	: Rs. 7,556.00

Time Allowed	: Sixty days

SCHEDULE OF QUANTITY

Name of work:	C/o 70 Nos. Two Rooms Apartments including internal development in Block-BH(Poorvi), Shalimar Bagh.

Sub-Head:		C/o Pump house at 70 Nos. Two Room Apartments in Block BH (Poorvi), Shalimar Bagh.

| S. No. | Description of Item | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Earth work in excavation over areas (exceeding 30 cm in depth, 1.5m in width as well as 10sqm. On plan) i/c disposal of excavated earth, lead upto 50m and lift upto 1.5m, disposed earth to be leveled and neatly dressed. - All kinds of soil. | 104.89 | Cu.m. |  |  |
| 2. | Extra for every additional lift of 1.5m or part thereof in: a) All kinds of soil. | 25.28 | Cu.m. |  |  |
| 3. | Providing and laying in position cement concrete of specified grade excluding the cost of centring and shuttering – all work upto plinth level:- a) 1:5:10 (1 cement : 5 fine sand : 10 graded stone aggregate 40mm nominal size). | 8.17 | Cu.m. |  |  |
| 4. | Providing & laying in position machine batched, machine mixed and machine vibrated design mix cement concrete of specified grade for reinforced cement concrete structural elements, excluding the cost of centring, shuttering, finishing and reinforcement, including Admixtures in recommended proportions (as per IS: 9103) to accelerate, retard settling of concrete improve workability without impairing strength and durability as per direction of Engineer-in-Charge – M-20 grade reinforced cement concrete. - All work upto plinth level. - All work from plinth level upto floor one level - Beams, plinth beams, girders, bresssumers, cantilevers, suspended floors, lintels roofs and staircases including spiral staircases, shelves etc. - Walls, columns, pillars, posts and struts. | 22.40 10.64 1.62 | Cu.m. Cu.m. Cu.m. |  |  |
| 5. | Providing and laying demp-proof course 40mm thick with cement concrete 1:2:4 (1 cement: 2 Coarse Sand : 4 Graded stone Aggregate 12.5mm nominal size). | 10.50 | Sq.m. |  |  |
| 6. | Applying a coat of residual petroleum bitumen of penetration 80/100 of approved quality using 1.7 Kg. per sqm. On damp proof course after cleaning the surface with brushes and finally with a piece of cloth lightly soaked in kerosene oil. | 10.50 | Sq.m. |  |  |
| 7. | Centring & shuttering i/c strutting, propping etc. and removal of form for :- - Foundations, footings, bases of columns etc. for mass concrete. - Columns, pillars, piers, abutments, posts and struts. - Stairs (excluding landings) except spiral – staircases. - Lintels, beams, plinth beams, girders, bressumers and cantilevers. - Suspended floors, roofs, landings, balconies and access platform. - Walls (any thickness) including attached pilasters, butteresses, plinth and string courses etc. | 10.12 40.05 4.17 46.14 49.97 82.32 | Sq.m. Sq.m. Sq.m. Sq.m. Sq.m. Sq.m. |  |  |
| 8. | Brick work with F.P.S. bricks of class designation 75 in foundation and plinth in:- - Cement Mortar 1:6 (1 Cement : 6 Coarse Sand) | 8.53 | Cu.m. |  |  |
| 9. | Brick work with F.P.S. Bricks of class designation 75 in super structure in cement mortar 1:6 (1 Cement: 6 Coarse Sand) a) from Plinth upto floor one level | 22.87 | Cu.m. |  |  |
| 10. | 6mm Cement plaster to ceiling of mix 1:3 (1 Cement : 3 fine sand) | 49.88 | Sq.m. |  |  |
| 11. | 15mm cement plaster on the rough side of single or half brick wall of mix 1:6 (1 Cement : 6 fine sand) | 105.08 | Sq.m. |  |  |
| 12 | 12mm Cement Plaster of Mix 1:6 (1 Cement: 6 Fine Sand) | 206.41 | Sq.m. |  |  |
| 13. | Providing & fixing T-iron frame for doors, windows and ventilators of mild steel tee-sections, joints mitred and welded with 15x3mm lugs including 10cms long embedded in cement concrete blocks 15 x 10x10cm of 1:3:6 (1 Cement: 3 Coarse sand : 6 graded stone aggregate 20mm nominal size) or with wooden plugs and screws or rawl plugs and screws or with fixing clips or with bolts and nuts as required i/c fixing of necessary butt hinges and screws and applying a priming coat of approved steel primer. | 389.40 | Kg. |  |  |
| 14. | Steel work welded in built up sections/framed work including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer using structural steel etc., as required. - In gratings, frames, guard bar, ladders, railings, brackets, gates & similar works. | 366.63 | Kg. |  |  |
| 15. | Providing and fixing ISI marked steel glazed doors, windows and ventilators of standard rolled steel sections, joints mitred and welded with 15x3mm lugs 10cm long, embedded in cement concrete blocks 15 x 10x10cm of 1:3:6 (1 Cement: 3 Coarse sand : 6 graded stone aggregate 20mm nominal size) or with wooden plugs and screws or rawl plugs and screws or with fixing clips or with bolts and nuts as required i/c providing and fixing of glass panes with glazing clips and special metal sash putty of approved make complete including applying a priming coat of approved steel primer, excluding the cost of metal beading and other fitting except necessary hinges or pivots as required. - Window-side hung | 2.70 | Sq.m. |  |  |
| 16. | Cement concrete flooring 1:2:4 (1 Cement : 2 Coarse Sand : 4 Graded Stone Aggregate) finished with a floating coat of neat cement including cement slurry, but excluding the cost of nosing of steps etc. complete. - 40mm thick with 20mm nominal size stone aggregate. | 32.64 | Sq.m. |  |  |
| 17. | Providing and fixing glass strips in joints of terrazzo / cement concrete floors. - 40mm wide and 4mm thick | 120.00 | Mtr. |  |  |
| 18. | Cement plaster skirting (upto 30cm height) with cement mortar 1:3 (1 Cement : 3 Coarse Sand) finished with a floating coat of neat cement: - 18mm thick - 21mm thick | 0.37 3.23 | Sq.m. Sq.m. |  |  |
| 19. | Supplying & fixing rolling shutters of approved make, made of required size M.S. laths interlocked together through their entire length and jointed together at the end by end locks mounted on specially designed pipe shaft with brackets, side guides and arrangements for inside and outside locking with push and pull operation complete including the cost of providing and fixing necessary 27.5cm long wire springs grade No.2 and M.S. top cover of required thickness for rolling shutters. - 80 x 1.20mm M.S. Laths with 1.20mm thick top cover. | 4.60 | Sq.m. |  |  |
| 20. | Providing and fixing ball bearing for Rolling Shutters. | 4.00 | Each |  |  |
| 21. | Providing and fixing ISI marked flush doors shutters non-decorative type, core of block board construction with frame of 1st class hard wood and well matched commercial 3 ply veneering with vertical grains or cross bands and face veneers on both faces of shutters:- - 35mm thick including ISI marked Stainless Steel butt hinges with necessary screws. | 1.89 | Sq.m. |  |  |
| 22. | Providing and fixing ISI marked oxidized M.S. sliding door bolts with nuts and screws etc. complete:- 300 x 16mm | 4 | Each |  |  |
| 23. | Providing and fixing ISI marked oxidized M.S. tower Bolt black finish (Barrel type) with necessary screws etc. complete. 250 x 10mm | 10 | Each |  |  |
| 24. | Providing & fixing ISI marked 85mm x 42mm oxidized M.S. pull bolt lock conforming to IS: 7534 with necessary screws, bolts, nut and washers etc. complete. | 3 | Each |  |  |
| 25. | Providing and fixing ISI marked oxidized M.S. handles conforming to IS: 4992 with necessary screws etc. complete 125mm | 2 | Each |  |  |
| 26. | Providing & fixing on wall face unplasticised – Rigid PVC rain water pipes conforming to IS: 13592 type A including jointing with seal ring conforming to IS:5382 leaving 10mm gap for thermal expansion:- - Single Socketed pipes (110mm diameter) | 7.20 | Mtr. |  |  |
| 27. | Providing & fixing on wall face unplasticised – PVC moulded fittings / accessories for unplasticised – Rigid PVC rain water pipes conforming to IS:13592 Type A including jointing with seal ring conforming to IS: 5382 leaving 10mm gap for thermal expansion. - Bend 87.5o (110mm bend) | 4 | Each |  |  |
| 28. | Reinforcement for RCC work including straightening, cutting, bending, placing in position and binding all complete. - Thermo-mechanical treated bars. | 3900 | Kg. |  |  |
| 29. | Providing & fixing oxidized M.S. casement stays (straight peg type) with necessary screws etc. complete. - 300mm weighing not less than 200gms. | 6 | Each |  |  |
| 30. | 10cm thick (average) mud phaska of damped brick earth on roofs laid to slope consolidated and plastered with 25mm thick mud mortar mixed with bhusa at 35kg. per cum of earth and gobri leaping with mix 1:1 (1 clay : 1 cow dung) and covered with flat tile bricks of class designation 100 grouted with cement mortar 1:3 (1 Cement : 3 Fine sand) mixed with 2% of integral water proofing compound by weight of cement and finished neat:- - With F.P.S. Brick tiles | 32.64 | Sq.m. |  |  |
| 31. | Making Khurras 45 x 45cm with average minimum thickness of 5cm cement concrete 1:2:4 (1 Cement : 2 Coarse sand : 4 Graded stone aggregate of 20mm nominal size) over PVC sheet 1m x 1m x 400micron, finished with 12mm cement plaster 1:3 (1 Cement : 3 coarse sand) and a coat of neat cement rounding the edges and making and finishing the outlet complete. | 2 | Each |  |  |
| 32. | Providing Gola 75 x 75cm in cement concrete 1:2:4 (1 cement : 2 Coarse Sand : 4 Stone Aggregate 10mm and down gauge) including finishing with cement mortar 1:3 (1 Cement : 3 fine sand) as per standard design. - In 75 x 75mm deep chase. | 32.32 | Mtr. |  |  |
| 33. | White washing with lime to give an even shade. - New work (three or more coats) | 163.43 | Sq.m. |  |  |
| 34. | Finishing walls with water proofing cement paint ‘Snowcem-Plus’ of M/s. Snowcem India Ltd., or equivalent of required shade:- - New work (two or more coats applied @ 3.84 kg/10m2) | 170.02 | Sq.m. |  |  |
| 35. | Painting top of roofs with bitumen of approved quality at 17 Kg. per 10 sq.m. Impregnated with a coat of coarse sand at 60 cud m per 10 sqm. including cleaning the slab surface with brushes and finally with a piece of cloth lightly soaked in kerosene oil complete. - With residual type petroleum bitumen of penetration 80/100. | 32.64 | Sqm. |  |  |
| 36. | Painting with synthetic enamel paint of approved brand and manufacture to give an even shade: - Two or more coats on new work. | 92.82 | Sq.m. |  |  |
| 37. | Applying priming coat with ready mixed red oxide zinc chromate primer of approved brand and manufacturing on steel galvanized iron/steel works. | 83.76 | Sq.m. |  |  |
| 38. | Filling available excavated earth (excluding rock) in trenches, plinth, sides of foundations etc. in layers not exceeding 20cm in depth, consolidating each deposited layer by ramming and watering, lead upto 50m and lift upto 1.5m. | 29.28 | Cum. |  |  |
| 39. | Providing and laying S&S centrifugally cast (spun) iron pipes (Class LA) conforming to IS: 1536. - 100mm dia pipe | 65.00 | Mtr. |  |  |
| 40. | Providing and laying S&S CI standard specials such as tees, bends, collars, tapers, caps etc. (heavy class).Upto 300mm dia. | 0.78 | Qtl. |  |  |
| 41. | Providing & laying flanged CI standard specials such as tees, bends, collars, tapers, caps etc. suitable for flanged jointing as per IS: 1538. - Upto 300mm dia. | 0.68 | Qtl. |  |  |
| 42. | Providing & fixing CI sluice valves (with cap) ISI marked complete with bolts, nuts, rubber, insertions etc. (the tail pieces if required will be paid separately). - 100mm diameter (Class-I) | 2 | Each |  |  |
| 43. | Supplying pig lead at site of work. | 0.25 | Qtl. |  |  |
| 44. | Constructing masonry Chamber 60x60x75cm, inside with 75 class designation brick work in cement mortar 1:5(1 Cement : 5 Fine Sand) for sluice valve, with C.I. surface box 100mm top diameter, 160mm bottom diameter and 180mm deep (inside) with chained lid and RCC top slab 1:2:4 mix (1 Cement : 2 Coarse sand : 4 Graded stone aggregate 20mm nominal size) necessary excavation foundation concrete 1:5:10 (1 Cement : 5 Fine sand : 10 graded stone aggregate 40mm nominal size) and inside plastering with cement mortar 1:3 (1 Cement : 3 coarse sand) 12mm thick finished with a floating coat of neat cement complete as per standard design. - With F.P.S. bricks. | 2 | Each |  |  |
| 45. | Carriage of surplus excavated earth by mechanical transport i/c loading, unloading with in a lead of 3km. | 76.00 | Cum. |  |  |
| 46. | Providing lead caulked joints to spun iron or C.I. pipes and specials including testing of joints but excluding the cost of pig lead. - 100mm diameter pipe | 10 | Each |  |  |
| 47. | Making plinth protection 50mm thick of cement concrete 1:3:6 (1 Cement : 3 Coarse sand : 6 graded stone aggregate 20mm nominal size) over 175mm bed of dry brick ballast 40mm nominal size well rammed and consolidated and grouted with fine sand including finishing the top smooth. | 12.96 | Sqm. |  |  |

Executive Engineer

ND-11/DDA

Delhi

OFFICE OF THE EXECUTIVE ENGINEER

NORTHERN DIVISION NO.11 DDA,

ASHOK VIHAR, PH-I, DELHI-110052

Date of Sale	: 04-08-07

Date of Opening	: 08-08-07

Tender cost	: Rs. 500/-

Estimated Cost	: Rs. 13,15,828.00

Earnest Money	: Rs. 26,317.00

Time Allowed	: Three months

SCHEDULE OF QUANTITY

Name of work:	D/o Land at Block C&D Shalimar Bagh.

Sub-Head:		Restoration of Road & Footpath (damaged by NDPL).

| S. No. | Description of Item | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Surface dressing of the ground including removing vegetation and in-equalities not exceeding 15cm deep and disposal of rubbish, lead upto 50m and lift upto 1.5m (a) all kinds of soil. | 2006.00 | Sq.m. |  |  |
| 2. | Providing and laying in position cement concrete of specified grade excluding the cost of centring and shuttering all work upto plinth level. a) 1:5:10 (1 Cement : 5 Fine Sand : 10 Graded stone aggregate 40mm nominal size) | 150.00 | Cu.m. |  |  |
| 3. | Making good the existing road damaged during laying electric cable including supply of extra quantities of materials i.e. aggregate, moorum, screening, red bajri and labour required. a) Bituminous road | 117.00 | Cu.m. |  |  |
| 4. | Providing high strength interlocking paver block 100mm thick for heavy traffic density of approved design and manufacture with M-40 grade cement concrete including setting in position over 30mm thick bed in coarse sand and filling of joints with Jamuna Sand including leveling, cutting of paver block wherever necessary and leveling etc. (rate include the cost of interlocking paver block, cost of transportation, labour, 30mm thick coarse sand, bed, refilling joints with Jamuna sand, wastage, breakage, cutting, supervision, fixing, composition with plates vibrator complete) as per direction of Engineer-in-Charge and to match the existing paver blocks. | 2006.00 | Sqm. |  |  |

Executive Engineer

ND-11/DDA

Delhi