DRAFT

ZONAL DEVELOPMENT PLAN FOR ZONE ‘N’ (NORTH WEST DELHI-III)

ROHINI PROJECT (PLANNING)

DELHI DEVELOPMENT AUTHORITY

CONTENTS

1.	PREAMBLE								1

2.	INTRODUCTION								1

3.	LOCATION/BOUNDARIES & AREA				1-2

4.	POPULATION, EMPLOYMENT& WORKFORCE		2

5	EXISTING PROFILE							2

6.	PHYSICAL CHARACTERSTICS					3

7.	CONCEPT									3

8.	 PROPOSED LANDUSE							5

9.	RE-DEVELOPMENT OF VILLAGES				9

10.	INFLUENCE ZONE							10

- PREAMBLE

In accordance with Authority’s Resolution in its meeting held on 6.9.07, genuine pre-existing institutions, i.e. before 1.1.2006, rendering cultural, religious(including spiritual) healthcare and educational services to the people but which do not form part of the ridge or Gram Sabha or public land, shall be incorporated vis-a-viz their current land uses in the respective zonal plans keeping in views clauses 3 &4 of the Master Plan-2021.  Such institutions will, however, be subject to reasonable policy and procedural stipulations regarding factual verification, FAR, development charges, land use, etc.

- INTRODUCTION:

2.1	The National Capital Territory of Delhi is divided into 15 zones as 	per the Master Plan of Delhi 2021. Out of these  A to H, 	P, M & K1 	are in  Urban  Delhi and  J,K2,L,N & P2 fall in Urbanisable  	Area/urban Extension. Zone O covers the area of river 	Yamuna(River Front Area) As part of the urbanisable area, the MPD-	2021 stipulates that the 77 lacs population is to be accommodated in 	new planned areas at a city level density of 250-300 pph for which 	lands needs to be extended to about the range of 20,000-	22,000hact.This include the existing population of about 29 lacs in 	villages and unauthorized area /colonies

2.2	Zone ‘N mainly comprises of the proposed Urban Extn./green belt  and forms part of urbanisable area under the Master Plan of  Delhi (MPD-2021).  The zone is situated on the North West part of Delhi and mainly constitutes of rural settlements and the industrial area of Bawana. It is connected by railway line  towards Rohtak.   Urban Extension Road –I is passing through this zone.

3.0	LOCATION, BOUNDARIES AND AREA:

3.1	Mainly ‘N’ is situated in the North West Delhi.  The area of the Zone is bounded by the following:

North	East		:		Western Yamuna Canal/Narela 							Project

-2-

South			:		Rohtak Railway line/Zone ‘L’

South East		:		Existing H.T Line/proposed 								100 mt wide road boundary of 								Zone ‘M’(Rohini Ph.IV,V)

West&North

West			:		National capital of	Delhi 									/Haryana State boundary

3.2	The total area of the Zonal Plan is	13975 hact.

Existing Bawana Udyog Nagar		676 hact.

Peripherial green belt			4305 hact

Net urbansiable area			8994 hact

4.0	POPULATION, EMPLOYMENT AND WORK FORCE:

4.1	As per the MPD-2021, the Urbanisable areas are proposed to be planned with in overall gross density 250-300 persons per Hact.

  - The population of this zone as per the census 2001 is 3,50,000.  The 	projected population of this zone by 2021 is about 24,17,500.  As 	per MPD-2021, the anticipated workforce participation rate is about 	39% by 2021 and thereby the workforce in this zone would be about 	9,65,250.

5.0	EXISTING PROFILE:

Break-up of major land uses as per MPD-2021 to be provided in Zonal Development Plan.

Landuse				Percentage

Residential				45-55

Commercial				4-5

Industrial				4-5

Recreational				15-20

Public and Semi Public		8-10

Circulation				10-12

-3-

6.0	PHYSICAL CHARACTERISTICS:

The location of the proposed sub-city is on the terrain which is ideally	 suited for Urban development.  The entire terrain is very well connected and has a extremely good drainage system and good soil for construction.  The entire area has a contoured topography with land gradually slopping towards the south.

7.0	Existing Development:

The significant existing developments in Zone-N are as under:

- The abadies of 29 villages and the census town of Bawana falls within the zone area.  The list of these villages which include 10 villages in peripherial green belt is given in Annexure-1.

- An important feature of this zone is Bawana Industrial area(Udyog Vihar) which is the urban area approved as part of Narela sub-city zonal development plan.

- Slum pocket with an area of 105 hact. exists at Saveda Ghevera.

- Reserved forest of 72 hact of area also exist in this zone.

8.0	CONCEPT

8.1	The concept of the Zonal plan is based on land use transport 	interface so as to provide an environment friendly, safe, comfortable 	and efficient sub-city with a congenial residential environment.

8.2	Zone N is in continuation with the present zone –M(Ph.IV & V)where 	land is under acquisition.  It is anticipated that this area will be 	developed by 2021.

8.3	Facility/Infrastructure corridor is mainly proposed along the major 	roads of 45 mtrs and above ROW with 2.5 times of proposed ROW.

The suggested facilities which can be provided at district and community level are as follows:

| Use | Facilities | Agency involved |
| --- | --- | --- |
| Utilities | Land reservation for utilities like water supply, sewage, storm water drainage. | DDA, DJB,MCD |
|  | Electicity:220 kv HT line grid connecting with the northern grid with 440 kv,220 kv sub-station. 1000 MW power generation plant | Service providing agencies. |
|  | Gas pipeline: IPG, CNG pipe lines by private and public sector. | GAIL,Inderprastha Gas Agency, Indian Oil, Reliance etc. |
|  | Optical fibre network: For satellite transmission | MTNL, VSNL etc. Cellular service providers. |
| Facilities | District Centre/Community Centre service market, MRTS station Transportation terminals education and health facility | DDA DMRC GNCTD Public & Private sector |

9.0	Rural and Green Belt area:

The green belt has been identified along the NCT Delhi boundary.  The criteria for  delineation has been taken as boundaries of all the revenue villages abutting the NCTD boundary as shown in MPD-2021 landuse plan.  The policy outline for this area is as follows:

  - Low intensity development shall be permitted in this

zone to stop unauthorized development of residential 	areas, such as farm houses.

  - Govt. activities requiring large area like camping sites, firing range etc. could be located here.

  - There are 10 revenue villages in the propose green belt, covering an area 4305 hact. (Annexure-I)

10.0	Linkages:

Regional Linkage:

  - The existing MRTS Shahdara-Rithala  route can be further extended through Zone-M(Rohini Project Ph.IV & V).  In addition of this the proposed MRTS net work as part of 100 & 80 mt wide road will provide accessibility to other areas.

-5-

District Level Linkages:

  - The confluence point of the MRTS routes connecting central part of the proposed urban area to other part of zone has been proposed as the sub-city node.

  - The influence area along the proposed MRTS Shahdra-Rithala route extension in the zone has been conceived as he sub-CBD of the proposed urbanisable area.

Community level Linkages

The structure plan envisaged the proposed development at 4 levels.  The development of major transportation routes of the zone comprises the first level.  The development of the community centre at intersection point of major routes comprises the second level.  The growth the commercial zones along the transportation route and the development of residential areas within the commercial zones comprise of the third and forth levels of development respectively.

11.0 PROPOSED LAND USE

11.1	Residential

The land use plan of urban area is based on the framework of  MPD-	2021together with its existing and proposed circulation system and the          	differential residential density pattern to be followed for the various 	residential pockets.

  - Existing Settlements:

It is proposed to take-up redevelopment of existing settlements i.e. villages and regularized unauthorized colonies with provision of  various community facilities and utility services such as green, villages facilities like old age home, school/college for women etc. will be governed as per Special Area Regulations alongwith permission for buildings on applicable development control norms/building bye laws.

  - Redevelopment Projects of existing  village/settlement areas under clause 3.3.2(viii) of MPD-2021

The redevelopment plan for the villages shall be prepared by the concerned local agency with the aim of provision of optimal facilities like community hall, schools, playground within the  abadies in an integrated manner with the surrounding areas.  Mixed land use shall also be permissible as per the mixed use regulations of MPD-2021 as given for existing village abadi and unauthorized regularized colonies.

-6-

11.4   Commercial

Linear Commercial development along the MRTS radial routes an integrated Freight complex is  proposed near the proposed transport node.  Community centre shall be developed at the intersection points of 45 & 60mt wide road.

11.5  Public & Semi Public Facilities:

P.S.P. facility in this zone have been provided as facility corridors mostly along major roads as per norms.  These facility corridors includes PSP areas/facilities, commercial and recreational as per norms.

  - Existing Institutional activities/PSP facilities.

The following facilities are permissible under clause-8 (2) of MPD-2021 ( page 125, 126 of Gazette of India) as part of the approval of layout plan or as a case of special permission from the authority.

  - Hospital (upto 100 beds)

  - Primary health centre/family welfare centre/maternity home/dispensary etc.

  - Nursing home/polyclinic/Clinic/Clinical laboratory etc.

  - Dispensary for pets & animals.

  - Primary school/middle school.

  - School for mentally/physically challenged.

  - Technical training centre (ITI/Polytechnic/vocational/ training institute/Management Institute/ Teacher Institute etc.

  - Facilities- Bus terminal, taxi stand, milk/vegetable booth, religious premises, vending booth, petrol/CNG filling pumps, recreational  club, police post, police station, fire station, post office. telegraph office & telephone exchange.

11.7	Circulation

The circulation system of the subcity is based on mainly surface transport/roads.  However, it should be strengthened by MRTS as a major public transport to the urban area.  The details of which are as under:

a) Road network:	The major road network of 100mt , 80mt, 60mt and 45mt ROW.   The proposed radial roads have been taken as the

-7-

extension of the roads in Rohini Ph.IV & V in zone-M. 30M and 24M roads are also provided.

b)Freight Terminal: A freight terminal has been proposed adjacent to the proposed intra city node so that an integrated traffic circulation system is worked out considering requirement of the long distance/intercity freight movement bringing goods from outside the city and intercity movement of goods to the wholesale market and other part of the subcity.

( c)	Fuel Stations: Petrol pumps and CNG stations shall be provided as per norms and to be indicated while preparing the re-development,

local area and lay out plans.  Approved motel and petrol pumps and CNG stations sites shall continue.

- Multi Model Transport System

- Metro line/corridors would ensure high level of public transport linkages with the main city and other sub-cities. All the metro stations would have non-motorized/pedestrian linkages to make a favourable condition for the metro users.

- Public transport route would generally follow the major arterial/sub arterial roads, which have been carefully planned to make the public transport system assessable in 0.5-1km walking distance.

All  the road owing agencies shall get road development plan prepared for 30 mtr. & above R/W roads shown in MPD-2021 along with inter-change details of junctions/intersection  falling in the alignment showing location of bus stop, metro station and pedestrian linkage for smooth  change over from one mode to another.

  - Recreational

In the Zonal plan, 18% of the urbanisable area of the zone has been kept under the green/recreational activities.  This will include the city park/distt. Park and community park.  The other level of the park i.e. neighbourhood, housing area  shall be provided  in the residential area.  Beside these open spaces in the green area for multipurpose ground at city level, distt. Level & community level shall also be provided, out of the green earmarked  in the zone.  Since people are coming more conscious about the recreational activities. Sports activities, amusement park, theme park and other open spaces shall be suitably located in the zone.

-8-

Multipurpose Grounds:  A network of the multipurpose grounds, a special category for holding marriages and other special functions has been proposed.  These multi purpose ground at the community level shall be upto 2 hact. at the distt. Level of 4 hact. and the city (Zone) level shall be upto 8 hact.  Already approved  parks and farm houses on 60 mtr. roads approved by the local body/MCD for holding marriages are designated as multipurpose ground.  The area requirement  for 24 lac population proposed in this zone have been given in the Annexure-III.

11.9.	Water Bodies

Existing water bodies are proposed to be conserved within green area around.

  - Urban Design

Along the important main movement corridor i.e. MRTS corridor, major roads i.e 80 mtr. & 60 mtr. wide road, Urban design aspect shall be considered at appropriate places. Corridor of heritage buildings and the urban form exercise shall be taken up  while preparing detailed plans for the following:

- Green corridors along existing drains.

- Specialized institutes and commercial centres.

- City park complex.

- Facility corridor.

- Super tall building with Heliport/Helipad facility:

This zone is suitable for such development being far away from air funnel.  Three such complexes are proposed to be developed as a land mark points: One for PSP use zone, one for commercial use zone and one for residential use zone.  Apart from main land use 25% of FAR shall be allowed for other uses.  The  FAR shall be determined while working out the detailed schemes.

These complexes of super tall building can effectively tackle transportation and vehicular pollution in addition to releasing land for green spaces in abundance.  This will not only change the sky line of Delhi but also and facilitate international investors to develop state of the arts urban design projects.  These complexes

-9-

shall also have Helipad/Heliport facilities, which will provide direct accessibility to various cities in North India.

  - Utilities /Physical Infrastructure

The main source of water supply is the Western Yamuna Canal which is in the north eastern side of Zone-N.  The area adjoining the canal of north of the Bawana town has been ear marked for utility. Water treatment plan has also been proposed.  A power generation plant of 1000 MW is also proposed in this zone.  Two STP sites are also proposed one on North side and another on the Southern side. Sanitary landfill sites will be as identified as per MPD-2021.

- Redevelopment of villages:

As per census of 2001, there are 19 villages in the list(Annexure-II) coming under urbanisable area and 10 villages in(Annexure-I) peripheral green belt.

It is proposed to redevelop these villages by providing required facilities around the village with proper access from the major road to satisfy the need of village population  in one hand  and to also provide a buffer of facility between the existing village and the proposed development to control unauthorized construction of village extension areas.

13.0	LANDUSE BREAK-UP:

Total area of zone N				13975 hact.

Peripheral Green					4305 hact

Urbanized area					9670 hact

(a) Already built up				676 hact.

(b) Remaining area				8994 hact.

Break-up of urbanized (9670 hact) area

| Land use | Area in hact | Percentage |
| --- | --- | --- |
| Residential | 4719 | 48.8 |
| Commercial | 386.8 | 4.00 |
| Industrial | 1059.0 | 10.95 |
| Recreational | 1740.6 | 18.00 |
| PSP | 797.7 | 8.25 |
| Transportation | 967 | 10.0 |
| Total | 9670.0 ha. | 100% |

-10-

14.0	Unauthorized Colonies:

The unauthorised colonies which are existing will be governed by prevailing Govt. policies.

15.0	Influence Zone:

In MPD-2021, high rise and high density corridor has been proposed on either side of MRTS corridor to a width of about 500mt, which will be delineated in local level plans in consultation with GNCTD.

ANNEXURE-I

VILLAGES FALLING IN PERIPHERAL GREEN BELT

- AUCHANDI

- MUNGESHPUR

- QUTABGARH

- KHOR PUNJAB

- CHATESAR

- JAUNTI

- GARHI RINDHALA

- NIZAMPUR-RASHIDPUR

- TIKRIKALAN

- HAREOLI

ANNEXURE-II

VILLAGES

- BAZID PUR THAKRON

- NANGAL THAKRON

- BAWANA

- PUTH-KHURD

- SULTANPUR DABAS

- SALAHPUR MAJRA

- KATEORA

- KHORJAT

- CHANDPUR KALAN

- LADPUR

- KANJHAWALA

- KARALA

- MOHAMMADPUR MAJRI

- MADANPUR DABAS

- RANI KHERA

- RASULPUR

- GHEORA

- SAUDA

- DARYAPUR KALAN

ANNEXURE-III

LIST OF COMMERCIAL/PUBLIC/SEMI PUBLIC FACILITIES AND UTILITIES PROPOSED IN ZONAL PLAN OF ZONE-N

COMMERCIAL

1.	District Centre			4

2.	Community Centre			24

3.	Informal Bazar			24

4.	Service Market			5

5.	Sub-city wholesale market	2

UTILITY

1	ESS(220 kv)				5

2.	ESS(66 kv)				48

3.	Swerage Treatment Plant		2

4.	Sewage Pumping Station		24(2000 sqm each)

5.	Sewage Pumping Station		2(1000 sqm each)

6	 Waste Water Treatment Facility As per requirement

7	MCD	Office				2

PUBLIC, SEMI PUBLIC

1.	Hospital(A)				5

2.	Hospital(B)				9

3.	Hospital(C)				24

4.	Hospital(D)				24

5.	Maternity Home			48

6.	Nursing Home/Polyclinic		48

7.	Vaternity Hospital			5

8.	Multi purpose Community Hall	24

9.	Vocational Training Centre	5

10.	School for mentally challenged	8

11.	School for physically challenged 8

12.	Professional College(Engg.)	5

13.	Police Station			9

14.	Fire Station				14

15.	Care Centre for physically/	5

Mentally challenged.

16.	Working women hostel/Men

Hostel				5

-2-

17.	Orphange/ children centre	9

(one each)

18.	Cremation ground			5

19.	Burial ground/cemetery		2

20.	Medical College			2

21.	Telephone Exchange		2

22.	Head Post Office			2

23.	Old age home			5

24.	Religious Centre			2

25.	Science Centre 		as required

26.	District Office & Battalian	1

27.	Exhibition cum Fare Ground	1

28.	District Sport Centre		5

29.	Divisional Sports Centre		2

30.	Recreational Club			5

RECREATION

1.	 Community Park			4(5.0 hact each)

2.	Park					2(100 hact each)

3.	District Park				5

4.	Multi purpose park/ground	24(2 hact each)

5.	Multi purpose ground		5 (4 hact each)

6.	Multi purpose ground		2(8 hact each)

7.	Amusement Park			1 (10 hact each)

TRANSPORTATION

1.	Bus Terminal			24(1000 sqm each)

2.	Bus Terminal			5(0.2 hact each)

3.	Bus Depot parking space		5(0.4 hact each)

4.	Parking  sites			48

ANNEXURE-IV

List of Unauthorized Colonies to be Regulaized(Zone-N)

GNCTD/Registration

Sl.No	  No.			Name of Colony

1.		101/399		Anandpur Dham Village Karala

2.		1162/50		Prem Colony Bawana