DRAFT

ZONAL DEVELOPMENT PLAN

FOR

ZONE – “C”

(CIVIL LINES ZONE)

APPROVED BY AUTHORITY ON 30.10.2007

VIDE ITEM NO.82/2007

AREA PLANNING - II

DELHI DEVELOPMENT AUTHORITY

ZONAL  DEVELOPMENT  PLAN  FOR  ZONE ‘C’

CONTENTS

| SL. NO | CHAPTER |  | PAGE NO. |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.0 |  | Introduction | 5 |  |  |
| 1.1 |  | Background, Location, Boundaries & Areas | 5 |  |  |
| 1.2 |  | Statutory Provisions & Objectives | 5 |  |  |
| 1.3 |  | Earlier approved ZDP proposals | 6 |  |  |
| 1.4 |  | Special characteristics of the Zone | 7 | - | 10 |
| 2.0 |  | Zonal Development Plan Proposals | 10 |  |  |
| 2.1 |  | Population & Employment | 10 | - | 11 |
| 2.2 |  | Residential Development | 11 |  |  |
| 2.2.1 |  | Restructuring and upgrading/redevelopment of existing areas | 11 | - | 12 |
| 2.2.1.1 |  | Planned Areas :A.Plotted/Gr. Housing B.Employer Housing C.Bungalow Area | 12 | - | 16 |
| 2.2.2 |  | Traditional inner City & Unplanned areas:A.Special areas B. villages C. unauthorized / regularized unauthorized colonies | 16 | - | 17 |
| 2.2.2.1 |  | Facilities in Unplanned Areas | 17 | - | 18 |
| 2.2.3 |  | Mixed land use | 18 |  |  |
| 2.2.4 |  | Housing for urban poor | 18 | - | 19 |
| 2.2.5 |  | Redevelopment area along MRTS corridors | 19 |  |  |
| 2.3 |  | Work Centres | 19 |  |  |
| 2.3.1 |  | Industry | 19 | - | 21 |
| 2.3.2 |  | Trade and commerce, wholesale Trade | 21 | - | 22 |
| 2.3.3 |  | Informal trade and commerce | 23 |  |  |
| 2.3.4 |  | Government Use | 23 |  |  |
| 2.4 |  | Social Infrastructure | 24 |  |  |
| 2.4.1 |  | Health | 24 | - | 25 |
| 2.4.2 |  | Education | 25 | - | 26 |
| 2.4.3 |  | Sports facilities | 26 |  |  |
| 2.4.4 |  | Communications | 26 |  |  |
| 2.4.5 |  | Security and safety | 26 |  |  |
| 2.4.6 |  | Disaster Management Centre | 27 |  |  |
| 2.4.7 |  | Distributive | 27 |  |  |
| 2.4.8 |  | Other Community facilities | 28 | - | 29 |
| 2.5 |  | Transportation | 29 |  |  |
| 2.5.1 |  | Road Network | 29 |  |  |
| 2.5.1.1 |  | Road Development Plans | 29 |  |  |
| 2.5.2 |  | Road Network not implemented | 29 | - | 30 |
| 2.5.3 |  | Urban Relief Roads | 30 |  |  |
| 2.5.4 |  | Grade Separators | 30 | - | 31 |
| 2.5.5 |  | MRTS corridor | 31 |  |  |
| 2.5.6 |  | Integrated Multi-Modal Transport System | 31 |  |  |
| 2.5.6.1 |  | BRT Corridor | 32 |  |  |
| 2.5.7 |  | Bus Depot | 32 |  |  |
| 2.5.8 |  | Bus Terminals | 32 |  |  |
| 2.5.9 |  | Bicycle Tracks/ Cycle Rickshaw Tracks | 32 |  |  |
| 2.5.10 |  | Railway Stations | 33 |  |  |
| 2.5.11 |  | Goods Movement by Rail | 33 |  |  |
| 2.5.12 |  | Fuel Stations | 33 |  |  |
| 2.5.13 |  | Parking | 33 | - | 35 |
| 2.6 |  | Environment/Areas of Significance | 35 |  |  |
| 2.6.1 |  | Northern Ridge/ Regional Park | 35 |  |  |
| 2.6.2 |  | Multipurpose Grounds | 36 |  |  |
| 2.6.3 |  | Pollution control | 36 |  |  |
| 2.7 |  | Areas for urban design | 36 |  |  |
| 2.8 |  | Conservation of built heritage | 37 |  |  |
| 2.9 |  | Physical Infrastructure | 38 |  |  |
| 2.9.1 |  | Water Supply | 38 |  |  |
| 2.9.2 |  | Sewerage | 38 |  |  |
| 2.9.3 |  | Power | 38 |  |  |
| 2.9.4 |  | Solid Waste | 38 |  |  |
| 2.10 |  | Land Use Plan | 38 |  |  |
| 2.10.1 |  | Change of Landuse | 39 |  |  |
| 2.10.1.1 |  | CLU to be processed under Sec.11 A | 39 |  |  |
| 2.10.1.2 |  | CLU incorporated in Draft ZDP | 40 |  |  |
| 2.10.2 |  | Land Use Analysis | 40 |  |  |
| 2.10.3 |  | CLU (Not incorporated in Landuse Plan, MPD-2021) | 41 |  |  |
| 2.10.4 |  | Other Changes Incorporated in the Plan | 41 |  |  |
|  |  | Maps | 42 | - | 43 |
|  |  | Annexures 1-12 | 44 | - | 59 |
|  |  | Map | 60 |  |  |

##### PREAMBLE

“In accordance with the Authority’s Resolution in its meeting held on 6.9.2007, genuine pre-existing institutions, i.e. before 1.1.2006, rendering Cultural, religious (including spiritual) health care and educational services to the people, but which do not form part of the ridge or Gram Sabha or public  land , shall be incorporated vis-à-vis their current land uses in the respective Zonal Plans keeping in view  clauses 3 and 4 of the Master   Plan-2021.Such Institutions will, however, be subject to reasonable policy and procedural stipulations regarding factual verification, FAR, Development Charges, Land Use etc.”

1.0        ZONAL  DEVELOPMENT  PLAN  FOR ZONE ‘C’

1.1       BACKGROUND, LOCATION , BOUNDARIES AND AREAS

The NCTD has been divided into 15 planning Zones (Divisions) designated as 'A' to 'P' (except Zone 'I') in the Master Plan 2021. Zone 'C', is also known as ‘Civil Lines Zone’  and it  is located  towards  North, covering an area of 3959 hac. and consists of 21 Sub-Zones .

The boundaries of  the  Zone- C are as under:

| North-East | Part Ring Road, Outer Ring Road, Zone O (River Yamuna) N.H.-1 & P-II (North Delhi) adjoining Sub-zone C-21. |
| --- | --- |
| North-West | Zone P-I (Narela) & Zone P-II (North Delhi) |
| South-East | Old City (Zone-A) |
| South-West | Delhi Amritsar Railway Line Zone-H (North West Delhi-I) |

Zone-C is identifiable with its prestigious Delhi University, Northern Ridge, ISBT (Kashmiri Gate) and Tis Hazari Courts. A significant feature of this Zone is the Old Secretariat Complex and Civil Lines Bungalow area of the Colonial period. Roshanara Bagh and Qudsia Bagh are historical gardens from the Mughal peroid. This Zone has posh residential areas, rehabilitation colonies and pre 1962 residential colonies. A part of Special area also falls in this zone. Other important activities in the Zone are Azadpur wholesale Fruit and Vegetable market and Sanjay Gandhi Transport Nagar.  Metro Corridor is operational in this Zone. Dheerpur (Phase –I & II) is an important project in this Zone.

This zone has been developing since pre independence era and through the MPD-1962, MPD-2001 and now MPD-2021. As such the zone has heterogeneous character where the unplanned areas and planned areas developed under the norms of various plans co-exist.

1.2        STATUTORY PROVISIONS AND OBJECTIVES

1.2.1  	Already approved Sub-Zonal /earlier Zonal Plans in conformity with the Master Plan shall continue for the areas where the Zonal Plans have not been approved. The Zonal Plans in the form of structure plans shall be prepared within 12 months of the approval of MPD-2021.

1.2.2 	The Zonal (Divisional) plan is being prepared under Section-8 and processed under Section-1O of the Delhi Development Act-1957. The modification of land use shall be processed under Section-11(A), simultaneously.

1.3      EARLIER APPROVED  ZDP PROPOSALS

1.3.1   MASTER PLAN FOR DELHI-1962

As per MPD-62, the Zonal Plan for Zone-C was prepared for a population of 3,87,685 and the total area of the Zone was divided into 20 Sub-Zones (C-1 to C- 20).  The Status of approval of sub Zonal Plans as per MPD-62 is given.

(Refer Annexure –1)

1.3.2    MASTER PLAN FOR DELHI-2001

The Zonal Development Plan  had stipulated Population holding capacity of this Zone to be about  8,37,200 by the year 2001 and the total area of the Zone as 3888 Hac. A new Sub-Zone C-21 was added to the Zone. The Sub-Zone wise area is given. (Refer Annexure- 15)

MPD-2001 Stipulated that Civil Lines Bungalow Area  shall  be conserved to maintain its basic character. Part of sub-zone C-1 (Kashmiri Gate), C-4 & C-5 (Old Subzi – Mandi), C-6 (West Malka Ganj ) ,C-7 (East Malka Ganj) and C-10 (Roop Nagar- Part) have been identified as part of Special Area and therefore it shall  be  dealt as per special provisions  given in the Master Plan .

The Zonal Plan has been approved  by  Govt. of India, Ministry of Urban Affairs & Employment, Vide No. K-13011/22/93-DDIB Dated 24th Sept. 1998.

###### 1.3.3    MASTER PLAN FOR DELHI-2021

MPD-2021 has stipulated  the Population holding capacity of this Zone to be about 7,88,000 and the total area of the Zone to be  3959 Hac.

The zone has not much area for Greenfield development as this zone has already been saturated. MPD-2021 has stipulated a series of innovative concepts such as Local Area Planning , Redevelopment  of Villages, Unauthorized Colonies and Built up areas, Restructuring and up gradation of Residential Areas in the Influence Zone of MRTS  , a network of Recreational and Sports facilities to be suitably incorporated while preparing the detailed Schemes.

1.4      SPECIAL CHARACTERISTICS OF THE ZONE

Zone-C has the warmth of various cultures, Solace of its gardens and the elegance and dignity of its monuments. A part of the Special Area- Shahjahanabad, (walled City ) is located in this zone. The Roshanara garden was laid out in 1640 around the time Shahjahanabad was being built. At the historical Delhi Darbar at Coronation Pillar in 1911, King George V announced the transfer of the seat of Imperial Government  from Calcutta  to the ancient capital of  Delhi. The Old Secretariat of the Imperial Government was built in Civil lines with large and spacious residential plots  in the Civil lines Bungalow area.  Qudesia garden formed a green  buffer between the walled city and Bungalow zone. Viceroy’s palace was built on the western slopes of the ridge  which was later transferred  to the University of Delhi which was established in 1924.

Later, posh residential areas like Model Town etc.,were developed  near to the Bungalow area.. After Independence, Kingsway camp was developed as a Rehabilitation Colony  for the Pakistan  refugees  by the Rehabilitation Ministry. During emergency, few resettlement Colonies like Jahangirpuri were added to this Zone. Some of the Regularized Unauthorized Colonies like Adarsh Nagar Group of Colonies, Nirankari Colony etc. are part of this zone. A number of plotted residential colonies like Kamla  Nagar, Roop Nagar etc. came up before MPD-62.

N.H-1(G.T.Karnal Road) converges into this zone and as per MPD-62, the Old Sabzi Mandi was shifted to New Azadpur Fruit and Vegetable Market  on this Road. ISBT at Kashmere Gate is the major entry point of Delhi. Outer Ring Road  which forms the North -Eastern boundary of this zone was added  as an embankment to protect this low lying area from floods in 1982. The First phase of Underground Metro Corridor has improved the connectivity of this zone. The significant features of this Zone are:

i)	   Bungalow Area.

ii)	   Special Area,

iii)	   Old Secretariat

iv)	   Delhi University

v)	   Northern Ridge (Regional Park)

vi)	   Azad Pur Fruit  & Vegetable Wholesale Market

vii)	   ISBT,Kashmere Gate

viii)	   Dhirpur project Phase-I.

ix)	   Dhirpur project Phase-II.

x)	   Coronation Pillar

1.4.1       Bungalow Area

The Sub-Zones C-2 (Qudesia Garden/I.P. College) and C-3 (Civil lines) covering an area of around 250 Ha. form a part of Civil Lines Bungalow Area as envisaged in Master Plan. As per Mpd-2021, restructuring and up gradation of the existing areas is proposed . But  to maintain the basic Character of this area separate development norms have been detailed out in a separate section. The essential character of wide avenues, large plots, extensive landscape and low rise development ,has a heritage value which has to be conserved. Mixed use, high intensity development along MRTS corridor and de-densification of trees/reduction of green cover is not permitted at all.

In order to maintain the city’s ambience and pollution free environment in important areas of Delhi, Civil Lines Bungalow Area is categorized as ‘No Industrial Activity Zone’ where no industrial activity including household industry, shall be permitted.

Restrictions on tall buildings would be necessary in Civil Lines Bungalow Area. MRTS Influence Zone in Civil lines shall be excluded from enhancement of FAR.

1.4.2      Special Area

A part of Sub-Zone C-1 (Kashmiri Gate), Sub-Zones C-4 & C-5 (Old Subzi Mandi),C-6 (West MaIka Ganj) ,C-7 (East MaIka Ganj) and C-I0 (Jawahar Nagar-Part) have been identified as a part of special Area and therefore Redevelopment plans shall be made  as  per  the special provisions given in the Master Plan.

1.4.3     Old Secretariat

Major GNCTD offices, which were located in Old Secretariat building, have been shifted to the new premises at Indraprastha Estate. Old Secretariat is a historic building and needs to be conserved. Barracks area adjacent to the old Secretariat could be developed to accommodate additional GNCTD offices, Museum, tourist infrastructure etc. due to its location in between NH-1 and MRTS corridor.

MPD-2021 proposes optimum utilization of all Government lands with surplus lands to be utilized by government for residential development.  It also proposes utilization of 10% of total FAR for commercial uses to make the restructuring process financially feasible.

1.4.4      Delhi University

The Delhi University was established in 1924 and it has a number of old historical buildings, Colleges, Viceroy’s lodge etc. of the Colonial period and therefore efforts shall be done  to preserve this character . Also, Efforts shall be done to make it an Integrated Campus (without thoroughfare) and self sufficient in terms of modern infrastructure and residential requirements like Hostels, Staff quarters, Security arrangements etc. by optimum utilization of the land .Attempt shall be made to accommodate all institutional requirements within the Campus. Therefore an Urban Design study shall be taken up for this sub-zone. MPD-2021 has restricted this area for tall buildings.

Delhi University has a  large chunk of land allotted in Sub-zone C-15, (opposite Dushehra ground) and it is being utilized  as Hostel and Staff quarters. Optimum utilization of  this land shall be done to meet the requirements in future.

Listed Heritage buildings, some residences  and Colleges of  Historical importance prepared by DDA ,INTACH,ASI & GNCTD  are  given.(Refer  Annexure-9,10,11).

1.4.5      Northern Ridge

Northern Ridge is the rocky outcrop of Aravali Hills covering an area of 87 hac. as per notification no.F.10(42)-I /PA /DCF /93 /II dated 24.5.94 issues by GNCTD and MPD-2021.It needs to be preserved in its natural State. Metro Stations along with property development shall not be permitted in Regional Park / Ridge use Zone.

1.4.6      Azadpur  Fruit & Vegetable  market

As per MPD-62 proposals, the Fruit and Vegetable market in Subzi Mandi (C-5) was shifted to the Azadpur Mandi (C-20)which was then  on the periphery of the City. But due to fast pace of urbanization, the Azadpur Mandi   now forms part of the  Centrally located Urbanized area with a heavy inter City and intra City truck movement. MPD-2021 has designated the market as Sub-City level market. The City level requirement shall be shifted to Integrated Frieght Complexes in Narela near entry point of Delhi.

1.4.7     Inter State Bus Terminal (ISBT), Kashmere Gate

The major Entry point to Delhi by Bus is located in this Zone. It is proposed to integrate ISBT with Delhi main railway station and the land should be made available by the shifting of the IP University to the new Campus.

1.4.8     Coronation Pillar

Coronation Memorial reminds of the foundation stone laid for Imperial Delhi as the new capital of India  which was subsequently shifted at Raisina hills. Coronation Park houses the marble Statues of King George V,which was originally under the canopy at India Gate, and  the other  Important personalities of the British period in India . Earlier Zonal plan had proposed to develop this land as special children play area similar to the one existing at India Gate.

The land around Coronation Pillar  is under the jurisdiction of L&DO.It  is near to NH-1 Bye-pass and therefore it has the potential of being developed as a tourist spot. The adjoining area earmarked for Green/water body can be taken for development of sports facilities etc.

1.4.9        Dhirpur Project,   Phase-I

Dhirpur Phase-I Project covering an area 194.5 Ha., is proposed for a population  of 40,000. As per MPD-2021, a non- hierarchical Commercial Centre has been proposed in Dheerpur Phase-I.

1.4.10      Dhirpur Project,  Phase-II

The Master Plan envisages the area to the West of Dheerpur Phase-I to be developed for partly commercial (District Centre), residential, facility centers, recreational, local shopping centre etc. It is proposed to develop this area on a comprehensive basis as per the uses proposed in MPD-2021.

A large portion of this area is owned by DJB and shall  be developed as per recommendations for various uses jointly by DDA and DJB. The Public Private Partnership model can also be worked out for development of this Project.

2.0      ZONAL DEVELOPMENT PLAN   PROPOSALS

The Zonal plans shall detail out the policies of the Master Plan 2021and act as a link between the Layout Plan and Master Plan. The development schemes and layout plans indicating various use premises shall conform to the Master Plan / Zonal Plans

2.1     POPULATION AND EMPLOYMENT

| i) | Existing Population 2001 | -6,79,000 |
| --- | --- | --- |
| ii) | Holding capacity as per MPD-2021 | -7,88,000 |

Zone-C is a saturated zone in terms of population and as per MPD-2021 the additional  net population to be accommodated in this zone is 1,09,000.

2.1.1    Population  holding  capacity

Population holding capacity of Zone C is to be enhanced through a redevelopment strategy and differential  development norms as per MPD-2021. This Shall  be related with:

- Residential development types and their potential for higher absorption.

- Re densification of housing areas developed at lower densities and along selected sections of the metro corridor.

- Redevelopment areas should be identified by the concerned agencies and special redevelopment schemes should be prepared with regard there to for implementation with in a stipulated time frame work.

- Employment areas/centers.

- Augmentation and rationalization of infrastructure – physical and social.

- Increase in transportation network capacity.

###### 2.1.2     EMPLOYMENT

Based on various activities, employment centres have been identified with estimated workforce of about 3,00,228 (Participation Rate-38.1% as per MPD-2021). The following work places have more than 10,000 work force:

- Old Secretariat.

- Tis  Hazari Courts

- Delhi University

- Regional Wholesale Market with warehousing & Truck Terminal-               North Samaypur ( C-21)

- New  Azadpur, Fruit & Vegetable Market

- Rajasthan Udyog  Nagar G.T.Road near Jahangirpuri.

- SMA Co-op Indl. Estate G.T.Road near Jahangirpuri

- SISI Indl. Area G.T.Road nearJahangirpuri

- G.T.Karnal Road Indl. Area near Rana Pratap Bagh.

- ISBT, Kashmere Gate.

2.2      RESIDENTIAL DEVELOPMENT

2.2.1  RESTRUCTURING AND UPGRADATION / REDEVELOPMENT OF THE     EXISTING  AREAS

In Zone-C, a large number of areas are old and are characterized by poor structural condition of buildings, sub optimal utilization of land, congestion, poor urban form, inadequate infrastructure services, lack of community facilities, etc.

The housing stock in both planned and unplanned areas can be enhanced through various approaches as per MPD-2021.The areas for which redevelopment schemes can be prepared are:

2.2.1.1   Planned Areas

A.	  Plotted/Group Housing

There are a number of plotted Residential Colonies  like Model Town, Gujrawalan Town, Derawal Nagar, CC Colony, , Kamla Nagar, Rana Pratap Bagh, Shakti Nagar, Roop Nagar, Jawahar Nagar, Kingsway Camp etc. which came up before MPD-1962. Outram Lines and Hudson Lines were the Rehabilitation Colonies which were re planned by DDA . (Refer Annexure-2)

The flats built by DDA, particularly those, which have become aged, may be redeveloped with permission and subject to the condition that the structural safety of other flats is not impinged. Already developed group housing inclusive of public (DDA and others), co-operative housing may be redeveloped on the basis of prescribed norms and regulations by formulating co-operative societies or self-managing communities. The funds for redevelopment shall  be contributed by the residents / beneficiaries.

B. 	Employer Housing

In Zone-C substantial areas were developed at low density after Independence, and have potential for densification. Employer housing has come up in Delhi University, Police Lines, T.B.Hospital, Timarpur, Radio Colony, MCD Colony, DJB Colony, I.P. College etc. Most of them are located within walking distance from MRTS.

These are mainly government areas. In order to optimally utilize these prime lands there is need of intensive development. On a conservative estimate the present housing stock can be increased to more than double. Infrastructure enhancement and provision for additional housing can be financed from the funds generated through cross-subsidisation between commercial and residential use for EWS and LIG categories.

C. 	 Bungalow Area

The Scheme for Redevelopment of Bungalow area was approved vide D.D.A Resolution no. 38/93 dated 23.3.93.

Necessary modifications in the plan have been incorporated in the plan on 8.4.99 after the Zonal Development Plan for Zone-C was approved by the ministry of Urban Affairs and Employment, Department of Urban Development ( Delhi Division) vide No. K-13011/22/93-DDIB dated 24th September 1998.

RECOMMENDATIONS :

BOUNDARY OF THE BUNGALOW AREA, CIVIL LINES :

The boundary of the Civil Lines Bungalow area shall contain the areas of earlier sub-zones C-2 and C-3, measuring about 250 hect.  Thus the boundary is to run from ISBT, Mahatma Gandhi Marg in the east, Ashram Marg/part of Shamnath Marg in the North and Ridge boundary in the West, Boulevard Road (Qudesia Road) in the South meeting at ISBT.

DEVELOPMENT CONTROL NORMS :

There were considerable number of large size (more than one acre) bungalow plots, but with the passage of time quite a number of  plots have been utilized/sub-divided for Group Housing Scheme and otherwise.  These plots are generally fronting on narrow roads/lanes (8 to 10 mts. wide) which are to be widened reasonably (min. 13.5 mts.).  Therefore, remaining plots shall be further reduced in size.  Considering such grounds realities it may not be appropriate to follow the bungalow policy of New Delhi Area, i.e. a bungalow to be replaced by a bungalow.  Norms for development for residential categories shall be as under :

i)	BUNGALOW PLOT (4000 sqm. And above for Group Housing) Development norms shall be as per D.D.A. resolution NO. 184 dt. 24.12.1980 and 38/93 dated 23.3.1993 (annexture x) i.e.

a)   Ground Coverage	25%

b)   FAR	75

c)   Height	11 M (35)

d)   Density	62-75 ppa or 15 DUs/Acre

(38 DUs/Hact.)

ii)	STREET WIDTH FOR GROUP HOUSING :

Since this is an existing area and there is hardly any scope to achieve considerable road widening it is recommended that the fronting road to be widened 13.5m minimum R/W equally on either side from the centre line of the road.

iii)	OTHER RESIDENTIAL PLOTS:

The norms for development of plots less than 4000 sq.mt. shall be on low intensity as under :

| Plot size (sq.mt.) | Max. Ground coverage (%) | FAR | Max. Height (metre) | No. of DUs |
| --- | --- | --- | --- | --- |
| Below 4000 | 33.33 | 75 | 11 | Upto 15 DUs/ Acre or 38 Dus/Hect. |

OTHER CONDITIONS:

a)	Basement, if any, shall be counted in FAR if not used for parking/services where applicable.

b)	Parking and tree plantation/landscape shall be governed as per MPD-2021 norms.

iv)	Existing residential plots shall not be permitted to be sub-divided.

v)	Plots other than residential

Development norms for such plots shall be worked out keeping in view the urban form studies within the prescribed code of MPD-2021 so as to maintain the existing character of the area.

vi)	Two local shopping centre are proposed on Govt. land at R.T.  Office on under Hill Road (C-3) and another on Shamnath Marg.  A convenient shopping centre be developed on Yamuna Road adjacent to Govt.  Office complex (C-2).  The existing exchange store be re-developed as convenient shopping with adequate parking facilities.

vii)	Existing intensive/non-compatible uses to be shifted to regain the tranquility of the area :

a)	National Institute of Communicable Diseases (5 Hect.) on Shanmath Marg. Vacated land to be utilized for meeting out the deficiency in community facilities like local Shopping Centre Car Parking, park and play area etc.

b)	Regional Transport Office on Crossing of Rajpur Road and Under Hill Road land vacated is to be utilized for Local Shopping Centre and Parking etc.

c)	Office of STC on Rajpur Road.

d)	Office of Commissioner of weights and measures on Under Hill Road.

e)	Office of Commissioner of Taxes and Excise on Battery Lane.

viii)	Green Area between Under Hill Road and Raj Niwas Marg is to be redeveloped.

ix)	Mixed use :

This area is practically free from the mixed use and considering the tranquility of the area, no mixed use be permitted except professional activity as per MPD.

x)	River bed should be exploited for providing recreational facilities, play area etc. by providing subways.

xi)	In the zone there are number of buildings of architectural value, which may be listed for conservation like Metcalf home etc.

xii)	Sham Nath Marg and Rajpur Road to be widened to 30 metre and 24 metres respectively and other internal roads in C-2 to be widened to 13.5 metre equally on either side from the centre line of the road.  The court Road and Under Hill Road be widened to 24 metre and Raj Niwas Marg to 18 metres.

xiii)	Existing road side plantation should be retained and more trees are to be planted wherever possible.

xiv)	Detailed urban form study should be conducted around important junctions.

xv)	Hoardings- No Hoardings be permitted unless properly designed and approved by local Authority.

- The Landuse of property No. 7 Court Road, Civil Lines, Delhi will remain ‘Residential’.  Vide the directions of Min. of Urban affair and Employment Govt. of India of No. K13011/22/98-DD-II Dated 24th Sept 98.

2.2.2    TRADITIOAL INNER CITY AND UNPLANNED AREAS

###### A.        Special  Area (Walled city and Walled City Extension)

The Special Area as defined in the Plan has been divided into three separate parts, namely (i) Walled City (ii) Walled City and Extension and (iii) Karol Bagh.

The most important part of the special area is the traditional city of Shahajahanabad (Walled City), part of which falls in C-1.Pahar Ganj, Sadar Bazar, Roshanara Road and their adjoining areas comprise the Walled City and Extension. These are old congested built up areas and for up gradation of the environment of these areas, minimum level of infrastructure and parking facilities shall be provided.

The redevelopment in these areas shall be in accordance with the respective comprehensive redevelopment schemes with conservative surgery as a planning tool.

A part of Sub-Zone C-1 (Kashmiri Gate), Sub-Zones C-4 & C-5 (Old Subzi Mandi),C-6 (West MaIka Ganj) ,C-7 (East MaIka Ganj) and C-I0 (Jawahar Nagar-Part) have been identified as a part of special Area and therefore Redevelopment plans shall be made  as  per  the special provisions given in the Master Plan

Special Area Redevelopment Schemes shall be prepared for traditional areas giving the development control parameters for the heritage areas keeping in view archaeological norms / architectural character and general parameters for the non-heritage segment of the traditional area. Each Special Area Redevelopment Scheme shall entail an enumeration of the monuments and old buildings within the heritage areas, which shall be conserved. The MCD is the nodal agency who shall prepare Special Area Redevelopment Schemes for all traditional areas within a time frame.

The basic objective of Special Area Redevelopment Scheme in traditional areas is to bring about in situ improvements which help in improving architectural character of the area, i.e., design and layout, as well as revitalising trade and commerce in the area.

###### B.        Villages

The Urban  villages in Zone-C (Refer Annexure -2) have experienced substantial socio-economic changes. Village Redevelopment Schemes shall be prepared for villages giving the development control parameters by the concerned local body. The redevelopment plans shall  ensure that the permissibility of mixed use zoning at property or within the premise level is compatible to the predominant residential areas.

C.      Unauthorized /  Regularized  Unauthorized Colonies

Out of the list of  unauthorized colonies identified by Govt. of NCTD which are under the process of regularization, some of the colonies fall in Zone-C and shall be dealt as per the policy for regularization of Unauthorized Colonies. (Refer Annexure- 5)

In all unauthorized colonies whether on private or public land, regularization shall be done as per the government orders issued from time to time. It shall be ensured that for improvement of physical and social infrastructure, the minimum necessary/feasible level of services and community facilities are provided as per norms given in MPD-2021.

Tentative List of Unauthorised Colonies identified in Zone-C as per the GNCTD list of 1520 Unauthorised Colonies is enclosed at Annexure –5 (ii).

2.2.2.1   Facilities in Unplanned  Areas

As Special permission from  the Authority or as per approved Layout Plan, the following institutional activity / PSP facility under Residential Use Zone, is permissible under Sub-Clause 8(2) of MPD-2021:

i)   Hospital (upto 100 beds)

ii)  Primary Health Centre / Family Welfare   Centre / Maternity Home

/dispensary  etc.

iii) Nursing Home / Poly Clinic / Clinic /Clinical  lab. Etc.

iv) Dispensary for Pet and animals

v)  Primary School / Middle School

vi) School for Mentally / Physically Challenged

vii) Technical Training Centre ( I.T.I /   Polytechnic / Vocational Training

Institute/   Management   Institute /Teacher Training   Institute, etc.)

Facilities- Bus terminal, Taxi stand, milk / vegetable booths, religious premises,  vending booth, petrol /   CNG filling  pump, recreational club,police post, police station, fire station, post office,& telegraph office aand telephone exchange.

2.2.3     MIXED LAND USE

2183 streets have been notified by the GNCTD vide notification dated 15.09.06 for local commercial and mixed use activities.. Again, local commercial and mixed use Streets were notified on 12.4.07.In case there is a difference in ROW between  GNCTD notification and  Zonal Development Plan , the ROW given in ZDP shall prevail.

Some areas developed prior to 1962 like Kamla Nagar having concentration of commercial activities shall continue subject to conditions prescribed under the Mixed Use Regulations..

List of Mixed Land Use Streets notified in Zone-C is given at  Annexures-6,7,8.The List of Mixed LandUse Streets is subject to  Amendments/Variations   from   time to time.

2.2.4      HOUSING FOR URBAN POOR

2.2.4.1   Rehabilitation/Relocation of Slum & JJ Clusters

A number of Slum & JJ Clusters are existing in Zone-C which have located themselves on the prime commercial / Green /PSP  lands etc. These shall be dealt as per policy depending on its location , Use etc. while preparing Local area plans.

2.2.4.2    Resettlement Colonies

The resettlement colonies   in this Zone are Jahangirpuri, Nehru Vihar, Gandhi Vihar etc. These resettlement colonies have been provided with essential services, but the immediate need is of individual services i.e. water, sewerage and electricity. To ensure healthy and better environment, up gradation of infrastructure is needed.

2.2.4.3    New Housing for Urban Poor

New housing shall be in the form of one or two room units, which shall  be developed through public and private agencies and through Cooperative societies. The developers shall ensure that the dwelling units are constructed for Community-Service Personnel/EWS and lower income category as per the norms given in MPD-2021.

2.2.4.4    Night Shelter

Night Shelter  is existing near New Azadpur Mandi on G.T. Karnal Road. New Night Shelters shall be proposed near workplaces like ISBT ,Sanjay Gandhi Transport Centre and in  Facility Centres etc. by the respective implementing agency.

    - REDEVELOPMENT AREA ALONG METRO CORRIDOR

(INFLUENCE ZONE)

Phase-I of Underground Metro Corridor passing through Shamnath Marg  and Mall Road is functional in this Zone. This MRTS network would  bring sizable urban area within walking distance from the proposed stations.

Influence Zone along MRTS corridor is envisaged as intensive development zone  The Residential Areas coming in the Influence Zone Maximum upto 500 m. wide belt on both sides from the centre line of the MRTS / Major Transport Corridor (to be identified in consultation with GNCTD) shall be designated as Influence Zone .The Planned residential areas falling in the influence Zone along MRTS  shall  be taken for  Redevelopment .These  schemes  for Redevelopment of Influence Zone shall be prepared on the basis of the norms given in MPD-2021.

The Civil Lines Bungalow Area shall be excluded from the enhancement of FAR.

  - WORK CENTRES

2.3.1       INDUSTRIES

The classification of industries is proposed to be simplified with prohibited category and non-prohibited category subject to proposals regarding specific category of industries permitted in different use zones.(Annexure –I,II,III MPD-2021) The Industries listed under prohibited category shall not be permitted to be set up in Delhi. Further, all planned industrial areas in Zone-C are designated under a single land use category namely, Industry.  The standards prescribed by the Pollution Control Authorities would have to be met by all industrial units, in addition to specific conditions in terms of number of workers and power load in specified categories.

2.3.1.1      Redevelopment of existing planned Industrial Areas

The following planned industrial areas were developed in the 70’s and over the years, have deteriorated considerably in terms of physical infrastructure and, in some cases deficiencies on this score have persisted in an overall sense.  Besides, there have been changes in the nature of activities in some of the areas and there is need to prepare Redevelopment Plans together with up gradation of infrastructural services.

| 1. | Rajasthan Udyog Nagar, G.T.Road near Jahangirpuri. |
| --- | --- |
| 2. | SMA Co-op. Indl. Estate, G.T.Road near Jahangirpuri. |
| 3. | SISI Indl. Area G.T.Road nearJahangirpuri. |
| 4. | G.T.Karnal Road Indl. Area near Rana Pratap Bagh. |

There is a need for modernization and up gradation of the existing industrial areas with due regard to environmental consideration.  Since most of the industrial areas are located along the Mass Public Transport Corridors, there is also a need of redevelopment.

This process of up gradation and redevelopment will need to be carried out in a planned manner, and in public-private partnership framework, in which the entrepreneurs contribute to the betterment and subsequent maintenance through suitable Operation and Maintenance arrangements.

2.3.1.2      Redevelopment  of  Unplanned Industrial Clusters

MPD-2021 has proposed policy for re-development of unplanned industrial areas, which includes 20 clusters with industrial activities for re-development. These are categorized in two parts.

Samai Pur Badli was included in MPD-2001 for Industrial areas re-development schemes.  In this case, the land use was indicated as extensive industries. As per MPD-2021,the Land use is industrial (Manufacturing, Service and Repair industries)

MPD-2021 has included Libaspur unplanned industrial area for redevelopment in this Zone. The Govt. of Delhi has notified this  non- conforming clusters of industrial concentration having more than 70% plots in the cluster with industrial activities for redevelopment vide notification dated 5.12.2005.This will be dealt further as per the actions taken for redevelopment prescribed in MPD-2021.

2.3.1.3     No  Industrial Activity zone

In order to maintain the City’s ambience and pollution free environment in important and historic areas of Delhi, Civil Lines Bungalow Area has been categorized as ‘No Industrial Activity Zone’ where no Industrial activity including household industry , shall be permitted.

2.3.2      TRADE AND COMMERCE:

As per the MPD-2021 the five tier hierarchy of Commercial areas is envisaged to accommodate required shopping, commercial office and other service activities like cinema, hotel and restaurant and various community services and facilities in an integrated manner. Service Market and Informal Bazar need to be integrated with Disrict Centres wherever possible.  Involvement of Developers, Owners Associations in upgradation of infrastructure and built environment in Old District Centres is recommended.

In addition, some components of commercial use have been provided under mixed    use, non- hierarchical commercial centers, and informal sector .

2.3.2.1       District Centres/ Community Centres

District  Centres in Zone-C , in the process of development are indicated below:

| 1. | Dheerpur Ext. (Jahangirpuri) |
| --- | --- |
| 2. | Majnu Ka Tilla (Khyber Pass) |

The site for proposed District Centre at Dheerpur Phase-II has been  suitably modified  as per approved scheme plan of Jahangirpuri Resettlement Colony  prepared by MCD.

2.3.2.2     Non- Hierarchical Commercial Centre

Besides the above District Centres, the following non-hierarchical commercial Centre shall be developed with specific scheme:

| 1. | Non-hierarchical Commercial Centre, Dheepur ph.-I |
| --- | --- |

2.3.2.3     Community Centres (CC)

The Community Centres in Zone-C are as under:

| 1. | Tis Hazari |
| --- | --- |
| 2. | Dheerpur PH.-I |
| 3. | Model Town/AzadPur |
| 4. | G.T.Karnal Road Industrial Area |
| 5. | Timarpur |
| 6. | Malka Ganj |
| 7. | Nehru Vihar |
| 8. | Mukherjee Nagar |
| 9. | Outram Lines |
| 10. | Adarsh Nagar |

All these Community Centres are indicated on the Land use plan of MPD-2021 & Zonal Development Plan.

In unplanned / undeveloped centers adequate provisions shall be made for Service Markets and Informal Bazaars etc. as per MPD-2021 norms.

2.3.2.5     WHOLESALE TRADE

One regional wholesale market with warehousing and truck terminal facilities has been proposed at Samepur Badli. Similarly warehousing and depots have been proposed at G.T. Karnal Road Industrial Area and at the crossing of Outer Ring Road and G.T. Road

New Azadpur Mandi was developed in early seventies for Fruit and Vegetables. This has been designated as Sub City level wholesale markets to cater to the needs of population at local level. The Wholesale trade at City level shall be shifted to IFC Narela.

The traders associations shall share the responsibility of redevelopment to modern wholesale markets. The  beneficiaries will have to pay the betterment charges to the implementing agency. Space for commodities that are not permissible in mixed-use streets shall be made available in wholesale city level market and sub-city level market to enable their relocation.

The existing planned wholesale markets, warehousing and transport centers in Zone-C are as under :

| S,No. | Location | Commodity/Activity |
| --- | --- | --- |
| 1. | North Samaypur , C-21 | wholesale market with warehousing and truck terminal. |
| 2. | Azad pur | Fruit & Vegetable |
| 3. | Sanjay Gandhi Transport Center | Transport/Warehousing |
| 4 | Outer Ring Road & G.T. Karnal Road Crossing | Warehousing |

The existing Wholesale markets in Special Area shall be dealt as per the policies prescribed in MPD-2021.

#### 2.3.3        INFORMAL TRADE AND COMMERCE

#### 2.3.3.1    Service Markets/ Informal Bazar

Special attention is required for the low turnover and space extensive shops for fruits and vegetables, service and repair, junk and scrap materials (kabari), building materials, automobile workshops etc.. The grouping of such activities with planned retail markets leads to conversion of shops into high profit commercial activity.

2.3.3.2    Informal  Sector

The informal sector units locate themselves strategically near work centers, commercial areas, outside boundaries of schools, colleges, hospitals and transport nodes and near large housing clusters. A large number of units are mobile in nature. A very high percentage of this activity has been observed in in Zone –C.

‘Hawking’ and ‘No Hawking’ Zones shall  be identified by the concerned local authority.

      - Organized Informal Sector Places (Haat)

There are large numbers of informal sector units but no organized clusters. There is a need to provide for organized informal eating-places along with casual shopping etc. to be located strategically in  Dheerpur Phase-II.

2.3.4      GOVERNMENT USE

Government of India, Govt. of NCTD and local bodies are occupying prime land in Delhi for their offices.  Most of the offices have been setup before or immediately after Independence.

Major GNCTD offices, which were located in Old Secretariat building, have been shifted to the new premises at Indraprastha Estate. Old Secretariat is a historical building and needs to be conserved.  Barracks area adjacent to the Old Secretariat could be redeveloped to accommodate additional GNCTD offices.

Tis- Hazari District Courts is a major employment center on Government land in sub-zone C-1. Government Lands which are underutilized and in old and dilapidated state shall  be redeveloped as per Land use and norms given in MPD-2021.

The optimum utilization of the following Government lands  shall be worked out;

| 1. | Old Sectt. Complex, (C-12) |
| --- | --- |
| 2. | Tis Hazari Courts(C-1) |
| 3. | Rajan Babu T.B.Hospital, (C-19) |
| 4. | Delhi University Campus ,( C-13) |
| 5. | DJB land at Dheerpur ph.-II, C-19 |
| 6. | Police Lines (C-16) |
| 7. | Transmission Centres at Radio Colony & Mall Road. (C-19 & C-14) |
| 8. | Govt. Land (Timarpur)(C-14) |
| 9. | Land for Communications (Post/Telegraph/Telephone Facility) Banda Bahadur Marg.(C-15) |
| 10. | Govt. Land Coronation Pillar(C-15) |

2.4          SOCIAL INFRASTRUCTURE

The facilities at  the Zonal Plan level have been indicated on the plan but the facilities at the Neighbourhood level shall be worked out at the time of preparation of Local Area Plans.(Refer annexure-14).There is very little scope for development ,but where ever re develoment shall take place, MPD-2021 nnorms shall be followed.

The Social Infrastructure in this zone shall be upgraded in this zone through the following  measures :

- Redevelopment of large Campuses to accommodate additional requirements.

- Enhancement in FAR as per MPD-2021.

- Additional requirement at city level shall be met by utilizing underutilized Government lands.

2.4.1        HEALTH

2.4.1.1   Hospitals

MPD-2021 states that the existing bed density per thousand population in Delhi works out to only 2.2. The World Health Organization (WHO) has recommended a norm of 5 beds per thousand population.

MPD-2021 has categorized Hospitals in to the following  Categories:

Category A(501 beds & above) for every 5 lakh population

Category B(201 beds to 500 beds) for every 2.5 lakh population

Category C(101 beds to200 beds) for every 1 lakh population

Category D (Upto 100 beds) for every 1 lakh population

Other health facilities, which include maternity home, nursing home, family welfare center, polyclinic, pediatrics center, geriatric center, diagnostic center etc. shall be provided as per the norms given in MPD-2021.

List of  Government Hospitals on sites earmarked for Hospitals is as under:

| S.NO | Existing Hospitals | Sub-Zone |
| --- | --- | --- |
| 1. | St. Stephens Hospital | C-1 |
| 2. | Hindu Rao Hospital | C-11 |
| 3. | Rajan Babu T.B Hospital | C-19 |
| 4. | Maharishi Balmiki I.D.Hospital | C-19 |
| 5. | Babu Jagjivan Ram Hospital, Jahangirpuri | C-20 |
| 6. | Balak Ram Hospital, Timarpur | C-14 |

Re building ,up gradation and modernization of existing old hospitals is proposed by enhancement of FAR for various levels of health facilities.

Hospitals Proposed  in Facility Centres in Zone-C are :

| S.NO | Existing Hospitals/Sites | Sub-Zone |
| --- | --- | --- |
| 1. | Prop. Hospital (FC-3) | C-15 |
| 2. | Prop. Hospitals (FC-2) (2 Hospitals- B & C) | C-18 |
| 3. | Prop. Hospital (FC-6) Dheer pur Ph. II | C-20 |

Rajan Babu TB Hospital and Maharishi  Balmiki  ID Hospitals are  contagious  diseases Hospitals in Zone-C  and as per MPD-2021,these Hospitals shall be shifted in Urban Extensions with proper seclusion facilities and connectivity, and using the space thus made available for general hospitals.  These Hospitals are in dilapidated state and also invite a large number of Informal sector.

2.4.2        EDUCATION

2.4.2.1     Delhi University

Delhi University has a total of 11 colleges including some other specialized institutions within its complex. It shall cater to this zone as well as the adjoining other Zones.

###### 2.4.2.2    Vocational Training Centres

As per MPD-2021 norms one ITI(Vocational Training Institute) is required for every 5 lakh  population. The Zone has three proposed Industrial Training Institutes:

i)    ITI in FC-4 (C-15)

ii)   ITI, G.T.Road  (C-18)

iii)  ITI in Jahangirpuri ( C-20)

###### 2.4.3     SPORTS FACILITIES

The land around Coronation Pillar  is under the jurisdiction of L&DO. It  is near to NH-1 Bye-pass and therefore it has the potential of being developed as a tourist spot  including the adjoining Green/water body for development of District sports Centre etc.

Chattarsal Stadium is located in sub-zone  C-18  on Ring Road opposite Model Town-III and a DDA Sports center  measuring 0.665 Hac. is situated in C.C.Colony (C-17)

2.4.4    COMMUNICATIONS-POST/TELEGRAPH/TELEPHONE FACILITY

As per MPD-2021,the sites for Telephone Exchange and other means of Communications shall not be reserved  in layout Plan due to liberalization and technological advancement.. The site  earlier earmarked for Communications in sub-zone C-15 shall  be utilized for Science Centre.

In view of the less demand, the sites earlier earmarked for these facilities shall be utilized for new facilities as proposed in MPD-2021 such as Disaster Management Centre, Old age, Multipurpose Community hall etc.

Administrative Office of the Director General of Postal Accounts is located adjoining Old Secretariat Complex in sub-zone C-12. Transmission Centre sites in sub-zone C-19 and C-14 are underutilized and therefore optimum utilization of all these lands is proposed.

2.4.5       SECURITY AND SAFETY

2.4.5.1     Police Lines

The main agency looking after security aspect in the city is Delhi Police. Delhi  Police offices, employer housing Complex and Police Grounds is situated in sub-Zone C-16. These Government Lands are underutilized and it is proposed to make optimum utilization of these lands.

Change of Land Use of an area measuring 7.99 hac. has been notified  from Residential to PSP ( Police Headquarters and Police Line)in Civil lines Bungalow  Zone ( C-3) .

2.4.5.2    Fire Stations

As per the norms three Fire stations  for every 5 lakh population are to be provided in  5-7 Km. Radius, therefore total 5 Fire stations are required in Zone-C.

There are four  fire stations  sites proposed in this  Zone;

I)	 Roop Nagar (C-17)

ii)	 Chandrawal Water Works (C-12)

iii)	Jahangirpuri, (C-20) near Jagjivan Ram Hospital.

iv)        FC-7 in Sub-Zone C-20.(opposite Azadpur Mandi)

Fire Posts shall be provided in District Centres and Wholesale Markets.

###### 2.4.6     DISASTER MANAGEMENT CENTRE

As per MPD-2021, it is proposed to have a Disaster Management Centre in each Administrative Zone. Disaster Management Centre  measuring 3 hac.(1.0 Ha. along with suitable open area of 2 Ha. for soft parking, temporary shelter, parade ground etc.) shall be Integrated with the existing Fire Station at Chandrawal  water works on  DJB/GNCTD land in Sub-zone C-12. It could also be proposed in FC-5 (C-19).

2.4.7      DISTRIBUTIVE

2.4.7.1    LPG Godowns

As against the 24 Gas Godowns required as per MPD-2021, @ 3 LPG Godowns per one lakh population, 20 gas Godowns are already existing in the zone. No new Gas Godown sites are envisaged  as Piped Gas supply network is being provided in Residential Colonies by I.P.Gas agency.

2.4.8      SOCIO CULTURAL  FACILITIES

The Site for Socio-Cultural facilities  earmarked in sub- zone C-14, Opposite  proposed Khyber Pass District Centre has been  taken over by DMRC and Change of Land use as per  MOUD,GOI notification no..1273(E) dated 5.11.2003 has taken place. An alternate Site earlier earmarked for Communications in sub zone C-15 shall be utilized  for Science Centre. Socio Cultural facilities like Banquet Halls, Multipurpose Community Hall, Recreational Club, Exhibition cum Fair ground etc. shall be detailed out while redesigning FC-5 in Sub-zone C-19.

2.4.9     OTHER COMMUNITY FACILITIES

A number of public and semi-public facilities have been augmented in the zone mainly in six facility centres as per detail in the table below;

###### Facility Centres  in Zone-C

| S. No. | Sub-Zone | Facility Centre No. | Area in Hac. (Approx.) | Facilities Suggested |
| --- | --- | --- | --- | --- |
| 1. | C-15 | FC-3 | 7.5 | Telephone Exchange*, Students Hostel, Guest House. |
| 2. | C-15 | FC-4 | 7.0 | Police Station, Hospital-A, College, ESS |
| 3. | C-18 | FC-2 | 5.0. | Hospital-B & C, Nursing Home, Head Post Office, Telegraph Office*. |
| 4. | C-19 | FC-5 | 9.0 | Social Cultural Facilities. |
| 5. | C-20 | FC-6 | 9.5 | Hospital- B, Nursing Home. |
| 6. | C-20 | FC-7 | 4.5 | Fire Station. |
|  | Total | 6 Nos. | 42.5 |  |

Source:ZDP-C, MPD-2001

The schemes for Facility Centres were prepared as per the norms given in the previous Master Plan. It is proposed to accommodate other Community facilities like Old Age home, Orphanage, Night Shelter etc. in Facility center -6 in Dheerpur Phase-II .

The site for proposed FC-6  at Dheerpur Phase-II (C-20) has been  suitably modified as per approved scheme plan of Jahangirpuri Resettlement Colony  prepared by MCD.A Bus Terminal site has been proposed in F.C-6 near Jahangirpuri.

2.4.9.1   Cremation Ground, Burial Ground & Cemetry

As per the norms given in MPD-2021, One Burial ground /Cemetry is required for every 10,00,000 Population. Nigambodh Ghat Cremation ground and Electric Crematorium in Zone-O serves the needs of the adjoining  Zones. Site for  any future Requirement  shall be worked out  with the help of MCD.

A number of historical Cemetries of the Colonial period are existing in this zone like Rajpura Cemetry, Nicholson Cemetry, Kashmere Gate Cemetry etc. Some of the Urban Villages have  there local  Cremation Grounds/burial grounds etc.

1. Proposed Cremation Ground Site along Railway  Line ,near  Sanjay

Gandhi Transport Nagar.

2. Qadimi Kabristan at Mukarba Chowk

3. A new Cremation Ground site has  been proposed in  Jahangirpuri

(FC-6).

2.5     TRANSPORTATION

2.5.1   Road  Network

Outer Ring Road (Road No.50) , Mall Road, Mall Road Extension and Road No. 51 are the major Arterial Roads  which  are maintained by PWD, NHAI etc.

MPD-2021 proposes optimal use and utilization of the existing road network and full development of ROW by removing all impediments. All arterial roads will be restructured to allow for smooth and safe flow of buses and non-motorized transport to minimize pollution and congestion.

2.5.1.1 Road Development Plans( Under JNNURM Project)

All the Road owning agencies shall get Road Development Plan prepared for 30 M & above ROW Roads shown in MPD-2021 along with interchange details of junction / intersection falling in the alignment showing location of bus stop, metro station &  the pedestrian linkage for smooth changeover from one mode to another

2.5.2   Road  Network  not implemented

There are other Sub- Arterial Roads which have not been implemented partly and the continuity of these Roads needs to be checked by the concerned Department for good connectivity and  easy  flow of the traffic:

i) Road No. 48 -starting from Ring Road (along Najafgarh Drain ,Hudson Line) and linking G.T.Karnal Road is encroached in Vijay Nagar .

ii) Master Plan  Road, 30 m ROW -Princess Road / SRP Bismil Road connecting Ring Road (near Alpana Cinema) to Coronation Pillar in Dheerpur Phase-II is interrupted at North of Model Town.

iii) Master Plan Road 30 m ROW at the North of Model Town connecting Princess Road and Road No. 51  is encroached near Gopalpur Village.

iv) Realignment Plan of Road No. 46, near Khyber Pass is not implemented.

2.5.3   Urban Relief Roads

MPD-2021 has proposed a number of Urban Relief Roads all over Delhi. The following additional Urban Relief Roads could be considered for detail feasibility by GNCTD :

i) In order to reduce congestion on G.T.Karnal Road near Azad pur Fruit and Vegetable market, a Relief Road to give access to this Market from its back along the existing Railway line is being proposed. This would help to curtail the Truck movement on G.T.Karnal Road and deflect the traffic at the back of the Azad pur market.

ii) Dheerpur phase-I, Village Gopalpur, Resettlement colonies- Gandhi Vihar, Nehru Vihar etc. have a poor connectivity with the rest of the zone and Metro Corridor.  A relief Road is being proposed parallel to the Najafgarh drain starting  from Ring Road and linking Outer Ring Road at  Wazirabad.

2.5.4   Grade Separators

The Master Plan studies indicate the need for provision of intersections with grade separators.  In case of existing grade separators the possibility of providing cloverleaves and direct interchanges, wherever necessary and feasible, may be examined in order to make the junctions signal free.  Outer Ring Road (Road no. 50) is being up graded as part of NHDP and it shall be a limited access road. Grade separators with Cloverleaves shall be provided on all 60 m ROW Roads.

A Grade separator shall be planned at the intersection of every 45 m and above ROW Road intersection. A Cloverleaf Grade separator shall  be planned at the Intersection of 60 m ROW Road Intersection depending on the availability of land. The existing  grade separators in the zone are:

i) 	Near I.S.B.T.

ii) 	On Railway crossing near FC-2.

iii) 	Railway Crossing on Ring Road near Azadpur.

Iv)	Railway crossing on Outer Ring Road near Sanjay Transport  Nagar.

Looking into the overall development/future development of Delhi and the Northern part of Delhi it is proposed to have few more grade separators in Zone-C presently in the following locations:

i) 	Junction of Outer Ring Road and Wazirabad barrage

ii) 	Junction of Mall Road and Road No. 51 (Majnu Ka Tilla).

iii) 	Junction of G.T. Road and Outer Ring Road (Mukarba Chowk).

iv)	Junction of Azadpur and G.T. Road

v)	Junction of Rani Jhansi Road and Lala Hardev Sahai Road.

vi)	Junction of Auchandi and G.T.Karnal Road.

2.5.5    Mass Rapid Transit System (MRTS)

The Metro Rail System is the most important component of a Mass Rapid Transport System (MRTS) in the City. The Metro Rail Network for the entire city has been identified in various phases, which comprises of a network of underground, elevated and surface corridors aggregating to approximately 250 Kms.

Phase I of the underground network from Rajeev Chowk upto Vishwavidyalaya  in Zone-C is already operational . It covers Shamnath Marg and  Mall Road.

Phase-II covers partly underground and elevated network from Vishwavidyalaya (Mall Road) to Jahangirpuri (G.T.Road).

2.5.6   Integrated Multi-Modal Transport System

Integrated Multimodal Transportation proposal may be earmarked / incorporated on the zonal plan with the help of  DIMTS, GNCTD.It is envisaged that the future transport system shall consist of a mix of rail and road based systems which may include Metro Rail, Ring Rail, Dedicated Rail Corridors for daily commuters, Bus Rapid Transit System (BRTS), other mass transit modes as technologies become available and Intermediate Passenger Transport (IPT) and private modes on selected corridors to be identified as per the needs from time to time.  All roads should be made pedestrian, disabled and bicycle friendly as far as possible.

Vishwavidyalaya , Vidhan Sabha and Civil Lines metro stations are covered in the phase-I of metro corridor. Preparation of detailed multi-modal transport plans with reference to each major Metro Station, with particular reference to bus transport routes, which could provide inter-linkages and feeder arrangements is envisaged.

2.5.6.1 BRT Corridor

A BRT Corridor of approximately 14 Km. has been proposed by DIMTS from  Mool- Chand to Jahangirpuri with a Proposed  terminal at Jahangirpuri. In Zone-C, It shall start from  Salimgarh Fort on Ring Road to Metcalf  House (2.25 Km.) and further from Metcalf House on Outer Ring Road to  Jahangirpuri, Mukarba –Chowk (11.65 Km.)

2.5.7   Bus Depot

In MPD-2021 it is envisaged to have 2 Bus Depots for a population of 5 lakhs. Accordingly, the required number of bus depot is 3 Nos. for a population of 7,88,000 persons. At present there are 2 Bus Depots operating under Delhi Transport Corporation. Besides this, one Haryana Bus depot is also located in the Sub-Zone C-15.

2.5.8   Bus Terminals

The major Entry point to Delhi by Bus ie  ISBT  is located in this Zone. It is proposed to integrate ISBT with Delhi  main railway station and the land should be made available by the shifting of the IP University to the new Campus. Restructuring of ISBT is proposed as per the changing requirements.

It is envisaged in the MPD-2021 that one Bus Terminal shall be provided for a population of 1 lakh. Hence the required  no. of bus terminals is 9 nos. At present 5 nos. of bus terminals sites are existing over this zone. Therefore, there is a shortfall of 4 bus terminals sites. These shall be planned in District Centres (2 No’s), Delhi University Campus (1 No.), Dheerpur Phase-II (1 No.),Redevelopment Scheme of sub-zone C-21(1 N0.)New bus terminals (1000 sq. m) need to be planned and developed in strategic locations to make the use of BRTS and Metro Stations convenient for all commuters at local area planning level.

2.5.9     Bicycle Tracks/Cycle Rickshaw Tracks

Segregated Cycle tracks/Rickshaw  tracks shall be proposed on feeder roads to Metro Stations only where Park and Ride facilities are available. These shall not be permitted on Arterial Roads .Existing  Cycle tracks are  on the following Roads:

| S.NO | LOCATION | STRETCH |
| --- | --- | --- |
| 1. | Along Shah Alam Bund | From T-Junction of Adarsh Nagar to Coronation Pillar. |
| 2. | Road No. 51 | From Azadpur junction to Outer Ring Road. |
| 3. | Along Najafgarh Drain | From Mall Road to Outer Ring Road. |
| 4. | Along Parade Road passing through Dhirpur Phase-I. | From T-Junction near F.C-3 to village Gopalpur. |

2.5.10     Railway  Stations

Presently 4 Railway Stations and one Railway Yard are  existing in Zone-C. These are Old Subzi Mandi, Old Azadpur, New Subzi Mandi and Badli.

2.5.11     Goods movement by Rail

Presently the goods are terminating at  New Azad pur Railway station.

2.5.12   FUEL STATIONS

A.          Petrol  Pumps

23 petrol pumps are already  existing in the zone. .Additional petrol pumps need to be proposed mainly in new Commercial Centres  and other new schemes as per the norms proposed in MPD-2021.

B.         CNG Stations

Already existing authorized CNG service stations for public transport vehicles may continue for this purpose alone and new CNG Stations shall  be given in District Centres/ Community Centres, Large PSP Campuses  schemes to be prepared as per the norms given in MPD-2021.

CNG sites are to be located while  preparing Local Area Plans and  Layout  Plans as  per the norms laid down in the Master Plan 2021 .

2.5.13    PARKING

With the phenomenal increase in personalized motor vehicles, one of the major problems being faced today is an acute shortage of parking space.

Parking has become a matter of serious public concern and requires a carefully considered policy and planned measures to alleviate the problem to the maximum feasible extent in existing areas and for adequate provisioning with reference to future developments.  In this background, the following measures are proposed:

A.        Park and Ride

Park & Ride lots are available on MRTS station on Vishwa vidyalaya only. Apart from providing Park and Ride facilities with reference to integration between the Road and Metro Rail/Rail Transport systems, such facilities shall be provided to reduce the problem of parking on main arterial roads in the context of identified work and activity centers which may not be directly connected by the MRTS and to encourage use of public transport.

B.        Public Parking

The major efforts will have to come through the creation of public facilities in designated commercial/work centers and other areas and corridors where significant commercial activity has developed by way of mixed use.  In the context of the latter, it would also need to be linked to pedestrianisation within the identified areas.

C.         Parking Facilities in  DTC Depots

At present there are 2 Bus Depots operating under Delhi Transport Corporation. Besides this one Haryana Bus depot is also located in the Sub-Zone of C-15.

The use of DTC terminals and depots for development of public parking along with parking of DTC buses, private buses and chartered buses should be explored and specific projects developed.

D.        Underground Parking

Based on the site feasibility, parking facilities can be created under the open spaces without disturbing the green areas on the surface and surrounding environment.  The approvals from the concerned agencies are mandatory before taking up such works.

E.        Parking in Residential Areas

Most of the parking is, in fact, being done on the road, which significantly reduces the carriageway width.  The problem has been exacerbated by the traffic congestion generated by schools in gross residential use areas.  Some measures required to alleviate the problem, to some extent, will be as under:

i)	All the encroachments on residential streets in the form of kitchen gardens/ roadside private greens, large projections/ramps, etc. need to be removed.

ii)	Road cross sections may be redesigned wherever possible to accommodate planned car parking along the residential streets, and also creating more surface movement space.

iii)	Other options, in selected areas, such as creation of underground parking below parks and open spaces will also have to be considered.

iv)	Resident Welfare Associations will have to be called upon to participate in this process by raising contributions from the residents on the basis of objective criteria such as number of cars owned, etc.

v)	Problem of congestion arising on account of the traffic generated by schools have to be specifically addressed, and the main responsibility for putting up the required additional facilities has to be borne by the schools themselves.  Policy guidelines will have to be evolved for this purpose.

vi)       In Pre-1962 Residential and Rehabilitation  Colonies given in Annexure-3,

creation of   underground  Parking below  Parks and Open spaces shall be

detailed out at the time of  preparation of Local Area Plans like :

* under the Neidhbourhood Park  (Vrindavan Vatika ) in Outram Lines.

F.        Multilevel Parking

As per MPD-2021, Multilevel parking is proposed in Kamla Nagar. Multilevel parking facility shall  also be developed in the following parking spaces subject to  availability of land and feasibility by  the concerned local body:

- Nangia Park roundabout

- Near Naini Lake and Model Town-II

- Near Gurudwara Nanak Piao

- Near Tis Hazari Courts & Khanna Market (Community Centre)

- Near Kamla Nehru Ridge and Delhi University.

- Jahangirpuri

###### G.         Idle Truck Parking

Change of Land use of an area measuring 4.0 hac. from Recreational to Metro parking  has been notified as per GOI notification no.1273 (E) dated 5.11.2003 for Idle Truck parking near Khyber pass depot in sub zone C-14.

  - ENVIRONMENT/ AREAS OF SIGNIFICANCE

2.6.1    NORTHERN  RIDGE / REGIONAL PARK

Northern Ridge is  a rocky outcrop of Aravali Hills covering an area of 87 hac. As Per notification no.F.10(42)-I /PA /DCF /93 /II dated 24.5.94 issues by GNCTD.

It needs to be preserved in its natural State. Metro Stations along with

The other major green areas in the zone are Roshanara Garden, Qudesia Garden built in 18th Century, Chhatrasal Stadium, Police grounds, University Grounds, etc. In addition a number of district parks and green areas have been proposed in Dhirpur Scheme. The Bungalow area is other major green area of the zone.

Sports Complexes which were included in the green/recreational use category under the MPD-2001 will be seen under a separate category of sports as per MPD-2021 because Delhi is emerging as an important center for National and International Sports Events.

2.6.2

Multipurpose Grounds

Formal parks are fouled if used for marriage / functions etc.  Therefore, a special category is proposed as per MPD-2021 to take care of the same at three levels viz City Multipurpose Ground, District Multipurpose Ground and Community Multipurpose Ground. District   Multipurpose Ground shall form part of Dhirpur Phase-II. Already approved parks on 60’ and   above roads  for holding  marriages  by the  Local body/ MCD  shall be  designated as Multipurpose   grounds.

2.6.3        POLLUTION  CONTROL

      - Drains

The drains are major pollutants of River Yamuna. Najafgarh drain is the major drain passing through this zone. All the drains shall  be developed as Green Corridors. Development of Greens along the drains is also recommended for Ground water recharge. Treatment Plants at the outfall of drains are proposed for pollution control and facilitating waste  water recycling.

2.6.3.2      Water Bodies

Water bodies ,having a minimum size of surface area of 1 ha., shall be preserved by the concerned authorities. Further efforts shall be made at the local area level to retain the smaller water bodies.(Refer Annexure-12)

2.7          AREAS OF URBAN  DESIGN

Zone–C is bounded by River Yamuna (Zone-O) on the  North-Eastern side . An Urban design study shall to be taken up for this area so as to integrate  the River front with the Urban area.

A part of the Walled City falls in this Zone.  The guidelines for urban Design of Walled City Area   are given in  MPD-2021.

Urban Design Scheme for Coronation Pillar / Memorial also  shall  to be taken up.    Other areas of Urban Design importance are as follows:

a.	Historical Monuments and Gardens .(Roshanara Bagh, Qudsia Bagh, etc.)

b 	Areas along entry routes and other important routes in Delhi. (NH-1)

c. 	Road and Rail, MRTS corridors, entries, and terminals.(ISBT etc. )

With the development and introduction of MRTS in Zone- C on Shamnath  Marg and Mall Road, need is felt to connect these scattered districts with more imageable components. These components with enhanced built up areas and activities, form a network by which the experience of various District and commercial centres  shall become  a part of continued urban experience

The Vidhan Sabha of Delhi state(Old Secretariat) marks the northern end of Shamnath Marg. Many public institutions flank its length. At the Southern end of Shamnath Marg is Kashmiri Gate and City wall of Shahjahanabad, which has now been eclipsed by the Kashmiri Gate Metro Station- the new ‘gateway’ to the modern city of Delhi. An Urban design study shall be done for Shamnah Marg .

As per MPD-2021, restrictions on tall buildings shall  be necessary in important areas like  Civil lines Bungalow Zone  and North Delhi campus. An urban design study shall be done for Civil lines bungalow zone to conserve its basic character.

The Urban Design study shall  be done for Delhi University Campus .It is suggested to develop it as an Integrated campus.

2.8         CONSERVATION   OF BUILT  HERITAGE

2.8.1      Special  Conservation  Plans

Each local body/ land owning agency should formulate "Special Development Plans" for the conservation and improvement of listed heritage complexes and their appurtenant areas. Alteration or demolition of any listed heritage building is prohibited without the prior approval of the Competent Authority.

The development plans/schemes for such areas shall conform to the provisions, in respect of Conservation of Heritage Sites including Heritage Buildings, Heritage Precincts and Natural Feature Areas.

The listed heritage buildings in Zone-C as per the surveys conducted by DDA,INTACH ASI and GNCTD are enclosed. (Refer Annexure- 9,10,11)

2.9       PHYSICAL INFRASTRUCTURE

As given under the requirement of Water, Sewerage, Power and Solid Waste Disposal for this Zone have been worked out in consultation with the concerned department and the  Engineering Wing of DDA.

2.9.1      Water Supply

In the zone there are two existing water treatment plants viz. Chandrawal I & II, having an existing capacity of 90 mgd in 1981.As per MPD-2021 , it is proposed to augment the capacity of these water treatment plants to 100 mgd  by 2021.

2.9.2    Sewerage

A sewage treatment plant covering an area of around 82 acres is existing in Sub-Zone C-19 (Model Town) near Coronation Pillar with a capacity of 40 mgd. As per MPD-2021, it is proposed to augment the capacity of this STP to 50 mgd by the year 2021. Similarly, an oxidation pond covering an area of around 30 acres is exiting in sub-zone C- 14 (Timarpur).

2.9.3      Power

Keeping in view, the norms of Master Plan-2021, the total power requirement for the entire zone shall be worked out. Existing 66 KV and above sub-stations and  high tension line passing through the zone are illustrated in detail.  A 220 KV Sub-Station is a part of Dhirpur Scheme.

2.9.4    Solid Waste

A large sanitary land fill site in Timarpur along the Outer Ring Road( Road No. 50.) has been filled up. This zone is saturated in terms of land and it is difficult to provide additional land for physical infrastructure at Zonal Development Plan level. Thus optimum utilization of existing lands shall be done and any additional request if any shall be seen by DJB / GNCTD who own land pockets in this zone.

2.10	  LAND USE PLAN

The proposed land use analysis at the Zonal Plan level is given with reference to various Use Zones .The facilities proposed  in the Landuse Plan shall be detailed out in the Zonal plan.

2.10.1  CHANGE OF LAND USE

2.10.1.1 CHANGE OF LANDUSE UNDER PROCESS AS PER  SEC.11-A

| S.NO | LOCATION | AREA (Hac. App.) | CLU |  | STATUS |
| --- | --- | --- | --- | --- | --- |
|  |  |  | From | To |  |
| 1. | Left over part of Samaipur Badli | 42.5 | Residential | Industrial | *As per minutes of the Authority held on 30.10.07 vide item no. 82/2007 |
| 2. | Graveyard at Mukarba Chowk near Jahangirpuri. | 1.0 | Utility (E.S.S) | Public & Semi- Public Facility (PS-1) | *Approved by T.C. Meeting held on 1.10.07 vide item No. 88/2007 |

2.10.1.2 CHANGE OF LAND USE INCORPORATED IN DRAFT ZDP

The following CLU cases have been  incorporated in the Zonal Plan and have already been notified or under the process of notification as per  Section 11-A of DD Act.1957:

| S.NO | LOCATION | AREA (Hac. APP.) | CLU |  | STATUS |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FROM | TO |  |
| 1. | Tis Hazari | 1.46 | Residential | Commercial | Notified as per GOI notification no.1273(E) dated 5.11.2003. |
| 2. | Khyber Pass Depot (a) | 2.80 | Distt. Park | Commercial | Notified as per GOI notification no.1273(E) dated 5.11.2003. |
| 3. | Khyber Pass Depot (b) | 10.0 | Distt. Park | Residential | Notified as per GOI notification no.1273(E) dated 5.11.2003. |
| 4. | Khyber Pass Depot © | 4.0 | Recreational | Metro Parking | Notified as per GOI notification no.1273(E) dated 5.11.2003. |
| 5 | Vishwavidyalaya | 3.05 | PSP | Residential | Notified as per GOI notification no.1017 dated 23.09.05 |
| 6 | 5, Rajpur road (Police lines) | 7.99 | Residential | PSP Use | Notified as per GOI notification no.281 dated 10.1.2007 |
| 7. | 12, Rajpur Road | 1.78 | Residential | PSP Use | Notified as per GOI notification no.955 dated 2.08.2007 |

2.10.2   LAND USE ANALYSIS

Taking into consideration the Change of Land use cases and  Zonal Development Plan proposals / Modifications , Land use analysis  is given below:

| S.no. | Land Use | Area (hac.) | Proposed %age as per ZDP-C, MPD-2001 | Change of Landuse |  | Area (hac.) | Proposed %age |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Area Added +(hac.) | Area Subtract. --(hac.) |  |  |
| 1 | Residential | 1625.3 | 41.8 | 13.05 | 11.23 | 1627.12 | 41.85 |
| 2. | Commercial | 255.7 | 6.64 | 4.26 | - | 262.34 | 6.74 |
| 3. | Industry | 162.1 | 4.2 | - | - | 162.1 | 4.2 |
| 4. | Recreational | 645.7 | 16.5 |  | 16.8 | 628.9 | 16.17 |
| 5. | Transportation | 410.4 | 10.66 | 4.0 |  | 414.4 | 10.65 |
| 6 | Utility | 94.7 | 2.46 | - | - | 94.7 | 2.46 |
| 7. | Government | 92.1 | 2.4 | - | - | 92.1 | 2.4 |
| 8. | PSP Facilities | 601.8 | 15.6 | 9.77 | 3.05 | 608.51 | 15.65 |
|  | Total | 3888* | 100% |  |  | 3888 | 100% |

* Excluding Walled City area of Zone-C

2.10.3   CLU (NOT INCORPORATED IN LANDUSE PLAN, MPD-2021)

The following CLU cases have not  been  incorporated in the Land use Plan (MPD-2021) but have already been notified under Section 11A of DD Act.1957:

| S.NO | LOCATION | AREA (Hac.) | CLU |  | STATUS |
| --- | --- | --- | --- | --- | --- |
| 1. | Tis Hazari | 1.46 | Residential | Commercial | Notified as per GOI notification no.1273(E) dated 5.11.2003. |
| 2. | Khyber Pass Depot | 2.80 | Distt. Park | Commercial | Notified as per GOI notification no.1273(E) dated 5.11.2003. |
| 3. | Khyber Pass Depot | 10.0 | Distt. Park | Residential | Notified as per GOI notification no.1273(E) dated 5.11.2003. |
| 4. | Khyber Pass Depot | 4.0 | Recreational | Metro Parking | Notified as per GOI notification no.1273(E) dated 5.11.2003. |

2.10.4   OTHER CHANGES INCORPORATED IN THE PLAN

The other changes incorporated in the plan are:

i)  District Centre at Dheerpur Phase-II suitably modified as per site conditions and

approved Layout plan of Jahangirpuri prepared by MCD.

ii) Facility Centres at Dheerpur Phase-II suitably modified as per site conditions

and  approved Layout plan of Jahangirpuri prepared by MCD.

iii) MRTS Corridor and Influence Zone incorporated in the plan.

iv) LSC at Kalyan Vihar deleted in view of  DDA Layout plan approved by the

Screening Committee. (prior to approved ZDP in 1998)

V)  Green strip along G.T.Karnal Road,  Libas pur  is   modified to Industrial  Use.

Annexure-1

Status of approval of sub- zonal plans as per MPD-1962:

SUB-ZONE	NAME OF THE AREA	STATUS

C-1	Kashmiri Gate (Part)	Approved

C-2	Qudsia Garden/

I.P. College	Approved

C-3	Civil Lines	Approved

C-4	Old Subzi Mandi	Draft

C-5	Old Subzi Mandi	Draft

C-6	Malka Ganj (West)	Draft

C-7	East Malka Ganj	Draft

C-8	Roshanara Garden	Approved

C-9	Shakti Nagar	Draft

C-10	Roop Nagar, Kamla Nagar

Jawahar Nagar	Draft

C-11	Northern Ridge	Approved

C-12	Old Secretariat	Approved

C-13	University Area	Draft

C-14	Timarpur	Approved

C-15	Kingsway Camp		Approved

C-16	Vijay Nagar	Approved

C-17	Rana Partap Nagar	Approved

C-18	Tripoli Area	Approved

C-19	Model Town	Approved

C-20	Adarsh Nagar	Draft

C-21	Samaypur Badli	New Sub-Zone

##### Annexure-2

###### Notified Villages

| S.NO | Village | Revenue Distt. | MCD Zone | Notification u/s 507 | Notification u/s 22(1) |
| --- | --- | --- | --- | --- | --- |
| 1. | Azadpur | North-West | Civil lines | RNZ/526 dt.13.6.1963 | 133021/3/70-LI dt. 20.8.1974 |
| 2. | Bharola | North-West | North | No.F9(2)/66/LAW/Corpn. Dt. 28.5.66 | 133021/3/70-LI dt. 20.8.1974 |
| 3. | Dhaka | North-West | Civil lines | RNZ/526 dt.13.6.1963 | 133021/3/70-LI dt. 20.8.1974 |
| 4. | Dhirpur | North-West | North | No.F9(2)/66/LAW/Corpn. Dt. 28.5.66 | 133021/3/70-LI dt. 20.8.1974 |
| 5. | Malikpur Chawni | North-West | North | No.F9(2)/66/LAW/Corpn. Dt. 28.5.66 | Yet to be notified u/s 22 |
| 6. | Pipalthala | North-West | North | No.F9(2)/66/LAW/Corpn. Dt. 28.5.66 | 133021/3/70-LI dt. 20.8.1974 |
| 7. | Samaypur | North-West | North | TCO/82/47 dt.23.4.82 | NO.9(4)/89-L&B dt. 5.10.89 |

Source: GNCTD Website

##### Annexure-3

Pre-1962 Built up Residential & Rehabilitation Colonies as  per MPD-2021:

| ###### S.NO | Name of the Colony | ##### Sub-zone |
| --- | --- | --- |
| 1. | Kamla Nagar | C-10 |
| 2. | Kingsway Camp | C-15 |
| 3. | Malka Ganj | C-6 |
| 4. | Model Town | C-19 |
| 5. | Nicholson Marg | C-1 |
| 6. | Outram Line | C-15 |
| 7. | Rana Pratap Bagh | C-17 |
| 8. | Roop Nagar | C-10 |
| 9. | Shakti Nagar | C-9 |
| 10. | Timar Pur | C-14 |
| 11. | Vijay Nagar | C-16 |
| 12. | Hudson Line | C-16 |

Source: MPD-2021

##### Annexure-4

###### Resettlement Colonies  in Zone-C

| S.NO | Name of the Colony | Sub-zone |
| --- | --- | --- |
| 1. | Jahangirpuri | C-20 |
| 2. | Nehru Vihar | C-14 |
| 3. | Gandhi Vihar | C-15 |

Source: Slum & JJ Wing, MCD

Annexure-5

Regularized  Unauthorized Colonies in Zone-C

| S.NO | Reg. unauth. colony | Year of Regularisation | sub-zone |
| --- | --- | --- | --- |
| 1. | Village Peepal Thala Ext. | 1977 | C-20 |
| 2. | Village Bharola Ext. | 1977 | C-20 |
| 3. | Village Azadpur Ext. | 1977 | C-18 |
| 4. | Kabir Basti near Sohan Ganj | 1977 | C-6 |

Source:GOI, Ministry of works & Housing notification dated 16.2.1977.

Regularized  Un authorized Colonies in Zone-C

| S.NO | Reg. unauth. Colony | Year of Regularisation | sub-zone |
| --- | --- | --- | --- |
| 1. | Mohan Park | 1978 | C-19 |
| 2. | Nirankari Colony | 1979 | C-15 |
| 3. | Rameshwar Nagar | 1979 | C-19 |
| 4. | Gupta Colony Ext. | 1979 | C-17 |
| 5. | Sardar Nagar near C.C.Colony | 1979 | C-17 |
| 6. | Majlis Park | 1980 | C-20 |
| 7. | Adarsh Nagar | 1980 | C-20 |
| 8. | Kewal Park & Gupta Nagar | 1980 | C-20 |
| 9. | Adarsh Nagar Ext. | 1980 | C-20 |
| 10. | Majlis Park Ext. | 1980 | C-20 |
| 11. | Kewal Park Ext. | 1980 | C-20 |
| 12. | Rishab Nagar Mohan Park | 1981 | C-19 |
| 13. | Gupta Colony Old | 1981 | C-17 |

Source: MCD Notification no.F.33/Engg. TP(DP)/1124/94 dated 24.10.1994.

#### Annexure-5(ii)

Un auth.  Colonies in  Zone-C as per the list of 1520  Unauth. Colonies given by GNCTD

| S.no | Name of the un authorized Colony | Si.No. as per GNCTD | Reg. No. as per GNCTD | sub-zone |
| --- | --- | --- | --- | --- |
| 1. | F-block Yadav Nagar | 556 | 34 | C-21 |
| 2. | Shiv Puri, near Badli Railway Station | 251 | 460 | C-21 |
| 3. | Teachers Colony, Samaypur viii, delhi-42* | 469 | 285 | C-21 |
| 4. | Mohan Park B-block, Model town | 191 | 698 | C-19 |
| 5. | Munshi Ram colony , near Mukherji Nagar,Delhi-9 | 1418 | 1032 | C-15 |
| 6. | Dhaka Johar | 25 | 1457 | C-15 |
| 7. | Raja Vihar Colony, Siraspur, Delhi –42* | 1351 | 407 | C-21 |
| 8. | Adarsh Nagar & Sarai Pipal Thala Extn.,Park-II, Delhi-33 | 143 | 362 | C-20 |
| 9. | Bhagat Singh Park Extn.Siraspur Village Delhi-42* | 858 | 1225 | C-21 |
| 10. | Bhagat Singh Park, Siras pur, Delhi-42* | 465 | 337 | C-21 |
| 11. | Chandan Park, Near Saras Pur, Delhi-42* | 431 | 639 | C-21 |
| 12. | Christian Colony, Patel Chest, Delhi- 7 | 645 | 487 | C-13 |
| 13. | Dhir Pur Enclave Near Satnam Bandh Delhi-9 | 947 | 1397 |  |
| 14. | Jeewan Park, Siras Pur, Delhi-42* | 57 | 758 | C-21 |
| 15. | Ladakh Budh Vihar Colony* | 1201 | 1 |  |
| 16. | Mahendru Enclave C & D Block G.T.Karnal Road Delhi-33 | 1243 | 1052 | C-18 |
| 17. | Mahendru Enclave, Model Town, Delhi-9 | 851 | 1150 | C-18 |
| 18. | Mahindru Enclave, Gali No. 3, G.T.Road, Delhi-33 | 928 | 1281 | C-18 |
| 19. | Munsi Ram Colony, Timarpur, Delhi-19 | 695 | 1209 | C-15 |
| 20. | New Aruna Nagar Colony ( Majnu ka Tilla) Delhi-54 | 1263 | 111 | C-14 |
| 21. | Parwana Lane, Under Hill Road, Civil Lines, New Delhi-54 | 9 | 795 | C-3 |
| 22. | Ram Garh, Jahangirpuri Delhi | 696 | 1210 | C-20 |
| 23. | Saroop Nagar Block-ABCDEFGH H1, I G.T.Road Libaspur* | 1287 | 905 | C-21 |
| 24. | Saroop Nagar Extn. WXYZ Block, Pt.-II,East Village, Libaspur* | 1286 | 904 | C-21 |
| 25. | Swarg Ashram Mandir, Swarg Ashram Colony, Kinsway Camp, Delhi |  |  |  |

Source: GNCTD list, Feb.2005.

1.Unauthorized colonies (except those inhabited by affluent sections of society) shall be regularized as per

Government Orders issued from time to time.

2.Affluent Unauthorized Colony- Mahendru Enclave (sub-zone C-18 ) shall be dealt as per the

separate policy for regularization of Affluent Unauthorized Colonies.

* Unauth. Colonies on the fringe area of Zone-C are partly in Zone-C, Zone-P-II & Zone-O.

Annexure-6

Mixed use streets in Zone-C as per MOUD,GOI notification dated 7.9.2006

| S.NO | Name of the Street | Length in Km. |
| --- | --- | --- |
| 1. | Roshanara Road | 1.00 |
| 2. | G.T.Road | 2.18 |
| 3. | Shakti Marg(Nangia Park round about) | 0.32 |
| 4. | Satyawati Road | 0.15 |
| 5. | Mandella Road | 0.06 |
| 6. | Kolhapur Road | 0.06 |
| 7. | Malka Ganj Road | 0.88 |

Source: MOUD,GOI notification dated 7.9.2006

Annexure-7

Commercial Streets as per GNCTD Notification No.149 dated 15.9.2006

| Sl. No. | Name of the Road/ Street | Stretch of Road | ROW (Mtrs.) |
| --- | --- | --- | --- |
| 1. | G.T.K. Road | From Govt. School Red Light to Azadpur Bus Terminal (excluding unauthorized area) | 45.00 |
| 2. | G.T.K. Road | Azadpur Bus Terminal to Shah Alam Bandh Road | 60.00 |
| 3 | Mall Road from Khybar Pass to Azad Pur Village | Mall Road from Khybar Pass to Azad Pur Village | 60.00 |
| 4. | Sarai Phoos Road (G.T.Road) | Baraf Khana Road to Rajinder Market | 30.00 |
| 5. | Gokhale Road | Metro Station to Zorawar Singh Marg Mor Gate Round About | 24.00 |
| 6. | Jahangirpur Main Road | Red Light to BJRM Hospital | 24.00 |
| 7. | Kingsway Camp Road | From Patel Chest to Dheerpur School | 30.00 |
| 8. | G.T.Road | Clock Tower to Najafgarh Drain | 30.00 |
| 9. | Lothian Road | From Lothian Bridge (Railway Bridge ) to Kashmere Gate Metro Station | 45.00 |
| 10. | Hamilton Road | From Lothian Road to Rajendra Market | 30.00 |
| 11. | B-Block Jahangirpur Streets | B-1001 to B-1050 | 9.00 |
| 12. | C-Block Jahangirpur Streets | B-1001 to B-1050 | 9.00 |
| 13. | K-Block Jahangirpur Main Road | K-1001 to K-2000 | 9.00 |
| 14. | BC-Block Jahangirpur Dividing Road | C-100 to C-2000 | 9.00 |
| 15. | IJ-Block Jahangirpur Dividing Road | I-50 to I-1951 | 9.00 |
| 16. | Main Road, Sanjay Nagar Jahangirpuri | G.T.K. Road, Krishna Sweet, H-Block Dispensary | 9.00 |
| 17. | Main Road Mahindra Park | G.T.K. Road I-27 to 4-Block Jahangirpuri | 9.00 |
| 18. | D-Block & E-Block, Connection Road, Nehru Vihar | D-1, Nehru Vihar to E-224 Nehru Vihar | 9.00 |
| 19. | Road from G.T. Road to Railway Line Azadpur Village | Azadpur Terminal to Railway Line | 13.50 |
| 20. | Piarey Lal Kaphi Road, Majlish Park, A-Block | S.S.Rana Road to A-10-11, Majlish Park | 13.50 |
| 21. | S.S.Rana Road, Kewal Park Extn to Majlish Park | Road No. 51 to Arya Samaj Road | 13.50 |
| 22. | Rajan Babu Road | G.T.K. Road to 80 ft. Road | 54.50 |
| 23. | Church Road | Madarsa Road to Lothain Road | 14.00 |
| 24. | A-Block Central Park Road, Nehru Vihar | A-202, Nehru Vihar to Senior Secondary School, Nehru Vihar | 13.50 |
| 25. | Link Road between Village Samaypur and Village Siraspur | Samaypur Chowk to Siraspur Village | 13.50 |
| 26. | Auchandi Road Samaypur | SGT Nagar to Canal | 24.00 |

Mixed Landuse Streets as per GNCTD Notification No.149 dated 15.9.2006

| Sl. No. | Name of the Road/ Street | Stretch of Road | ROW (Mtrs.) |
| --- | --- | --- | --- |
| 1. | A-Block Dividing Road Jahangirpuri | H. No. A-1001 to A-1050 | 9.00 |
| 2. | Peripheral Road E-Blk. | H. No. E-277 to E-19 | 7.00 |
| 3. | Peripheral Road C-Blk. | D-297 to C-31 | 7.00 |
| 4. | Main Road Gopal Pur Village | Road No. 50 to Flood Office | 7.50 |
| 5. | B-Block Jahangirpuri Streets | B-951 to B-1000 | 9.00 |
| 6. | C-Block Jahangirpuri Streets | B-951 to B-1000 | 9.00 |
| 7. | I-Block, Ramgarh Jahangirpur Road | I-1 to I-2000 | 9.00 |
| 8. | D-Block, Jahangirpur Dividing Road | D-951 to D-1000 | 9.00 |
| 9. | D-Block, Jahangirpur Dividing Road | D-1001 to D-1050 | 9.00 |
| 10. | E-Block Jahangirpur Dividing Road | E-1000 to E-1050 | 9.00 |
| 11. | DE-Block Jahangirpur Dividing Road | E-100 to E-2000 | 9.00 |
| 12. | Ashok Road Majlish Park | S.S.Rana Road to 80 ft. Road | 9.00 |
| 13. | Nehru Road | Arya Samaj Road to Rajan Babu Road | 9.00 |
| 14. | Dividing Road Mahendra Park | H-21, Mahendra Park to G.T.Road | 10.50 |
| 15. | Kabir Basti Road | Tanga Stand to Dhalao Kabir Basti | 9.00 |
| 16. | Arya Samaj Road | Arya Samaj Mandir to N-Block Last | 9.00 |
| 17. | Ambedkar Park Road | Q-Block, Malka Ganj, Ambedkar Park | 8.00 |
| 18. | A-Block & B-Block Nehru Vihar Dividing Road | B-1 Nehru Vihar to E-224 Nehru Vihar | 9.00 |
| 19. | D-Block & E-Block Nehru Vihar Dividing Road | E-1 Nehru Vihar to Family Home Nehru Vihar | 9.00 |
| 20. | Pardhan Road, Nirankari City | Burari Road to House No. 7/4, Nirankari City | 13.50 |
| 21. | Pole No. 5, Nirankari City | Pardhan Road to Avtar Marg | 12.00 |
| 22. | Dividing Road to Sanjay Nagar and Jahangirpur | Kushal Cinema Road to Shah Alam Bandh Road | 13.50 |
| 23. | C&D Block Dividing Road upto E-Block | C-1, Nehru Vihar to E-213, Nehru Vihar | 13.50 |
| 24. | Sukkar Bazar Road, Malka Ganj | Malka Ganj Road to HRH Stairs | 9.00 |
| 25. | Main Road E-EE Block Jahangirpuri | From Fire Station to Pump House | 18.00 |
| 26. | H-Block, Mangal Bazar Road in Jahangirpuri | Kushal Cinema Road to Shah Alam Bandh Road | 18.00 |
| 27. | Nehru Vihar Peripheral Road | Sunny Building Material to Mother Dairy | 18.00 |
| 28. | Nehru Vihar Peripheral Road | Sunny Building Material to A-1 Nehru Vihar | 18.00 |
| 29. | Ring Road | G.T.K. Road to Railway Line behind Azadpur Village (LHS) | 64.00 |
| 30. | G.T. Road | Shah Alam Bandh to Sanjay Enclave G.T.K. Depot (excluding unauthorized area) | 60.00 |
| 31. | G-Block, Dhobi Ghat Road, Jahangirpur | Kushal Cinema Road to Shah Alam Bandh Road | 24.00 |
| 32. | Gali No. 6, Libaspur | Main Libaspur Road to Shiv Mandir | 9.00 |
| 33. | Mandir Mohalla Road, Samaypur | Auchandi Road near State Bank to MCD Store | 9.00 |
| 34. | Main Road Libaspur | G.T. Road to Village Libaspur | 13.50 |
| 35. | Lane from G.T.Road 1-A G.T. Road to 198, Community Centre, Azadpur Village | 1-A G.T. Road to 198 Azadpur Nagar | 5.00 |
| 36. | Main G.T. Road Azadpur Village 55/1 to 460 Azadpur Village | 55/1 to 460 Azadpur Village | 5.00 |

Source: GNCTD Notification No.149 dated 15.9.2006

Note: In case there is difference between ROW mentioned in the list given by GNCTD and  the Zonal Plan, the ROW given in the  Zonal Plan / approved Layout plan shall prevail.

In case of  any clarification, GNCTD Notification may be referred.

Annexure-8

Commercial Streets in Civil Lines Zone as per Notification No.F.13/46/2006-UD/ 5754 dated 12.4.2007

| Sl. No. | Name of the Road/ Street | Stretch of Road | ROW (Mtrs.) |
| --- | --- | --- | --- |
| 1. | Sarai Pipal Thala Extn. Main Road (Swami Vivekanand Model School) | H. No. A-9 to B-11 | 9.00 |
| 2. | Main Road Gandhi Vihar passing through E&D Block Jan Suvidha Complex & C Block | E-293 to E-19 | 9.00 |
| 3. | A-Block Shukar Bazar Road (from main road to Mahendra Park one side) | A-1820 to A-180 | 9.00 |
| 4. | Main road of New Gupta Colony | A-28 to B-51 | 9.00 |
| 5. | Internal Road, Sindhora Kalan | S.K. 44 to SK-54A (front land one side only) | 9.00 |
| 6. | Main Road of Sindhora Kalan | Park to Dharamshala | 9.00 |
| 7. | Chopal Road, New Chandrawal Village | 5574A on Bhim Sain Marg to 5802 in Gali No. 7 | 6.70 |
| 8. | Internal Road, New Chandrawal Village | 5782 on Gali No. 7 to 5611 on Gali No. 0 (one side) | 12.00 |
| 9. | Kolhapur Road, Kamla Nagar | Mandelia Chowk to Roshanara Road | 12.00 |

Mixed Land Use Streets in Civil Lines Zone as per Notification No.F.13/46/2006-UD/ 5754 dated 12.4.2007

| Sl. No. | Name of the Road/ Street | Stretch of Road | ROW (Mtrs.) |
| --- | --- | --- | --- |
| 1. | Village Bharola | H. No. B-1 to B-12 (one side) | 13.5 |
| 2. | Road from Mandi Marg to Power House Road | Mandi Marg to Power House Road | 6.0 |
| 3. | Main dividing road A&B Block in Gandhi Vihar | D-31 to B-1 | 9.0 |
| 4. | Internal Road, Sindora Kalan | SK-38 to SK-35 | 9.0 |
| 5. | Internal Road, New Chandrawal Village | 5595 to 5574 | 12.10 |

Pedestrian Commercial Shopping Streets in Civil Lines Zone as per Notification No.F.13/46/2006-UD/ 5754 dated 12.4.2007

| Sl. No. | Name of the Road/ Street | Stretch of Road | ROW (Mtrs.) |
| --- | --- | --- | --- |
| 1. | Village Bharola | A-1 to A-26 (one side) | 5.4 |
| 2. | Village Bharola (Balmiki Mandir Wali Gali) | A-118 to A-124 | 5.0 |
| 3. | Village Bharola | H. No. 31 to H. No. 125 | 5.0 |
| 4. | Village Bharola | 43/1 to H. No. 2 | 5.0 |
| 5. | Gali No. 6, New Chandrawal Village | 5775 to 5814-5815/6 | 5.50 |
| 6. | Gali No. 5, New Chandrawal Village | 5760 to 5544 | 6.60 to 5.10 |
| 7. | Gali No. 4, New Chandrawal Village | 5557 to 5567A | 2.4 to 4.4 |
| 8. | Gali No. 3, New Chandrawal Village | 5574 to 5749 | 3.65 to 4.50 |

Pedestrian Mixed Land Use Streets in Civil Lines Zone as per Notification No.F.13/46/2006-UD/ 5754 dated 12.4.2007

| Sl. No. | Name of the Road/ Street | Stretch of Road | ROW (Mtrs.) |
| --- | --- | --- | --- |
| 1. | Village Bharola | H. No. 37 to H. No. 48 (one side) | 5.4 |
| 2. | Village Bharola | H. No. 70 to Entrance Road of APMC | 5.0 |
| 3. | Village Azadpur | 468-A to B-497 | 5.0 |
| 4. | Village Azadpur | H. No. 351 to G.T. Road | 5.0 |
| 5. | Gali No. 2, New Chandrawal Village | 5660 to 5575 | 2.40 to 4.45 |
| 6. | Gali No. 1, New Chandrawal Village | 5595 to 5620 | 2.85 to 3.70 |
| 7. | Gali No. 0, New Chandrawal Village | 5595 to 5611 | 4.30 to 4.45 |
| 8. | Internal Road, New Chandrawal village | 5824 on Gali No. 7 to 5811 on Gali No.6 | 2.80 |
| 9. | Internal Road, New Chandrawal village | 5820 on Gali No.7 to 5448-50 Balmiki Basti | 2.0 |
| 10. | Back lane of Jatav Basti Community hall, New Chandrawal village | 5340 / 6 to 5364 | 1.9 |

Source: GNCTD Notification No.F.13/46/2006-UD/ 5754 dated 12.4.2007

Note: In case there is difference between ROW mentioned in the list given by GNCTD and  the Zonal Plan, the ROW given in the  Zonal Plan / approved Layout plan shall prevail.

In case of  any clarification, GNCTD Notification may be referred.

Annexure-9

###### The listed heritage buildings as per the surveys conducted by DDA & INTACH

###### Churches/Chapels

1.	St. Stephen’s Church

2.	Chapel, St. Stephen’s College

3.	St. Stephen’s Hospital Chapel

4.	Chapel (Raj Niwas Marg)

5.	St. James Church

Commercial:

1.	Exchange Stores

Gardens/Parks:

1.	Maharaja Agrasen Park

Gateway:

1.	Tripolia Gateways

2.	Kashmere Gate

Graveyards/Cemeteries:

1.	Rajpura Cemetery

2.	Nicholson’s Cemetery

3.	Cemetery (Kashmere Gate)

Memorials/Commemorative Columns:

1.	Coronation Memorial

2.	Coronation Park

3.	Mutiny Memorial

Miscellaneous:

1.	Principal’s Residence (Delhi University)

2.	Principal’s Residence (Tis Hazari)

3.	Residential (Boulevard Road)

4.	Residence (Civil Lines)

5.	Residence (Kashmere Gate)

6.	Residence (Kashmere Gate)

Official Buildings:

1.	Delhi University Office

2.	Old Secretariat

Post Offices:

1.	Post Office, Kashmere Gate

Rest Houses/Hotels:

1.	Hotel Oberoi Maidens

2.	Canal Rest House

Schools and Colleges:

1.	Faculty of Arts

2.	St. Stephen’s College

3.	Indraprastha College for Women

Walls/Enclosures etc:

1.        City Wall, East of Kashmere Gate

Source : DELHI : The Built  Heritage, INTACH Delhi Chapter.

Annexure-10

Centrally Protected Monuments under Delhi Circle, Archeological Survey of India

| S.NO | Monument & Locality | Name of the Monument | Gazzette notification no. |
| --- | --- | --- | --- |
| 1. | Ashoka Pillar south of Hindu Rao Hospital |  |  |
| 2. | Badli ki Sarai, Gateways .Village Pipalthala,Badli | The Gateways of Badli ki Sarai | 2541 Edu.dt.17.04.1915 3588 Edu. Dt. 31.05.1915 |
| 3. | Baoli(Old) Ridge,Delhi | Old Baoli,immediately to the west of Hindu Rao’s House | 7911 Edu.dt.23.10.1913 9058 Edu. Dt. 11.12.1913 |
| 4. | Chauburji, Delhi | Chauburji, situated between Mosque,the Flag Staff and Hindu Roa’s House on the Ridge, Delhi. | 60 Edu. Dt.09.10.1912 3920 Edu.dt.21.07.1916 |
| 5. | Kashmeri Gate, Civil lines | Kashmere Gate and portion of the City wall on either side of Kashmere Gate i.e.,from Mori Gate to Kashmiri Gate on one side and on the other upto and including the water bastion at the northern most corner of the wall and also including the ditch outside the city wall where this is exposed. | 4948 Edu.dt.06.07.1927 6535 Edu.dt.13.09.1927. |
| 6. | Lothian Road Cemetery, Kashmiri Gate | Lothian Road Cemetery | 5956 Edu.dt.05.10.1922 7331 Edu.dt.13.12.1922 |
| 7. | Lt. Edwards Grave, Northern Ridge,near Flagstaff Tower, Civil Lines | Enclosure containing the grave of Lt. Edwards and others, murdered in 1857 | 858 Edu. Dt. 10.02.1926 1727 Edu.dt.20.03.1926 |
| 8. | Mutiny Telegraph Memorial | Mutiny Telegraph Memorial in front of Old Telegraph Building Kashmiri Gate, Delhi | 6368 Edu.dt.28.07.1936 B-1(19)/36 Edu.dt.14.10.1936 |
| 9. | Nicholson Cemetery, Kashmiri Gate | Nicholson or Kashmiri Gate Cemetery | 5956 Edu. Dt.05.10.1922 7331 Edu.dt.13.12.1922 |
| 10. | Nicholson statue, outside Kashmiri Gate, Delhi | Nicholson Statue and its platform and the surrounding, paths and enclosure wall. | 7911 Edu.dt.23.10.1913 |
| 11. | Qudsia Garden, North of Kashmiri Gate | The Old entrance gateway of the garden | 7911 Edu.dt.23.10.1913 9058 Edu. Dt. 11.12.1913 |
| 12. | Qudsia Garden, North of Kashmiri Gate | The Mosque | 7911 Edu.dt.23.10.1913 9058 Edu. Dt. 11.12.1913 |
| 13. | Siege Battery, East of Police Line | Site of Siege Battery | 5611 Edu.dt.16.09.1925 |
| 14. | Siege Battery, Compound of House No.7, Court Road Kashmiri Gate | Site of Siege Battery | 5611 Edu.dt.16.09.1925 6558 Edu.dt.30.10.1925 |
| 15. | Siege Battery | Site of Siege Battery bearing following inscription:”No.1 Battery-Left, Major Frank Turner, R.A.Commanding. Armament four 24-pounders. To silence Kashmir Bastion”. | 5611 Edu.dt.16.09.1925 6558 Edu.dt.30.10.1925 |
| 16. | Siege Battery (Sammy House)300 yards East of Mutiny memorial, Civil Lines, Kashmiri Gate. | Site of Siege Battery, known as Sammy House Battery | 5611 Edu.dt.16.09.1925 6558 Edu.dt.30.10.1925 |
| 17. | Siege Battery in the garden near south west Kashmiri Gate | Site of Siege Battery | 5611 Edu.dt.16.09.1925 6558 Edu.dt.30.10.1925 |
| 18. | Tripolia Gateways, on the Delhi Karnal Road | Tripolia Gateways | 3827 Edu.dt.02.06.1916 4657 Edu.dt.07.07.1916 |

Source: Archaeological Survey of India.

Annexure-11

Monuments notified by Department of Archaeology, GNCTD

| S.NO | Name of the Monument | Location |
| --- | --- | --- |
| 1. | Mutiny Memorial | Northern Ridge near Hindu Rao Hospital |
| 2. | Dara Shikoh Library Building | Kashmiri Gate |
| 3. | Maqbara Paik | Near G.T.K. Depot |
| 4. | Northern Guard House | Northern Ridge |
| 5. | Southern Guard House | Northern Ridge |
| 6. | Pavillion | Near Sabzi Mandi Rly. Stn. |
| 7. | Gateway of Mahaldar Khan’s Garden | Near Najafgarh drain Shakti Nagar and Tripolia Gate |
| 8. | Tomb unknown | On G.T.Karnal Road near Gurudwara Nanak Piao. |
| 9. | Hindu Rao’s House | Inside Hindu Rao Hospital Ridge Road, Civil Lines. |
| 10. | Metcalfe House | Inside the DRDO Office, near Old Secretariat |

Source: Department of Archeology, GNCTD

Annexure- 12

Water bodies in Zone- C

| SI.NO. | Name of the Village/ Census Town (CT) | No. of Ponds | Land Ownership | Surface Area (SQ.M) |
| --- | --- | --- | --- | --- |
| 1. | Gopalpur | a | DDA | 1,000 |
| 2. | Bhalaswa Jhangirpuri | a | DDA | 8,000 |
| 3. | Naini Lake | a | DDA | 21,000 |
| 4. | Jahangirpuri Marshes | a | DJB | 7,40,000 |
| 5. | Samaypur | a | BDO | 800 |
| 6. | Dhirpur | a | DDA | 50,000 |
|  |  | b | DDA | 10,000 |
|  |  | c | DDA | 10,000 |
|  |  | d | DDA | 2000 |
| 7. | Najafgarh Drain | a | BDO |  |

As per MPD-2021, Water bodies which are more than 1 hac. need to be conserved.

Source: MCD