###### DRAFT

ZONAL DEVELOPMENT PLAN

FOR

ZONE – “H”

(NORTH WEST DELHI-I)

APPROVED BY AUTHOURITY ON 19-11-2007 VIDE ITEM NO -100/2007

(FILE NO F4 (4) MP/2006)

AREA PLANNING - II

DELHI DEVELOPMENT AUTHORITY

### ZONAL DEVELOPMENT PLAN FOR ZONE ‘H’

##### CONTENTS

| SL. NO. | CHAPTER | PAGE NO |
| --- | --- | --- |
| 1.0 | Introduction |  |
| 1.1 | Background , Location, Boundaries & Areas | 4 |
| 1.2 | Statutory Provisions & Objectives | 4 |
| 1.3 | Earlier approved ZPD proposals | 4 |
| 1.4 | Special characteristics of the Zone | 5 |
| 2.0 | Zonal Development Plan Proposals | 6 |
| 2.1 | Population & Employment | 6 |
| 2.2 | Residential Development | 7 |
| 2.2.1 | * Redevelopment of existing areas-Planned Areas | 8 |
| 2.2.2 | * Unplanned areas: Special areas including villages and unauthorized colonies, Regularized unauthorized colonies | 9 |
| 2.2.3 | * Mixed land use | 9 |
| 2.2.4 | * Housing for the Urban poor | 10 |
| 2.2.5 | * Redevelopment Area along MRTS corridors | 10 |
| 2.3 | Work Centres | 10 |
| 2.3.1 | * Industry | 10 |
| 2.3.2 | * Trade and commerce, Wholesale markets | 11 |
| 2.3.3 | * Informal Trade and Commerce | 12 |
| 2.3.4 | * Government Offices | 12 |
| 2.4 | Social Infrastructure | 13 |
| 2.4.1 | * Health | 13 |
| 2.4.2 | * Education | 14 |
| 2.4.3 | * Sports facilities | 14 |
| 2.4.4 | * Communications | 14 |
| 2.4.5 | * Security and Safety | 14 |
| 2.4.6 | * Disaster Management Centre | 15 |
| 2.4.7 | * Distributive | 15 |
| 2.4.8 | * Other Community Facilities | 15 |
| 2.5 | Transportation | 16 |
| 2.5.1 | * Network Road, Rail | 16 |
| 2.5.2 | *Road Development Plan | 17 |
| 2.5.3 | * Road Network not implemented | 17 |
| 2.5.4 | *Grade Separators | 17 |
| 2.5.5 | * MRTS corridor | 17 |
| 2.5.6 | *Integrated Multi-Modal Transport System | 18 |
| 2.5.7 | *Bus Terminals | 18 |
| 2.5.8 | *Railway Stations | 18 |
| 2.5.9 | Goods Movement by Rail | 18 |
| 2.5.10 | Cycle Tracks & Pedestrian Corridor | 18 |
| 2.5.11 | Fuel Stations | 19 |
| 2.5.12 | Parking | 19 |
| 2.6 | Environment | 21 |
| 2.6.1 | * System of Open Spaces | 21 |
| 2.6.2 | *Protected Forest Area | 21 |
| 2.6.3 | * Multipurpose Grounds | 22 |
| 2.6.4 | * Water Bodies | 22 |
| 2.7 | Areas for urban design | 22 |
| 2.8 | Conservation of built heritage | 22 |
| 2.9 | Physical Infrastructure | 23 |
| 2.9.1 | * Water Supply | 23 |
| 2.9.2 | * Sewerage | 23 |
| 2.9.3 | * Drainage | 23 |
| 2.9.4 | * Solid Waste | 23 |
| 2.9.5 | * Electricity | 23 |
| 2.10 | Proposed Land use | 24 |
| 2.10.1 | Change of Land Use | 24-25 |
|  | Maps | 26-27 |
|  | Annexures 1- 18 | 28-50 |

# PREAMBLE

In accordance with the Authority’s Resolution in its meeting held on 6.9.2007, genuine pre-existing institutions, i.e. before 1.1.2006, rendering Cultural, religious (including spiritual) health care and educational services to the people, but which do not form part of the ridge or Gram Sabha or public land, shall be incorporated vis-à-vis their current land uses in the respective Zonal plans keeping in view clauses 3 and 4 of the Master plan-2021. Such institutions will, however, be subject to reasonable policy and procedual stipulations regarding Factual varification, FAR, Development Charges, Land-use, etc

- ZONAL DEVELOPMENT PLAN FOR  ZONE  ‘H’

1.1      BACKGROUND, LOCATION, BOUNDARIES AND AREAS:

Under the Master Plan for Delhi-2021 promulgated on 07.02.2007, the National Capital Territory of Delhi is divided into 15 Planning zones out of which 8 Zones are in Urban Delhi (A to H), 6 are in Urban Extension (J to N and P) and one for river Yamuna and river front area (Zone O).  Zone ‘H’ is also known as North West Delhi-I and it covers an area of 5677 Hac.

Zone H is distinctly characterized by well planned residential localities.  This zone is situated between two major railway lines viz., railway line to Ambala in the North East and the other to Rohtak in the South West.  Prestigious TV Tower, Delhi Haat, developed parks, DDA Sports Centres with International Standards, and protected forest area namely Shalimar Bagh are some of the significant features in this zone.  Rohini Project Ph-I/II(Part) forms part of this zone, mosly in zones H-7, 8 & 9.

The boundaries of the zone are as under: -

North-East:	Railway line to Karnal, Zone-C

West:               Rohini Phase-III, Zone- M

North-West:	Nangloi drain and Rithala STP, Zone-M

South-West: Railway Line to Rohtak, Zone-G

This zone has been developing since pre independence era and through the MPD-1962, MPD-2001 and now MPD-2021. As such the zone has heterogeneous character where the unplanned areas and planned areas developed under the norms of various plans co-exist.

  - STATUTORY PROVISIONS AND OBJECTIVES:

1.2.1 Already approved Sub-Zonal /earlier Zonal Plans in conformity with the Master Plan shall continue for the areas where the Zonal Plans have not been approved. The Zonal Plans in the form of structure plans shall be prepared within 12 months of the approval of MPD-2021.

    - The Zonal (Divisional) plan is being prepared under Section-8 and processed under Section-1O of the Delhi Development Act-1957. The modification of land use shall be processed under Section-11(A), simultaneously.

1.3	Earlier Approved Zonal Development Plans:

1.3.1   As per MPD-62 the zone had sub zones H-1 to H-8 (Annexure-I)

i)	Area under Sub Zone H-1 to H-6		2,980.70 Hects.

ii)	Area under Sub Zone H-7 to H-9		2,400.30 Hects.

(Rohini Ph. I&II and Mangolpuri)

Total						5,381.00 Hects.

However, as per MPD-2021 the total area of Zone ‘H’ is 5677 Hects.      (+296 Hac.)

The Zonal Plan of zone ‘H’ within the framework of MPD-2001 was approved by Govt. of India, Ministry of Urban Development vide Notification 13011/7/2006/DDIB, DT. 26.05.06.

1.4 SPECIAL CHARACTERISTICS OF THE ZONE:

The significant features of this Zone are:

1.4.1 Rohini Project (Phase- I & II)

‘ROHINI’ sub-city Project (Ph. I & II) was approved by DDA in

1980 based on MPD-1962. It is predominantly a residential project on

2497 Hac. Of land in North West Delhi within a distance of 15 Km. From

Connaught Place in continuation of Shalimar Bagh and Pitampura

Residential Schemes. The Area is situated along the Outer Ring Road

Between the two major traffic corridors- The G.T.Road with railway line

To Karnal and Rohtak Road.

1.4.2 T.V. Tower & Dilli Haat

The Major points of attraction in this zone are T.V. Tower complex. The T.V. Tower Complex is in the vicinity of District green and has a dominating feature having tremendous visual and special qualities.  An organized informal eating place, casual shopping and cultural activities centre such as Dilli Haat is coming up in the complex.

1.4.3   Netaji Subhash Place Complex and Manglam Place District Centres

Netaji Subhash Place and Manglam Place District Centres are a hub of commercial activities having shopping malls, offices, hospitals etc.

1.4.4   District Parks & DDA Sports Centres

Besides, it has District Parks, Picnic huts and Orchards in Rohini Scheme. DDA Sports Centres near T.V.Tower is one of the best Sports Centre with International Standards of Sports Facilities. DDA Sports Complexes are also situated in Ashok Vihar and Rohini.

1.4.5 Shalimar Bagh

Shalimar Bagh is a protected forest falling in this zone. It is a historical garden of the Mughal period with Sheesh Mahal as a centrally protected monument under ASI.  A Tank and a Well exist adjacent to Shalimar Bagh and are under serious detrioration. These are being protected by ASI except for the well which is under joint ownership of ASI and DDA.

1.4.6 Western Yamuna Canal

The Western Yamuna Canal, running more or less parallel to Delhi Karnal Railway Line divides zone H-4 & H-5 and forms an important feature of landscape.

1.4.7 MRTS Corridor

The 1st phase of MRTS passes through the zone covering Inder Lok to Rithala. This has improved the connectivity of this zone.

1.4.8 District Courts, Rohini

District Court (3 Hac.) has been accommodated on the junction of Outer Ring Road and Road No. 41.

1.4.9   Residential Areas

Most of the Residential Areas/colonies are well planned such as Ashok Vihar, Wazirpur, Pitampura, Shalimar Bagh and various Co-operative Societies.The Housing in this zone can be divided into the following categories:

1.  Residential (Plotted & Gr.Housing) areas developed by DDA

2.  Co-op. House Building Societies in Pitampura.

3.  Co.op.Group Housing Schemes in Pitampura & Rohini ph.I&II

4.  Plotted development for Rohini Registrant Scheme-1980.

5.  Resettlement Colonies.

6.  Pre-1962 Built up Residential & Re-habilitation colonies.

7.  Unauthorised regularized Colonies.

8.  Urbanised Villages.

1.4.10 Industrial Areas

There are planned industrial areas such as Wazirpur, Lawrence Road, and Mangolpuri.  These are developed as envisaged in the Zonal plan. As per MPD-2021, the unplanned industrial areas include Shalimar Village, Haider pur Village and Rithala Village.

- .0      ZONAL DEVELOPMENT PLAN PROPOSALS:

The Zonal plans shall detail out the policies of the Master Plan 2021 and act as a   link between the Layout plan and Master Plan. The development scheme and layout plans indicating various use premises shall confirm to the Master Plans / Zonal Plans.

  - POPULATION  AND EMPLOYMENT:

The projected population of Zone ‘H’ as per MPD- 2021 is 18, 65,000.

Based on various activities, employment centres have been identified with estimated workforce of about 7, 10,000 (Participation Rate-38.1% as per MPD-2021). The following are the major work centers:

i)   District Centre, Netaji Subhash Place

ii)   District Centre, Manglam Place

iii)  District Courts, Rohini

iv)  Wazirpur Industrial Area

v)   Lawrence Road Industrial Area

vi)  Mangolpuri Industrial Area Ph. I & II.

viii) Twin District center Rohini

RESIDENTIAL DEVELOPMENT:

This Zone is divided into two parts viz H-1 to H-6 and H-7, 8 & 9 (Rohini Scheme and Mangolpuri). Most of the land in sub-zone (H-4 & H-5) has been allotted to Co-operative Societies. In Sub-zones H-1 to H-6 there are two JJ resettlement colonies namely Wazirpur & Shakurpur and 9 urban villages. Colonies such as Ashok Vihar, Pitampura, Shalimar Bagh, and Wazirpur under residential use are developed and executed by DDA & other Govt. agencies. These colonies have recreational, shopping and other facilities.

ROHINI SCHEME - A CITY WITHIN METROPOLITAN DELHI

Rohini is predominantly a residential area planned for 8, 50,000 population. This Scheme was plannedin 1980.  In Sub-Zone H-7, 8 & 9. Land has been allotted to CGHS societies in Sector IX and XIII. A re-settlement colony namely Mangolpuri and 6 urban villages exist in sub- zone H -7, 8 & 9.

The Work centres provide employment for 3 lakh workers. Major facilities of health, education, social and cultural activities have been judiciously distributed on the main transportation routes within the scheme to ensure easy access. Intensive utilization of land has been achieved resulting in compact development to conserve land and energy resources without sacrificing the level of infrastructure. Large recreational areas have been conveniently located to provide for a variety of recreational facilities in addition to parks and playgrounds at neighbourhood level.

The gross residential area of 1413 Hect. Has been planned in a module of 100 Hect. The sector provides housing for about 60,000 populations and is self-contained in terms of essential facilities and amenities. The peripherial roads provide the public transport routes which are at not more than 500 meters distance from any part of the sector. The community centre, and four Senior Secondary Schools grouped together at the nucleus ensure equal accessibility to all facilities.

The following land use distribution was approved by Authority and it is a part of Zone-H. This has been included in Land Use Analysis table (2.10.1):-

LANDUSE DISTRIBUTION   IN ROHINI:

| Land Use | Area in Hect. | %age |
| --- | --- | --- |
| Residential (plots- 1,17,000 units Group housing -17,000) | 1,413 | 56.58 |
| Commercial | 108 | 4.35 |
| Industrial | 482 | 19.32 |
| Public & Semi Public Facilities | 126 | 5.06 |
| Circulation | 155 | 6.22 |
| Total | 2,497 | 100 |

2.2.1   REDEVELOPMENT OF THE EXISTING AREAS

In Zone-H, a large number of areas are old and are characterized by poor structural condition of buildings, sub optimal utilization of land, congestion, poor urban form, inadequate infrastructure services, lack of community facilities, etc.

The housing stock in both planned and unplanned areas can be enhanced through various approaches as per MPD-2021. Incentivised redevelopment with additional FAR has been envisaged in MPD -2021as a major element of city development covering all the areas.

2.2.1.1PLANNED AREAS:

A     Plotted / Group Housing

There are a number of plotted Residential Colonies like Ashok Vihar , Pitam Pura etc. Sarsawati Vihar, Kohat Enclave, Sainik Vihar, Anand Vihar, (zone- H-5) etc. came up under CHBS Schemes.  Pre 1962 Residential and Rehabilitation colonies include Gulabi Bagh, Bharat Nagar etc.  Co-operative Group Housing Societies (CGHS) have come up in Pitam Pura & Rohini.

The flats built by DDA, particularly those, which have become aged, may be redeveloped with permission and subject to the condition that the structural safety of other flats is not impinged. Already developed group housing inclusive of public (DDA and others), co-operative housing may be redeveloped on the basis of prescribed norms and regulations by formulating co-operative societies or self-managing communities. The funds for redevelopment shall be contributed by the residents / beneficiaries.

B    	Employer Housing

In Zone-H substantial areas were developed at low density after Independence, and have potential for densification. Employer housing such as Railway Colony, DDA staff Housing, Delhi Admn. Housing etc. has come up in   Shakur Basti, Pitam Pura, Keshav Puram, Gulabi Bagh, etc.

These are mainly government areas. In order to optimally utilize these prime lands there is need of intensive development. On a conservative estimate the present housing stock can be increased to more than double. Infrastructure enhancement and provision for additional housing can be financed from the funds generated through cross-subsidisation between commercial and residential use for EWS and LIG categories.

2.2.2    UNPLANNED AREAS:

A   Villages

There are 15 urbanized villages located in this area (Annexure– II) the Redevelopment plans for most of the villages were prepared earlier. However, during survey (Ref. Zonal Plan 2001) it has been noticed that most of the development in around the villages have come up against the village plans and Master plan land use. Lack of infrastructure and facilities have rendered them substandard.

Village Redevelopment Schemes shall be prepared for villages giving the development control parameters by the concerned local body. The redevelopment plans shall ensure that the permissibility of mixed use zoning at property or within the premise level is compatible to the predominant residential areas.

B    Unauthorized Regularised Colonies:

There are number of unauthorized colonies like Tri Nagar Group of Unauthorized colonies Shastri Nagar, Rishi Nagar Group of unauthorized colonies and other colonies in its surroundings, Sri Nagar Extension, Suraj Park, Vijay Vihar etc,. These have been regularised as per Govt. policy. Necessary steps however, shall be taken for improvement of physical and social infrastructure and the house owners in those unauthorized colonies should be asked to form societies and these should come forward with plans for improvement of the target group. The unauthorized pockets which have not been regularised shall be dealt with as per the Govt. policy.

2.2.2.1 FACILITIES IN UNPLANNED AREAS

As Special permission from the Authority or as per approved Layout Plan, the following institutional activity / PSP facility under Residential Use Zone is permissible under Sub-Clause 8(2) of MPD-2021:

i)   Hospital (upto 100 beds)

ii)  Primary Health Centre / Family Welfare Centre / Maternity Home /

Dispensary etc.

iii) Nursing Home / Poly Clinic / Clinic / Clinical lab. Etc.

iv) Dispensary for Pet and animals

v)  Primary School / Middle School

VI) School for Mentally / Physically Challenged

Vii Technical Training Centre (I.T.I / Polytechnic / Vocational Training

Institute/ Management Institute / Teacher Training Institute, etc.)

ix) Facilities- Bus terminal, Taxi stand, milk / vegetable booths, religious premises,  vending booth, petrol /   CNG filling  pump, recreational club   , police post, police station, fire station, post office, & telegraph office aand telephone exchange.

2.2.3   MIXED LAND USE:

As per MPD-2021 provisions to meet the growing demand of commercial activities and two overcome the shortfall of available commercial space, a liberalized provision of mixed use in residential areas has been adopted adhering to the requisites of the environment, while achieving better synergy between workplace, residence and transportation. 2183 streets have been notified by the GNCTD vide notification dated 15.09.06 for local commercial and mixed use activities.again local commercial and mixed use streets were notified on 12.04.2007.Any addition/deletion in mixed landuse streets subject to amendment from compentent authourity time to time. A list of mixed land use streets falling in Zone H in Appendix III.

2.2.4 HOUSING FOR THE URBAN POOR

A	Resettlement Colonies

There are three resettlement colonies namely Wazirpur, Shakupur and Mangolpuri in this Zone. These colonies have mixed land use character and most of these have been provided with essential services. The immediate need is services i.e. water, sewage & electricity. To ensure better environment the construction of houses needs to be as per approved / standard plan. The redevelopment schemes giving emphasis on mixed land use and providing other facilities and infrastructure shall be prepared by local bodies.

B     New Housing for the urban poor

New housing should be in the form of one or two room units which would be developed through Public and private agencies through cooperative societies as per Govt. policies.

The developers of group housing shall ensure that minimum dwelling units constructed for community service personal / EWS and lower income category shall be as per norms prescribed.

C    Night Shelter

Night shelter shall be provided at areas/ location where consentration of Industrial worker / Labours/ unorganized work force is more such as in areas like Wazirpur Industrial area, Mangolpuri Industrial area and Lawrance Road Industrial area etc.

2.2.5 REDEVELOPMENT AREA ALONG MRTS CORRIDOR (INFLUENCE ZONE)

Phase I Metro corridor passing through Inderlok to Rithala is functional in this Zone. MRTS network will bring sizable urban area with in walking distance from the proposed station. Influence Zone along MRTS corridor is envisaged as intensive development Zone. The Residential area coming in the Influence Zone maximum upto 500 meter wide belt on both sides of centre line of the MRTS/ Major Transport Corridor (to be identified in consultation with GNCTD shall be designated as Infulence Zone.

The Comprehensive Plans shall be prepared for the redevelopment of the residential areas along MRTS Corridor falling in the influence Zone as per norms prescribed in MPD -2021. The Local area plans which shall be prepared for these areas shall take into consideration the following requirements:

i) Social and Physical Infrastructure requirements,

ii) Parking requirements,

- Recreational area requirements etc.

2.3	WORK CENTRES

2.3.1    INDUSTRIES

The classification of industries is proposed to be simplified with prohibited category and non- prohibited category subject to proposals regarding specific category of industries permitted in different use Zones. (Annexure- I, II, III MPD – 2021)

Further, all planned industrial areas in Zone- H are designated under a single land use category namely, industry. The standards prescribed by the pollution Control authorities would have to be met by all industrial units, in addition to specific conditions terms of number of workers and power load in specified categories.

A	Redevelopment of Existing Planned Industrial Area:

In north Delhi there are four planned industrial areas in Zone ‘H’ namely:

i)  Wazirpur

ii)  Lawrence Road

Iii) Badli

iv) Mangolpuri Phase –I / II

There is a need for modernization and up gradation of the existing industrial areas with due regard to environmental consideration.  Since most of the industrial areas are located along the Mass Public Transport Corridors, there is also a need of redevelopment.

This process of up gradation and redevelopment will need to be carried out in a planned manner, and in public-private partnership framework, in which the entrepreneurs contribute to the betterment and subsequent maintenance through suitable Operation and Maintenance arrangements.

Common Effluent Treatment Plant:  (C.E.T.P.)

On the basis of the directions of Hon’ble Supreme Court, and with in the frame work   of MPD-2021 Common Effluent Treatment plants have been identified as under:

  - Opposite Wazirpur Distt. Centre on a land between Ring Road and Railway line.

  - Wazirpur Industrial Area.

  - Mangolpuri Industrial Area (Phase –I)

B	Redevelopment of Unplanned Industrial Area:

The Govt. of Delhi has notified these  non- conforming clusters of industrial concentration having more than 70% plots in the cluster with industrial activities for redevelopment vide notification dated 5.12.2005:

1. Shalimar Village

2. Haiderpur Village

3. Rithala Village

These will be dealt further as per the actions taken for redevelopment prescribed in MPD-2021

    - TRADE AND COMMERCE

As per the MPD-2021 the five tier hierarchy of Commercial areas is envisaged to accommodate required shopping, commercial office and other service activities like cinema, hotel and restaurant and various community services and facilities in an integrated manner. Service Market and Informal Bazar need to be integrated with Disrict Centres wherever possible.  Involvement of Developers, Owners Associations in upgradation of infrastructure and built environment in Old District Centres is recommended.

In addition, some components of commercial use have been provided under mixed    use and informal sector.

A	District Centre:

There are four District centers proposed in this Zone:

i)  District centre Netaji Subhash Place (Wazirpur)

ii)  Shalimar Bagh

iii)  Rohini –Twin District Center

iv)  Manglam Place

District Centres at Shalimar Bagh and Rohini (adjoining Swarn Jayanti Park) are yet to

Be developed.

B	Community Centre:

As per the Zonal Development plan, 25 sites for community centres have been identified. Out of these Community Centre Sites are located in Industrial Pockets of Sub-zone H1 to H6 namely Lawerence Road Industrial Area, Mangolpuri Phase I & II and Wazirpur Industrial Area.

2.3.3	INFORMAL TRADE AND COMMERCE:

Service Markets / Informal Bazar

Special attention is required for the low turnover and space extensive shops for fruits and vegetables, service and repair, junk and scrap materials (kabari), building materials, automobile workshops etc.. The grouping of such activities with planned retail markets leads to conversion of shops into high profit commercial activity.

There is a need to provide for organized informal eating places with casual shopping and cultural activities etc. to be located strategically in the city for Zone ‘H’ such a informal place (Dilli Haat) is provided near T.V. Tower.

    - GOVERNMENT USE

Government of India, Govt. of NCTD and local bodies are occupying prime land in Delhi for their offices.

District Courts, Rohini at the junction of the Outer Ring Road and Road No. 41 is a major employment center on Government land. Government Lands which are underutilized and in old and dilapidated state shall be redeveloped as per norms given in MPD-2021. A large pocket of Railway land is identified near Shakur Basti. The optimum utilization of all the Government lands shall be worked out.

  - SOCIAL INFRASTRUCTURE

The facilities at the Zonal Plan level have been indicated on the plan but the facilities at the Neighbourhood level shall be worked out at the time of preparation of Local Area Plans. There is very little scope for development, but where ever redeveloment shall take place; MPD-2021 norms shall be followed.

2.4.1	HEALTH

As per MPD 2021 the Hospital have been broadly divided in five categories:

Hospital category

a)   Category A- 501 beds and above

b)    Category B- 201 beds to 500 beads

c)    Category C- 101 beds to 200 beads

d)    Category D- Upto 100 beds

e)    Other health facilities which include maternity home, nursing home, Family

WelfareCentre, Polyclinic, Pediatric centre, Geriatric centre, diagnostic centre etc.

Three General hospitals and 12 Intermediate hospitals have been provided in this Zone.

The shortage in the availability of number of beds per thousand population (as per WHO 5 beds per thousand population is proposed to be meet through.

a)   Enhancement in FAR for various levels of health facilities

b)   Promoting rebuilding of the existing old hospitals

List of Government Hospitals on sites earmarked for Sub Zone (H-1 to H-6) is as under:-

| S.No | Name of the Hospitals | Sub –Zone (H-1 to H-6) |
| --- | --- | --- |
| 1. | Bhagwan Mahavir Hospital (Existing) | H-4 |
| 2. | Shastri Park Hospital (proposed) | H-1 |
| 3. | Koki Wala Bagh, Ashok Vihar (Proposed) | H-3 |
| 4. | Keshav Puram, Lawrance Road, (Proposed) | H-2 |

Hospital Proposed in Facility centre in Sub Zone (H-1 to H-6)

| S.No | Existing / Proposed Hospitals | Sub –Zone (H-1 to H-6) |
| --- | --- | --- |
| 1. | Proposed Hospital (FC-50) (3 Hospital category A & C) | H-6 |
| 2. | Proposed Hospital (FC-53) (1 Hospital category A) | H-5 |
| 3. | Proposed Hospital (FC-57) (4 Hospital category A & C) | H-4 |

2.4.2	  EDUCATION:

A	 College

As per MPD- 2021 colleges have been broadly divided in three categories. A general college for 5 lakh population on plot area as per UGC norms. A professional college (Engineering) for a 5 lakh population area as per AICTE norms. A vocational Training Centre for 5 lakh as on area of 0.4 hact.

In this Zone 10 General colleges and 2 professional colleges (Engineering) and one ITI + Polytechnic have been provided Details given below:

| S.No | College | Professional College (Engineering) | ITI + Polytechnic | Sub Zone |
| --- | --- | --- | --- | --- |
| 1. | 2 | 1 | -- | H-3 |
| 2 | 1 | -- | -- | H-4 |
| 3 | 1 | 1 | 1 | H-5 |
| 4 | 2 | -- | -- | H-6 |
| 5 | 4 | -- | -- | H-7, H-8 & H-9 |
| Total | 10 | 2 | 1 |  |

2.4.3	SPORTS FACILITIES:

Sports facilities have been developed in the city by various agencies like Sports Authority of India (SAI) Delhi Development Authority (DDA), GNCTD etc

There are three DDA Sports Centres in this zone:

i)   Rohini, sector-9

ii)   Ashok Vihar (Major Dhyan chand Sports Complex)

iii)   Pitam Pura, Near T.V. Tower (Rashtriya Swabhiman Sports complex)

    - COMMUNICATIONS

As per MPD-2021, the sites for Telephone Exchange and other means of Communications shall not be reserved in layout Plan due to liberalization and technological advancement

In this zone 7 sites have been proposed at different locations where as two sites are required as per MPD 2021 norms. The facilities like Telephone exchange shall not be reserved due to privatizitation and change in Technology. Vacant sites will be utilized for Disaster Management Centre.

    - SECURITY & SAFETY:

2.4.5.1 Fire Stations

As per the norms three Fire stations for every 5 lakh population are to be provided in 5-7 Km. Radius.  Three sites in sub zone (H-1 to H-6) are:

  - Wazirpur Industrial Area

  - Keshavpuram

  - Magolpuri Industrial Area Phase –II

Four sites are in Rohini Zone.

More sites shall be provided to meet the requirements as per norms stipulated in MPD 2021.

2.4.5.2 Police Stations

As per MPD – 2021, a police station has been proposed for a population of 2.5 lacs and area of 1.0 Hect. A police post for a population of 1.0 lacs on an area of 1000 sqmts. Where as Eight Police stations and nineteen police post are required. Fifteen police stations / Police post sites have been shown in zone H which includes existing as well as proposed sites.

    - Disaster Management Centre:

As per MPD -2021 provision it is proposed to have Disaster Management Centre in each Administrative zone. The unutilized telephone exchange shall be utilized for this activity. In addition one each centre shall be provided in following major projects:

i)   Mangolpuri Industrial Area

ii)   Wazirpur Industrial Area

2.4.7	DISTRIBUTIVE

Gas Godown:

Two sites for service centre are shown in Zonal plan whereas six gas- godowns are provided in each service centre (Total 12 number).

No new Gas Godown sites are envisaged as Piped Gas supply network is being provided in Residential Colonies by I.P.Gas agency.

2.4.8	OTHER COMMUNITY FACILITIES

A number of public & semi public facilities have been augmented in the zone mainly in facility centres as per MPD 2001. Other facilities like Old age home, community hall, Religious, Dispensaries, Schools etc are required and shall be provided while finalising the layout plans.

Cremation Ground, Burial Ground

As per the norms given in MPD 2021, one Burial ground/ Cenetry is required for every 10,00,000 population.  The cremation ground sites in sub zone H-1 to H-6 are located at Bariwala Bagh, Haiderpur Village, JJ Colony Wazirpur. Apart from this a Burial ground also exists at Bariwala Bagh. Other Existing / prooposed cremation grounds and burial grounds in Rohini Zone are as under:

Sector-4:

i)    Burial Ground

ii)   Cemetry

iii)  Cremation Ground

Sector-5:

i) Existing Cremation Ground

JAIL

The plan provides for one jail 5.06 hect. In Sub zone H-9 near Badli.

  - TRANSPORTATION

    - ROAD NETWORK

Zone H is well connected with the Master Plan roads and with the railway network for   the intricate movement of passengers and goods. Moreover, Master Plan roads directly serving the proposed / existing facility centers and the residential hubs of the areas. The predominant movement of this zone is mainly through the existing Master Plan roads. The zone is served by outer Ring Road, the Delhi Rohtak Railway line passes through the southern fringe of this zone. The major arterial road like Ring Road is diagonally penetrating through this zone. Moreover Road No, 37 which is presently connecting to Road No. 40 and Ring Road has been further extended along the Western Yamuna Canal passing through the already built up Pitampura colony and  connecting  to the Outer Ring Road. The Road No. 37 and its extension divide the zone in two parts. Road No. 41 which connects inner ring road and outer ring road extends into Rohini and forms central arterial road of the Rohini Scheme. Road No. 43 connects the GT road in East with Mangolouri Industrial area in west, passing through the various sub zones viz H-2 to H-5. This is a major link serving. the work centers like Lawrence Road Industrial area, Mangolpuri Industrial Area and land under Facility Centers etc.

Hierarchy of Roads

The Zonal plan primarily indicates roads of 18 mtr and above right of way for smooth     flow of the traffic. The following is the hierarchy of the road system:

1)  60 M R/W

i)   Auchandi Road

ii)  Outer Ringroad

iii)   Inner Ring Road

2)  36 M to 45 M. R/W

Roads Nos. 40,41,43,37 and

Proposed Road No. 37 Extn. Along Western Yamuna Canal. Other major limks areas between sub zone H-8 and H-9

3) 24 M to 30 M R/W

With the various sub-division a number of inter connecting roads having right of way of24 M and 30 M have been provided. The plan also indicates a number of roads of 18M and above as per the approved layout.

2.5.2   Road Development Plans

All the Road owning agencies shall get Road Development Plan prepared for 30 M & above ROW Roads shown in MPD-2021 alongwith interchange details of junction / intersection falling in the algnment showing location of bus stop, metro station &  the pedestrian linkage for smmoth changeover from one mode to another.

2.5.3   Road Network not implemented

The following Road network is not implemented:

- Prem Bari Pul (Pitampura) to Outer Ring Road along disused Western

Yamuna Canal (Road No. 37 Extension).

2.5.4 Grade Separators:

MPD-2021 Indicates the need for provision of Inter- section with grade separator. In   case of existing grade separator the possibility of providing clover leaf and direct inter changes wherever necessary and feasible may be technically examined to make the junction signal free. These are to be identified at the inter section of all roads of 30 m and above Row. Following intersections with grade separators have already been identified.

- Road No. 41 and Outer Ring Road crossing (Madhuban chowk) is a major intersection and caters to very heavy traffic.

- Road No. 43 and Outer Ring Road crossing near Mangolpuri.

- Road No. 37 and Inner Ring Road crossing near Shalimar Bagh.

- Inner Ring road and the proposed 30 mtrs road near village Shakurpur (Britania Chowk).

- Outer Ring Road and Railway line crossing near Shalimar Place.

- Inner Ring Road and Railway line crossing near Wazirpur Industrial area.

- Inner Ring Road and District Center Wazirpur Crossing.

- Extended Road No. 37 and Outer Ring Road crossing near proposed Jail in Rohini.

2.5.5   Mass Rapid Transit System (MRTS)

The Metro Rail System is the most important component of a Mass Rapid Transport System (MRTS) in the City. The Metro Rail Network for the entire city has been identified in various phases, which comprises of a network of underground, elevated and surface corridors aggregating to approximately 250 Kms.Phase-I of the elevated Metro Corridor from Shahdara to Rithala in Rohini is already operational in this zone.

2.5.6     Integrated Multi – Modal Transport System:

Integrated Multi- Model Transportation proposal duly approved may be earmarked /   incorporated on the zonal plan with the help of (DIMTS) GNCTD.

It is envisaged that the future transport system shall consist of a mix of rail and road based systems which may include Metro Rail, Ring Rail, Dedicated Rail Corridors for daily commuters, (IRBT/RRTS corridors as identified in NCR plan 2021.

Other mass transit modes shall be interdouced as technologies become available. Intermediate passenger Transport (IPT) and private modes on selected corridors is to be identified as per the needs from time to time. All roads should be made pedestrians disabled and bicycle friendly as far as possible.

In zone ‘H’ Tri Nagar corridor, the Metro Station and Metro Corridors are elevated and catering to large crowd till Rithala. Preparation of detailed multi-modal transport plans with reference to each major Metro Station, with particular reference to bus transport routes, which could provide inter-linkages and feeder arrangements, is envisaged.

2.5.7     Bus Terminals:

As per norms stipulated in MPD- 2021, 21 terminals and 4 bus depots are required for the projected population of this zone. At present, five sites have been provided in Rohini and two sites in Magolpuri. In addition, one site already exists in Lawrence Road where a site for bus deports has been provided. More sites have been provided to meet with the requirement at suitable locations.

2.5.8      Railway Stations

Railway Station at Shakur Basti is a major Station in Zone-H. For Inter city movement a new Railway Station at Rampura with proper approach from main Roads is provided.

2.5.9    Goods Movement by Rail

The goods are terminated at Kela Godown near New Azadpur Mandi.

2.5.10 Cycle Tracks and Corridor for pedestrian

The pedestrians pathway/cycle tracks have been proposed along the existing Nallah and along the green / open areas. Some Master Plan roads of this zone have been identified for pedestrians and cyclist. These are required to be developed for connecting the residential colonies of this areas and to develop the environmentally well connected system for cyclists and pedestrian and at the time of detailing these pathways proper attention is needed for the development of intersection areas to regulate the vehicular movement and slow moving traffic i.e pedestrians and cyclists.

Pedestrian pathways have been proposed along the existing Nallah; green area and along the inner ring road passing through different sub zones as indicated in the Zonal Development plan.

Proper environmental design need to be developed so that it may be matched with the built environment of the residential localities. Professional attention is to be given to develop the pathways along with green so that it could not be much disturbed. In addition exclusive pathways are being proposed through district parks and neighbouring park so that pedestrians can move freely for approaching to the works centers within the areas. Detailed designs to be developed of following areas to connect pedestrian pathway to the district center at the time of preparation of local area plan.

  - Road No. 37 along western Yanuma canal and further extending in sub-zone H-9 along the existing drain connecting Auchandi Road

  - Along Road No. 43 connecting Mangolpuri and Lawrence Road works centers

And further extending to meet G.T. Road in the east.

  - Along inner Ring road connecting Wazirpur and Lawrence road works centers.

2.5.11 FUEL STATIONS

A    Petrol Pump

As per MPD 2021, about 124 fuel stations sties are required to be provided in this zone 34 petrol pump sites existing / proposed in Sub zone (H -1 to H-6) and 29 sites in sub zone H 7 , 8 & 9 (Rohini & Mangolpuri) are shown in zonal plan. However. Remaining sites are, therefore, to be located while preparing Local Area Plans and Layout Plans as per the norms laid down in the Master Plan 2021.

B    CNG Stations

To combat the increasing vehicular pollution in the city the use of CNG as vehicular fuel (both for petrol and diesel driven vehicles) have been emphasized. Realising the urgency and importance of protection and Realising the urgency of improvement of the Hon’ble Supreme Court has given order dated July 28, 1998 on control of vehicular pollution in National Capital Region including Delhi. One of the steps need to be taken is by GAIL to expedite and expand from 9 to 80 CNG Supply Outlets

CNG sites are to be located while preparing Local Area Plans and Layout Plans as per the norms laid down in the Master Plan 2021.

2.5.12 PARKING

With the phenomenal increase in personalized motor vehicles, one of the major problems being faced today is an acute shortage of parking space.Parking has become a matter of serious public concern and requires a carefully considered policy and planned measures to alleviate the problem to the maximum feasible extent in existing areas and for adequate provision with reference of future developments.A traffic & transportation study is to be done before implementing large parking lots for smooth traffic movement in the area. In this background about 100 surface parking sites have been proposed in the city out of which the following sites are in Zone- H.

i)    Shalimar Bagh,

ii)   A D Block Shalimar Bagh commercial complex,

iii)   Rajiv Gandhi Cancer Hospital Rohini,

iv)   Outside MCD zonal office Rohini, near Kala Mandir Mangolpuri.

A      Park and Ride

Park & Ride lots are available on MRTS stations like Tri Nagar, Rithala etc,

B     Public Parking

The major efforts will have to come through the creation of public facilities in designated commercial /work centers and other areas and corridors where significant commercial activity has developed by way of mixed use. In the context of the latter, if would also need to be linked to pedestrianisation within the identified areas.

C    Parking Facilities in DTC depots

The Bus depot (Wazirpur) is operating under Delhi Transport Corporation. The use of DTC Terminals and depots for development of public parking along with parking of DTC buses, private buses and chartered buses should be explored and specific projects developed supported by traffic & transportation studies.

D   Under Ground Parking

Based on the traffic and transport study, parking facilities can be created under the open spaces without disturbing the green areas in the surface and surrounding environment. The approvals form the concerned agencies are mandatory before taking up such works.

E   Parking in Residential Areas

Most of the parking is, in fact being done on the road, which significantly reduces the carriageway width. The problem has been exacerbated by the traffic congestion generated by Schools in residential use areas. Following measures may to some extent deviate the problems

- All the encroachments on residential streets in the form of Kitchen gardens/ roadside private greens, large projections/ramps, etc. need to be removed

- Road cross sections may be redesigned wherever possible to accommodate planned car parking along the residential streets, and also creating more surface movement space.

- Other options, in selected areas, such as creation of underground parking below parks and open spaces will also have to be considered.

- Resident Welfare Associations will have to be called upon to participate in this process by raising contributions from the residents on the basis of objective criteria such as number of cars owned by each family.

F   Multilevel Parking

Multilevel parking facilities should preferably be developed in the designated parking space or in the residential public semi public facilities, commercial transport mode. In MPD -2021, a multilevel parking proposed at Manglam Place. Multilevel parking facility shall also be developed in the following parking spaces subject to availability of land / feasibility by the concerned local body.

i)    Rani Bagh Market

ii)   Dilli Haat near T.V. Tower,

iii)   Mangolpuri Industrial area

iv)   Wazirpur Industrial area

v)   Lawrance Road Industrial area

vi)   Ashok Vihar

2.6       ENVIRONMENT

2.6.1	SYSTEM OF OPEN SPACES

The major green areas are supplemented by area earmarked for neighbourhood parks and play areas provided in residential area. These would server a linkage for continous green through major green areas. Both these areas have been shown in the zonal plan the table below indicate the available green area in the zone.

|  | 1 .District Park, Nurseries, Children Park, Play Grounds, Sports Centers, Divisional sports Centers etc. |
| --- | --- |
|  | 2. Neighbourhood Park and Neighbourhood Play Areas. |

2.6.2   Protected Forest Area

The plan identified the land conferred by section 30 and 32 on the Indian Forest Act 2927 vide Notification No. SCD (C) Noti. 80-81; dt. 10.04.81. As per the notification following areas have been identified as protected forests:-

- Shalimar Bagh and other orchards situated therein;

- Area between Sador Kalan, Nimri, Gulabi Bagh, Darbar Khan Nursery and other areas;

- Orchard in Wazirpur near Bharat Nagar and Nimri Colony;

- District park in between Pitampura Co-op. Housing Society

Out of four areas mentioned above Shalimar Bagh is a site of historical importance. This area is bounded by 24 m road in the North and Western Yamuna canal in the West unauthorized construction in the East and North. As this area is subject to unauthorized encroachment, development schemes/landscape plan for upgradation of the environment is the need of the time. Apart there are few orchards in Rohini scheme. These should also be protected and taken care for improvement of this area.

2.6.3 Multipurpose Grounds

The network of the multipurpose grounds as a special category for holding marriages and other special functions has been proposed. These multipurpose grounds at the Community level shall be upto 2 Hac. , district level 4 Hac. And 8 hac. at the City (zone) level.

As per the details given by Landscape unit, DDA, Some of the Zonal level and District level  Multipurpose Grounds  are:

i)  Zonal Level (8 Hac.) – Avantika, Sector-1, Rohini.                                                -- Sainik Vihar

ii)  Distict Level (4 Hac.)- District Park   & Old Orchard at Ashok Vihar

-- Lekh Ram Park, Ashok Vihar

Already approved parks on 60’ and above roads  for holding marriages by the  Local body/ MCD shall be designated as Multipurpose grounds.

2.6.4	Water Bodies

As per MPD- 2021, water Bodies having a minimum surface area of 1 ha. Shall be preserved by concerned authorities. There are 7 water bodies located in the area (Annexure – 15 & 16).Out of these 3 water bodies having area of 1 hac. and above are in Rohini.

Source : List of water Bodies prepared by MCD.

  - AREAS OF URBAN  DESIGN

A city is an assemblage of buildings and streets, system of communication and utilities, places of work, transportation, leisure and meeting palces. The process of arranging these elements both funcitionally and beautifully is the essence of Urban Design.

The aspects need to be consider to arrive at the basis for policies affecting the urban fabric namely:-

(i)Areas of significance in built environment.

- Visual integration of the city.

- Policy on unhindered access movement, parking and pedestrian realm.

- City structure plan and Urban Design objective.

The area identified for Urban Design in Zone – H is in Ashok Vihar. While traveling in Metro between Inderlok, Kanhayanagar and Keshavpuram a unique citi park has emerged in Ashok Vihar as an important land mark of North Delhi. The small parks already existed can be intergrated with open spaces. The park will have haats, fair grounds, sports facilities and jogging tracks as well as space for outdoor cultural events.

  - CONSERVATION OF BUILT HERITAGE:

    - Special Conservation Plans

Each local body/ land owning agency shall formulate "Special Development Plans" for the conservation and improvement of listed heritage complexes and their appurtenant areas. Alteration or demolition of any listed heritage building is prohibited without the prior approval of the Competent Authorities.

The development plans/schemes for such areas shall conform to the provisions, in respect of Conservation of Heritage Sites including Heritage Buildings, Heritage Precincts and Natural Feature Areas.

According to the Built Heritage, INTACH – Delhi, Many monuments of historic

importance namely Shish Mahal, with two small pavilions on the north site of Sheesh Mahal. A Tank and a Well exist in the Zone H adjacent to Shalimar Bagh police station. All these are under serious deterioration. These are being protected by ASI except for the well which is the joint ownership of ASI and DDA. The well is unprotected, however, water from the well is still used to water the gardens. Apart from this a 30 M Rubble Masonary wall about 3.0 M high located at Swami Nagar Road, Shastri Nagar under the ownership of MCD , ‘Police new Clothing Dept’ owned by Delhi Police at Old Rohtak Road are also of Heritage Importance. These are also under serious deterioration.

2.9       PHYSICAL INFRASTRUCTURE

As given under the requirement of Water, Sewerage, Power and Solid Waste Disposal for this Zone , it shall be worked out in consultation with the concerned department and the  Engineering Wing of DDA.

2.9.1      Water Supply:

No such site has been provided in zone H-1 to H-6. However, one site (42 Hect) is     existing in sub zone H-9. Rohini Residential scheme and the same is indicated on the plan. The water treatment plan would require augmentation to raise its capacity to 216 mgd. As per MPD- 2021 to provide additional water supply augmentation  of existing water treatment plants to be carried out the provision of water treatment plant to be monitored depending on  availability of raw water and need of  potable water. In zone H the Haiderpur Plant I & II need to be augmented.

2.9.2    Sewerage Treatment Plant:

The plan identifies, one site for sewage treatment plant near village Rithala. It is         spread on an area of 79.3321 hect. Apart two more sites have been reserved for sewerage treatment plant in sub zone h -8 (150 hect Mangolpuri Industrial Area). As per MPD -2021 a new plant in Rohini is to be provided.

2.9.3    Drainage:

A division has already been developed on the basis of  the layout plans. It is assumed  that the requirement of the drainage system for the planned colonies has already taken care of. However, efforts should be made by the local agencies to provide a suitable drainage  system for the unauthorized colonies, urban villages and JJ resettlement schemes, and other area where such facilities are still lacking. 	   There are two drains passing through this zone.

- Najafgarh Drain

- Nangloi Drain

As per MPD- 2021 , it is recommended to develop green along the    drains  for ground

water recharge.

2.9.4     Electricity

The zone has two 220 KV sub –station and thirteen 66 KV substations. These are     evenly spread and are indicated on the plan.

  - LAND USE PLAN:

The Zonal Development Plan has been detailed out taking into consideration the   various proposals/approved layout plans and schemes. The plan also takes into consideration the and incorporates the requirements for the community facilities, green areas, infrastructure and traffic and transportation etc. The land use analysis is as under:

2.10.1 LAND USE ANALYSIS

Taking into consideration the change of Land use cases and Zonal Development   Plan proposals/ Modifications, Land use analysis is given below:

| S.No | Land Use | Area (hac.) | Proposed % age as per ZDP- H, MPD-2001 | Change of Landuse Area Area Added Sub. + Hac. – Hac. |  | Area (Hac.) | Proposed % age |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Residential | 2756.00 | 51.22 |  |  | 2756.00 | 51.22 |
| 2. | Commercial | 239.65 | 4.45 | 5.84 |  | 245.49 | 4.56 |
| 3 | Industry | 307.93 | 5.72 |  |  | 307.93 | 5.72 |
| 4. | Recreational | 836.89 | 15.56 |  | 13.85+ 3+6.8 | 813.24 | 15.11 |
| 5 | Transportation | 473.44 | 8.79 |  |  | 473.44 | 8.79 |
| 6 | Utility | 222.35 | 4.13 |  |  | 222.35 | 4.13 |
| 7 | Government | 133.08 | 2.48 | 0.267 |  | 133.34 | 2.50 |
| 8 | PSP Facilities | 411.66 | 7.65 | 13.85 + 6.8 | 2.84+0.267 | 429.21 | 7.97 |
|  | Total | 5381.00 | 100% |  |  | 5381.00 | 100 % |

    - CHANGE OF LANDUSE (Sub- Zone H-1 to H-6)

The following CLU cases have been incorporated in the Zonal Plan and have already been notified.

| S.NO | LOCATION | AREA (Hac.) | CLU |  | STATUS |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FROM | TO |  |
| 1. | Tri Nagar | 3.00 | Recreational | Commercial | Notified as per GOI notification no.1017 dated 23.09.2005. |
| 2. | Wazirpur | 2.84 | PSP Facilities | Commercial | Notified as per GOI notification no.1017 dated 23.09.2005. |

CHANGE OF LAND USE IN ROHINI (SUB-ZONE H-7, 8, 9)

| S.NO | LOCATION | AREA (Sq.Mt.) | CLU |  | STATUS |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FROM | TO |  |
| 1. | Rohini | 2670 | PSP Facilities | Govt. Use (Courts) | Notified as per GOI notification no.8901 dated 25.08.2005. |

2.10.2 MODIFICATIONS IN MPD-2021 BEING PROCESSED UNDER SEC. 11-A OF

DD ACT, 1957 ALONGWITH ZONAL PLAN

The area of Zone-‘H’ is given as 5677 Hac. Under Sub- Clause 16.1 of MPD- 2021

And this is proposed to be modified to 5381

OTHER CHANGES TO BE INCORPORATED IN   LANDUSE PLAN, MPD- 2021

| S.No | SUB ZONE / LOCATION | LAND USE AS PER ZDP | LAND USE AS PER MPD 2021 |
| --- | --- | --- | --- |
| 1 | H- 6, Shalimar Bagh Block C&D | Residential | District- Green |
| 2 | H -5, Pitampura | District- Green | Residential |
| 3 | H –2, Lawrence Road Industrial Ten. Scheme | Residential, 0.67 Ha. | Public & Semi Public (hospital) |
| 4 | H – 9, Sector 19, Rohini | Public & Semi Public (cremation / burial ground/ cemetry) 1.46 Ha. | Residential |

Annexure- 1

Status of Zonal Plans (now Sub- Zones):	As per 1962 Master Plan, the Zone (Planning Division-H) had 8 sub zones viz. H-1 to H-8.

The following table indicates status of these sub –zonal development plans (earlier Zonal Development Plans) formulated and processed with the framework of Master Plan of Delhi 1962 with the area of each sub-zone:-

| S.NO | SUB –ZONE | AREA (Hac) | STATUS | REMARKS |
| --- | --- | --- | --- | --- |
| 1 | H1 (Gulabi Bagh) | 216.2 | Approved | Approved by Central Govt. dt. 21.11.67 |
| 2 | H2 (Lawrence Road) | 335.2 | -do- | Approved by Central Govt. dt. 21.11.72 |
| 3 | H 3 (Wazir Pur) | 575.8 | -do- | Approved by Central Govt. dt. 22.09.73 |
| 4 | H 4, 5 & 6 (Shakur basti, Pitampura & Shalimar Bagh) | 1790.26 | -do- | Approved by Central Govt. dt. 02.04.73 |
| 5 | H 7 and H 8 | 2824 | -do- | Zonal Plan not Approved pertains to Rohini Project, Mangolpuri Colony and Mangolpuri Industrial Area Phase- I & II |

Annexure- 2

LIST OF URBAN VILLAGES IN ZONE ‘H’

ZONE (H 1 TO H 6)

Sidhera Kalan

1. Rampura

2. Wazirpur

3. Shakurpur

4. Pitampura

5. Shalimar Bagh

6. Shingalpur

7. Sahipur

8. Hyderpur

ROHINI ZONE

1. Badli

2. Naharpur

3. Rithala

4. Razapur

6. Mangolpur Kalan

7. Mangolpur Khurd

Annexure -3

Mixed Land Use Streets as per Notification No. SO 1456(E) dated 07.09.06

1. Road No. 43, Rani Bagh		200 ft*.

2. Main Bazar, Rishi Nagar		 50 ft.

3. Main Bazar, (Road Rani Bagh)	 50 ft.

4. Main Bazar, Raja Park	                   100 ft.*

5. Main Road, Shastri Nagar	           200 ft.

6. Main Bazar, Tri Nagar		           50 ft.

7. Road No. 41, Rohini			 45 mt.

Source: Notification No. SO 1456(E) dated 07.09.06

* 80’ as per ZDP.

* 150’ as per ZDP.

Annexure - 4

Commercial Streets as per GNCTD Notification No. 149 dated 15.09.2006

Commercial Streets in Civil line Zone as/ MCD

| S.No | Name of the Road/ Street | Stretch of Road | Row |
| --- | --- | --- | --- |
| 1 | Internal Road FK- Block Shastri Nagar | FK -39 to FK- 51 Shastri Nagar | 9 Mtrs |
| 2 | Internal Road B- Block Shastri Nagar | B -1223 to B- 1215 Shastri Nagar | 9 Mtrs |
| 3 | Internal Road B- Block Shastri Nagar | B -1387 to B- 1555 Shastri Nagar | 9 Mtrs |
| 4 | Internal Road B- Block Shastri Nagar | B -1554 to B- 1537 Shastri Nagar | 9 Mtrs |
| 5 | Internal Road B- Block Shastri Nagar | B -1863 to B- 1548 Shastri Nagar | 9 Mtrs |
| 6 | Internal Road L- Block Shastri Nagar | L -135 to L- 125 Shastri Nagar | 8 Mtrs |
| 7 | Internal Road M- Block Shastri Nagar | M -207 to M- 217 Shastri Nagar | 7.5 Mtrs |
| 8 | Internal Road Lalita Block Shastri Nagar | DDA Park to D-27/80, Lalita Block Shastri Nagar | 7.5 Mtrs |
| 9 | Internal Road B- Block Shastri Nagar | B -1004 to B- 1020 Shastri Nagar | 7.5 Mtrs |
| 10 | Internal Road B- Block Shastri Nagar | B -1662 to B- 1665 Shastri Nagar | 7.5 Mtrs |
| 11 | Internal Road B- Block Shastri Nagar | B -1716 to B- 1729 Shastri Nagar | 7.5 Mtrs |
| 12 | Internal Road E-2 Block Shastri Nagar | E -2/225 to E-2/189 Shastri Nagar | 7 Mtrs |
| 13 | Internal Road Lalita Block Shastri Nagar | D-2734 to D-2748, Lalita Block Shastri Nagar | 7 Mtrs |
| 14 | Internal Road B- Block Shastri Nagar | B -1123 to B- 1155 Shastri Nagar | 7 Mtrs |
| 15 | Internal Road B- Block Shastri Nagar | B -1041 to B- 1155 Shastri Nagar | 7 Mtrs |
| 16 | Internal Road B- Block Shastri Nagar | B -1692 to B- 1757/6C Shastri Nagar | 7 Mtrs |
| 17 | Internal Road B- Block Shastri Nagar | B -1751 to B- 1753/3 Shastri Nagar | 7 Mtrs |
| 18 | Internal Road M- Block Shastri Nagar | M -200 to M- 78 Shastri Nagar | 6 Mtrs |
| 19 | Internal Road F- Block Shastri Nagar | F -35 to M- 7 Shastri Nagar | 6 Mtrs |
| 20 | Internal Road E Block Shastri Nagar | E -76 to E- 65 Shastri Nagar | 6 Mtrs |
| 21 | Internal Road E Block Shastri Nagar | E -131 to E- 141 Shastri Nagar | 6 Mtrs |
| 22 | Internal Road E-2 Block Shastri Nagar | E -2/153 to E-2/188 Shastri Nagar | 6 Mtrs |
| 23 | Internal Road Lalita Block Shastri Nagar | D-20 to D-5 Lalita Block Shastri Nagar | 6 Mtrs |
| 24 | Internal Road B- Block Shastri Nagar | B-945 to B-995 Shastri Nagar | 6 Mtrs |
| 25 | Internal Road B- Block Shastri Nagar | B-1106 to B-1039 Shastri Nagar | 6 Mtrs |
| 26 | Internal Road A- Block Shastri Nagar | A-41 to A-55 Shastri Nagar | 6 Mtrs |
| 27 | Internal Road A- Block Shastri Nagar | A-41 to A-55 Shastri Nagar | 6 Mtrs |
| 28 | Road No. 40-11 / 6A C-120 | Road No. 40, H.No L-11/6 A Shastri Nagar | 13.50 Mtrs |
| 29 | Kalidas Marg to WZ -106 (Petrol Pump) | Kalidas Marg to H.No WZ -106 Nimri Village | 13.50 Mtrs |
| 30 | Internal Road to A- Block Shastri Nagar | A- 758 to A-268 Shastri Nagar | 13.50 Mtrs |
| 31 | Nand Kishore Gupta Marg Shastri Nagar | E- 2/2 to B 1687 Shastri Nagar | 13.50 Mtrs |

Source: As per GNCTD Notification No. 149 dated 15.09.2006

Note:  In case there is difference between ROW mentioned in the list given by GNCTD    and the Zonal Plan, the ROW given in the Zonal Plan/ approved Layput plan shall prevail.

In case of any clarification, GNCTD Notification may by referred

Annexure – 5

Commercial Streets in Rohini Zone as/ MCD

| S.No | Name of the Road/ Street | Stretch of Road | Row |
| --- | --- | --- | --- |
| 1 | Road between G & F –block in Shakurpur | Hawaldar Abdul Hameed Marg to Maharishi Balmiki Marg | 15 Mtrs |
| 2 | G- block C/ Center in Shakurpur JJ Colony | G& H block. Road to G& F- block Road,, Shakurpur | 15 Mtrs |
| 3 | Lane Rishi Nagar | From H.No. 677 to 545 Rishi Nagar | 6 Mtrs |
| 4 | Gurudwara Road, Wazirpur JJ Colony | Bharat Nagar Road to Wazirpur JJ Colony | 14 Mtrs |
| 5 | Internal Road of Wazirpur Village (Police Colony) | Shiv Market to Yadav Hotel | 6 Mtrs |
| 6 | Panchayat Ghar Road, (Shalimar Village) | MCD Primary School of DJB Pump House | 6 Mtrs |
| 7 | Trivilur Marg in Shakurpur JJ Colony | Maharishi Balmiki Marg to Road No -43 | 15 Mtrs |
| 8 | Road between A & C –block Shakurpur JJ Colony | Eklavya Marg to Thiruvalur Marg | 15 Mtrs |
| 9 | Road between G & H –block Shakurpur JJ Colony | Hawaldar Abdul Hameed Marg to Maharishi Balmiki Marg | 15 Mtrs |
| 10 | Eklavya Marg in Shakurpur JJ Colony | Kabir Dass Marg to Road No. 43 | 24 Mtrs |
| 11 | PWD Road No- 41 | Madhuban Chowk to Petrol Pump, Opp Netaji Subhash Place (LHS) | 45 Mtrs |
| 12 | Lane Rishi Nagar | From H.No 575 to 665 Rishi Nagar | 6 Mtrs |
| 13 | Lane Rani Bagh (Gurudwara Wali) | Main Rani Bagh Market Road to H.No.950 | 9 Mtrs |
| 14 | P.W.D. Road No. 43 Rani Bah , Rishi Nagar | Britannia Chowk to 107 C Rishi Nagar | 45 Mtrs |
| 15 | Approach Road to Singhal Pur | Shaheed Uham Singh marg to Choupal to H.No SE-1, Gyan Shakti Mandir Marg X-ing Road | 9 Mtrs |
| 16 | Around Vardaman Vatika, Tri Nagar | MC. School Deva Ram Park to Super Bazar Community Centre to Shiv Mandir | 8 Mtrs |
| 17 | Pitampura Village Shiva Market & Vats Market Road | Outer Ring Road to Janta Market chowk (Behind Mandakini Apartment. | 12 Mtrs |
| 18 | Pitampuaa Village Rama Market | Outer Ring Road to H.No. B-212 , Pitamputa Village | 15 Mtrs |
| 19 | Shalimar Haiderpur Road | Haiderpur Bus stop to Shalimar Village Crossing | 13.5 Mtrs |
| 20 | PWD Road No. 41 | Corner of Road No.44, Kapil Vihar to Nishant Kunj, hemu Marg (RHS) | 45 Mtrs |
| 21 | Gali No. 2, Main Bazar Road Shree Nagar | From Road No. 43 to Railway Road, Shree Nagar | 6 Mtrs |

Source: As per GNCTD Notification No. 149 dated 15.09.2006

Note:  In case there is difference between ROW mentioned in the list given by GNCTD

and the Zonal Plan, the ROW given in the Zonal Plan/ approved Layput plan shall

prevail.

In case of any clarification, GNCTD Notification may by referred.

Annexure – 6

Mixed Land Use Streets as per Notification No. 149 dated 15.09.2006

Mixed Land Use Streets  in Civil line Zone as / MCD

| 1 |  |  | Internal Road M- Block Shastri Nagar | M -1 to M- 19 Shastri Nagar | 9 Mtrs |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  | Internal Road M- Block Shastri Nagar | M -167 to M- 160 Shastri Nagar | 9 Mtrs |  |
| 3 |  |  | Internal Road M- Block Shastri Nagar | M -168 to M- 175 Shastri Nagar | 9 Mtrs |  |
| 4 |  |  | Internal Road M- Block Shastri Nagar | M -50 to M- 42 Shastri Nagar | 9 Mtrs |  |
| 5 |  |  | Internal Road M- Block Shastri Nagar | M -59/3 to M- 51 Shastri Nagar | 9 Mtrs |  |
| 6 |  |  | Internal Road M- Block Shastri Nagar | M -88 to M- 79 Shastri Nagar | 9 Mtrs |  |
| 7 |  |  | Internal Road M- Block Shastri Nagar | M -98 to M- 102 Shastri Nagar | 9 Mtrs |  |
| 8 |  |  | Internal Road F- Block Shastri Nagar | F-16 to F- 20 Shastri Nagar | 9 Mtrs |  |
| 9 |  |  | Internal Road F- Block Shastri Nagar | F-50 to F-42-19 Shastri Nagar | 9 Mtrs |  |
| 10 |  |  | Internal Road F- Block Shastri Nagar | F-1 to F- 8 Shastri Nagar | 9 Mtrs |  |
| 11 |  |  | PNB Road, Shastri Nagar | E-83 to E-99 , Shastri Nagar | 9 Mtrs |  |
| 12 |  |  | Internal Road Inderlok | 313/35/2 to 313/41/A Inderlok | 9 Mtrs |  |
| 13 |  |  | Internal Road B- Block Shastri Nagar | B –1088 to B-1091 Shastri Nagar | 9 Mtrs |  |
| 14 |  |  | Internal Road B- Block Shastri Nagar | B –Block Durga Mandir to B-1295 Shastri Nagar | 9 Mtrs |  |
| 15 |  |  | Internal Road A- Block Shastri Nagar | A -100 to A- 85 Shastri Nagar | 9 Mtrs |  |
| 16 |  |  | Internal Road A- Block Shastri Nagar | A -84 to A- 149 Shastri Nagar | 9 Mtrs |  |
| 17 |  |  | Internal Road A- Block Shastri Nagar | A -149 to A- 411 Shastri Nagar | 9 Mtrs |  |
| 18 |  |  | Internal Road A- Block Shastri Nagar | A -148 to A- 133 Shastri Nagar | 9 Mtrs |  |
| 19 |  | Internal Road A- Block Shastri Nagar |  | A -186 to A- 182 Shastri Nagar |  | 9 Mtrs |
| 20 |  | Internal Road A- Block Shastri Nagar |  | A -172 to A- 176 Shastri Nagar |  | 9 Mtrs |
| 21 |  | Internal Road B- Block Shastri Nagar |  | B -1631 to B- 1628 Shastri Nagar |  | 9 Mtrs |
| 22 |  | Internal Road B- Block Shastri Nagar |  | B -1643 to B- 1647 Shastri Nagar |  | 9 Mtrs |
| 23 |  | Internal Road B- Block Shastri Nagar |  | B -1780 to B- 1765 Shastri Nagar |  | 9 Mtrs |
| 24 | Internal Road A- Block Shastri Nagar |  |  | A -170 to A- 167 Shastri Nagar |  | 8.5 Mtrs |
| 25 | Internal Road B- Block Shastri Nagar |  |  | B -1594 to B- 1601 Shastri Nagar |  | 8.5 Mtrs |
| 26 | Internal Road M- Block Shastri Nagar |  |  | M -128 to M- 135 Shastri Nagar |  | 8 Mtrs |
| 27 | Internal Road M- Block Shastri Nagar |  |  | M -135 to M- 143 Shastri Nagar |  | 8 Mtrs |
| 28 | Internal Road M- Block Shastri Nagar |  |  | M -209 to M- 197 Shastri Nagar |  | 8 Mtrs |
| 29 | Internal Road L- Block Shastri Nagar |  |  | L -1 to L-209 Shastri Nagar |  | 8 Mtrs |
| 30 | Internal Road B- Block Shastri Nagar |  |  | B -1474 to B- 1481/1 Shastri Nagar |  | 8 Mtrs |
| 31 | Internal Road A- Block Shastri Nagar |  |  | A -413 to A- 425 Shastri Nagar |  | 8Mtrs |
| 32 | Internal Road M- Block Shastri Nagar |  |  | M -184 to M- 191 Shastri Nagar |  | 7.5 Mtrs |
| 33 | Internal Road M- Block Shastri Nagar |  |  | M -159 to M- 152 Shastri Nagar |  | 7.5 Mtrs |
| 34 | Internal Road M- Block Shastri Nagar |  |  | M -144 to M- 151 Shastri Nagar |  | 7.5 Mtrs |
| 35 | Internal Road M- Block Shastri Nagar |  |  | M -28 to M- 34 Shastri Nagar |  | 7.5 Mtrs |
| 36 | Internal Road Inderlok |  |  | 313/49 to 313/37 Inderlok |  | 7.5 Mtrs |
| 37 | Internal Road Inderlok |  |  | 313/104 to 313/102 Inderlok |  | 7.5 Mtrs |
| 38 | Internal Road Lalita Block Shastri Nagar |  |  | D-75 to D-32, Lalita Block Shastri Nagar |  | 7.5 Mtrs |
| 39 | Internal Road L- Block Shastri Nagar |  |  | L -42 to L- 52 Shastri Nagar |  | 7.5 Mtrs |
| 40 | Internal Road L- Block Shastri Nagar |  |  | L -177 to L-168 Shastri Nagar. F / Side |  | 7.5 Mtrs |
| 41 | Internal Road L- Block Shastri Nagar |  |  | L -157 to L- 147 Shastri Nagar |  | 7.5 Mtrs |
| 42 | Internal Road L- Block Shastri Nagar |  |  | L -116 to L- 106 Shastri Nagar |  | 7.5 Mtrs |
| 43 | Internal Road L- Block Shastri Nagar |  |  | L -32 to L- 22 Shastri Nagar |  | 7.5 Mtrs |
| 44 | Internal Road L- Block Shastri Nagar |  |  | L -266 to L- 270 Shastri Nagar |  | 7.5 Mtrs |
| 45 | Internal Road A- Block Shastri Nagar |  |  | A -60 to A- 69 Shastri Nagar |  | 7.5 Mtrs |
| 46 | Internal Road B- Block Shastri Nagar |  |  | B -1896 to B- 1903 Shastri Nagar |  | 7.5 Mtrs |
| 47 | Internal Road B- Block Shastri Nagar |  |  | B -1881 to B- 1886 Shastri Nagar |  | 7.5 Mtrs |
| 48 | Internal Road B- Block Shastri Nagar |  |  | B -1815 to B- 1829 Shastri Nagar |  | 7.5 Mtrs |
| 49 | Internal Road B- Block Shastri Nagar |  |  | B -1742 to B- 1737 Shastri Nagar |  | 7.5 Mtrs |
| 50 | Internal Road FK- Block Shastri Nagar |  |  | FK -40 to FK- 35 Shastri Nagar |  | 7 Mtrs |
| 51 | Internal Road L- Block Shastri Nagar |  |  | L -260 to L- 230 Shastri Nagar |  | 7 Mtrs |
| 52 | Internal Road L- Block Shastri Nagar |  |  | L -269 to L- 263 Shastri Nagar |  | 7 Mtrs |
| 53 | Internal Road B- Block Shastri Nagar |  |  | B -1041 to B- 1058 Shastri Nagar |  | 7 Mtrs |
| 54 | Internal Road B- Block Shastri Nagar |  |  | B -1449 to B- 1459 Shastri Nagar |  | 7 Mtrs |
| 55 | Internal Road B- Block Shastri Nagar |  |  | B -1449 to B- 1504 Shastri Nagar |  | 7 Mtrs |
| 56 | Internal Road M- Block Shastri Nagar |  |  | M -176 to M- 184 Shastri Nagar |  | 6.5 Mtrs |
| 57 | Internal Road M- Block Shastri Nagar |  |  | M -127 to M- 120 Shastri Nagar |  | 6.5 Mtrs |
| 58 | Internal Road M- Block Shastri Nagar |  |  | M -112 to M- 119 Shastri Nagar |  | 6.5 Mtrs |
| 59 | Internal Road FK- Block Shastri Nagar |  |  | FK -50 to FK- 45 Shastri Nagar |  | 6.5 Mtrs |
| 60 | Internal Road L-2 Block Shastri Nagar |  |  | L -2/29 to L- 2/21 Shastri Nagar |  | 6.5 Mtrs |
| 61 | Internal Road L-2 Block Shastri Nagar |  |  | L -2/60 to L- 2/72 Shastri Nagar |  | 6.5 Mtrs |
| 62 | Internal Road Lalita Block Shastri Nagar |  |  | D-2 to D-10, Lalita Block Shastri Nagar |  | 6.5 Mtrs |
| 63 | Internal Road L- Block Shastri Nagar |  |  | L -247 to L- 236, DDA Park Shastri Nagar |  | 6.5 Mtrs |
| 64 | Internal Road L- Block Shastri Nagar |  |  | L -236 to L- 261 Shastri Nagar |  | 6.5 Mtrs |
| 65 | Internal Road L- Block Shastri Nagar |  |  | L -260 to L- 261 Shastri Nagar |  | 6.5 Mtrs |
| 66 | Internal Road B- Block Shastri Nagar |  |  | B -1653 to B- 1657 Shastri Nagar |  | 6.5 Mtrs |
| 67 | Internal Road M- Block Shastri Nagar |  |  | M -200 to M- 192/12 Shastri Nagar |  | 6 Mtrs |
| 68 | Internal Road M- Block Shastri Nagar |  |  | M 51 to M- 19 Shastri Nagar |  | 6 Mtrs |
| 69 | Internal Road F Block Shastri Nagar |  |  | F -35 to F- 40 Shastri Nagar |  | 6 Mtrs |
| 70 | Internal Road E Block Shastri Nagar |  |  | E -177 to E- 224 Shastri Nagar |  | 6 Mtrs |
| 71 | Internal Road L-2 Block Shastri Nagar |  |  | L -2/53 to L- 2/47 Shastri Nagar |  | 6 Mtrs |
| 72 | Internal Road L-2 Block Shastri Nagar |  |  | L -2/76 to L- 2/79 Shastri Nagar |  | 6 Mtrs |
| 73 | Internal Road L-2 Block Shastri Nagar |  |  | DDA Park B/ side to L-2/82 |  | 6 Mtrs |
| 74 | Internal Road B- Block Shastri Nagar |  |  | B -983 to B- 976 Shastri Nagar |  | 6 Mtrs |
| 75 | Internal Road B- Block Shastri Nagar |  |  | B -956 to B- 965 Shastri Nagar |  | 6 Mtrs |
| 76 | Internal Road B- Block Shastri Nagar |  |  | B -1309 to B- 1316 Shastri Nagar |  | 6 Mtrs |
| 77 | Internal Road B- Block Shastri Nagar |  |  | B -1277 to B- 125 Shastri Nagar |  | 6 Mtrs |
| 78 | Internal Road B- Block Shastri Nagar |  |  | B -1372 to B- 1380 Shastri Nagar |  | 6 Mtrs |
| 79 | Internal Road B- Block Shastri Nagar |  |  | B -1406 to B- 1401 Shastri Nagar |  | 6 Mtrs |
| 80 | Internal Road B- Block Shastri Nagar |  |  | B -1498 to B- 1492 Shastri Nagar |  | 6 Mtrs |
| 81 | Internal Road B- Block Shastri Nagar |  |  | B -1400 to B- 1395 Shastri Nagar |  | 6 Mtrs |
| 82 | Internal Road B- Block Shastri Nagar |  |  | B -1555 to B- 1559 Shastri Nagar |  | 6 Mtrs |
| 83 | Internal Road B- Block Shastri Nagar |  |  | B -1871 to B- 1855 Shastri Nagar |  | 6 Mtrs |
| 84 | Internal Road B- Block Shastri Nagar |  |  | B -1602 to B- 1611 Shastri Nagar |  | 6 Mtrs |
| 85 | Shastri Nagar Metro Statation to Zakira RUB |  |  | Shastri Nagar Metro Statation to Zakira RUB |  | 60 Mtrs |

Source: As per GNCTD Notification No. 149 dated 15.09.2006

Note:  In case there is difference between ROW mentioned in the list given by GNCTD    and

the Zonal Plan, the ROW given in the Zonal Plan/ approved Layput plan shall  prevail.

In case of any clarification, GNCTD Notification may by referred.

Annexure – 7

Mixed Land Use Streets  in Rohini Zone as/ MCD

| S.No | Name of the Road/ Street | Stretch of Road | Row |
| --- | --- | --- | --- |
| 1 | Lane –Rani Bagh | From Road No. 43 to M.C.W Centre Rani Bagh | 6 Mtrs |
| 2 | Lane –Rani Bagh | From Road No. 43 to Mandir H.No. 1647 Rani Bagh | 6 Mtrs |
| 3 | Lane –Rani Bagh | Road No. 43 to Shiv Mandir At Sant Nagar Road | 6 Mtrs |
| 4 | Lane –Rani Bagh | From H No. 1083 to H.No. 1094 Rani Bagh | 6 Mtrs |
| 5 | Bharat Nagar Road | Swami Narayan Marg to Gulab Singh Marg | 18 Mtrs |
| 6 | Madhav Marg, Keshav Puram | Road from old Zonal Road to C 4 Market Keshav Puram | 18 Mtrs |
| 7 | Road between C- 4 & C-7 at Keshav Puram | From Madhav Marg to Kanhiya Nagar Drain Keshav Puram | 18 Mtrs |
| 8 | Madhav Marg, Keshav Puram | Keshav puram bus stop C- 4 Pkt to Britannia Road | 30 Mtrs |
| 9 | Main Road Sawan Park | From Mata Jai Kaur School to Railway X– ing Sawan park | 14 Mtrs |
| 10 | Main Market Road Wazirpur Village | Deep Cinema Road to Wazirpur Village (Choupal) | 6 Mtrs |
| 11 | Hawaldar Abdul Hameed Marg in Shakurpur JJ colony | Ring Road to Kabir Dass Marg | 20 Mtrs |
| 12 | Maharishi Balmiki Marg in Shakurpur JJ colony | Ring Road to Hawaldar Abdul Hameed Marg | 20 Mtrs |
| 13 | Police Chowki Road, Wazirpur JJ Colony | Wazirpur JJ Colony to Police Chowki | 14 Mtrs |
| 14 | K- Block Road, Wazirpur JJ Colony | Bharat Nagar Road to Crimination Ground,Wazirpur JJ Colony | 14 Mtrs |
| 15 | Road No. 44 (PWD Road) | Road No. 41 to Road No 43 | 30 Mtrs |
| 16 | K.G. Goyal Marg Ashok Vihar | Regard Rest To RUV Wazirpur | 30 Mtrs |
| 17 | Outer Ring Road | BU Block Petrol pump to Police line | 60 Mtrs |
| 18 | Gali No.1 Shree Nagar | From Road No. 43 to WZ -36 Shakurpur Village, Shree Nagar | 9 Mtrs |
| 19 | Raja Park Shree Nagar Railway Road | From Road No. X-ing to Shakurpur Railway X- ing at Raja Park Shree Nagar | 24 Mtrs |
| 20 | Road No. 44 Mahendra Park | From Road No. 43 Aggarwal Sweet Corner to Phauwra Chowk along Mahendra Park | 30 Mtrs |
| 21 | Phirni Road, Pitampura Village | Janta market Chowk to Shivaji Market Chowk | 18 Mtrs |
| 22 | Ch. Gulab Singh Marg | Water Tank to Satyawati College | 36 Mtrs |
| 23 | Internal Road B- Block Shastri Nagar | B -1263 to B- 1268 Shastri Nagar | 5.5Mtrs |
| 24 | Internal Road B- Block Shastri Nagar | B -1329 to B- 1505 Shastri Nagar | 5.5 Mtrs |
| 25 | Internal Road M- Block Shastri Nagar | M -71 to M- 66 Shastri Nagar | 5.5 Mtrs |
| 26 | Internal Road FK- Block Shastri Nagar | FK -34 to FK- 29 Shastri Nagar | 5 Mtrs |
| 27 | Internal Road FK- Block Shastri Nagar | FK -12 to FK- 17 Shastri Nagar | 5 Mtrs |
| 28 | Internal Road FK- Block Shastri Nagar | FK -17 to FK- 23 Shastri Nagar | 5 Mtrs |
| 29 | Internal Road E Block Shastri Nagar | E -89A to E- 89 Shastri Nagar | 5 Mtrs |
| 30 | Internal Road E-2 Block Shastri Nagar | E -2/16 to E-2/1 Shastri Nagar | 5 Mtrs |
| 31 | Internal Road L-2 Block Shastri Nagar | L -2/72 to Mandir, Shastri Nagar | 5 Mtrs |
| 32 | Internal Road B- Block Shastri Nagar | B -996 to B- 1003 Shastri Nagar | 5 Mtrs |
| 33 | Internal Road B- Block Shastri Nagar | B -1438 to B- 1439 Shastri Nagar | 5 Mtrs |
| 34 | Internal Road B- Block Shastri Nagar | B -1379 to B- 1487 Shastri Nagar | 5 Mtrs |
| 35 | Internal Road B- Block Shastri Nagar | B -1910 to B- 1871 Shastri Nagar | 5 Mtrs |
| 36 | Internal Road M- Block Shastri Nagar | M -60 to M- 35 Shastri Nagar | 4.5 Mtrs |
| 37 | Internal Road M- Block Shastri Nagar | M -41 to M- 34 Shastri Nagar | 4.5 Mtrs |
| 38 | Internal Road FK- Block Shastri Nagar | FK -7 to New FK- 30 A Shastri Nagar | 4.5 Mtrs |
| 39 | Internal Road FK- Block Shastri Nagar | FK -51 to FK- 49 Shastri Nagar | 4.5 Mtrs |
| 40 | Internal Road FK- Block Shastri Nagar | New F -31 to F- 35 Shastri Nagar | 4.5 Mtrs |
| 41 | Internal Road FK- Block Shastri Nagar | New F -17 to New F- 23 Shastri Nagar | 4.5 Mtrs |
| 42 | Internal Road FK- Block Shastri Nagar | New F -9 to New F- 4 Shastri Nagar | 4.5 Mtrs |
| 43 | Internal Road FK- Block Shastri Nagar | New F -11 to New F- 16 Shastri Nagar | 4.5 Mtrs |
| 44 | Internal Road FK- Block Shastri Nagar | New F -8 to New F- 1 Shastri Nagar | 4.5 Mtrs |
| 45 | Internal Road E Block Shastri Nagar | E -82 to E- 77 Shastri Nagar | 4.5 Mtrs |
| 46 | Internal Road E Block Shastri Nagar | E -62 to E- 52 Shastri Nagar | 4.5 Mtrs |
| 47 | Internal Road E Block Shastri Nagar | E -120 to E- 136 Shastri Nagar | 4.5 Mtrs |
| 48 | Internal Road E Block Shastri Nagar | E -119 to E- 110 Shastri Nagar | 4.5 Mtrs |
| 49 | Internal Road E Block Shastri Nagar | E -169 to E- 199 Shastri Nagar | 4.5 Mtrs |
| 50 | Internal Road E Block Shastri Nagar | E -170 to E- 181 Shastri Nagar | 4.5 Mtrs |
| 51 | Internal Road E Block Shastri Nagar | E -222 to E- 216 Shastri Nagar | 4.5 Mtrs |
| 52 | Internal Road E Block Shastri Nagar | E -266 to E- 255 Shastri Nagar | 4.5 Mtrs |
| 53 | Internal Road L-2 Block Shastri Nagar | L -2/38 to L-2/30, Shastri Nagar | 4.5 Mtrs |
| 54 | Internal Road L-2 Block Shastri Nagar | L -2/87 to L-2/90, Shastri Nagar | 4.5 Mtrs |
| 55 | Internal Road L-2 Block Shastri Nagar | L -2/6 A to L-2/12 | 4.5 Mtrs |
| 56 | Internal Road Lalita Block Shastri Nagar | D-73 to D-32, Lalita Block Shastri Nagar | 4.5 Mtrs |
| 57 | Internal Road Lalita Block Shastri Nagar | D-2734 to D-75, Lalita Block Shastri Nagar | 4.5 Mtrs |
| 58 | Internal Road Nimri Village, Shastri Nagar | WZ-149 to Nimri Dharm Shala | 4.5 Mtrs |
| 59 | Internal Road Nimri Village, Shastri Nagar | WZ Nimri Dharm Shala to WZ- 1 | 4.5 Mtrs |
| 60 | Internal Road L- Block Shastri Nagar | L -1 to L- 10 Shastri Nagar | 4.5 Mtrs |
| 61 | Internal Road L- Block Shastri Nagar | L -11 to L- 20 Shastri Nagar | 4.5 Mtrs |
| 62 | Internal Road L- Block Shastri Nagar | L -43/1 to L- 52 Shastri Nagar | 4.5 Mtrs |
| 63 | Internal Road L- Block Shastri Nagar | L -117 to L- 125 Shastri Nagar | 4.5 Mtrs |
| 64 | Internal Road L- Block Shastri Nagar | L -135 to L- 126 Shastri Nagar | 4.5 Mtrs |
| 65 | Internal Road L- Block Shastri Nagar | L -177 to L- 168 Shastri Nagar B/side | 4.5 Mtrs |
| 66 | Internal Road L- Block Shastri Nagar | L -178 to L- 188 Shastri Nagar | 4.5 Mtrs |
| 67 | Internal Road L- Block Shastri Nagar | L -136 to L- 140 Shastri Nagar | 4.5 Mtrs |
| 68 | Internal Road L- Block Shastri Nagar | L -95 to L- 105 Shastri Nagar | 4.5 Mtrs |
| 69 | Internal Road L- Block Shastri Nagar | L -53 to L- 63 Shastri Nagar | 4.5 Mtrs |
| 70 | Internal Road L- Block Shastri Nagar | L -235 to L- 227 Shastri Nagar | 4.5 Mtrs |
| 71 | Internal Road L- Block Shastri Nagar | L -216 to Shastri Nagar | 4.5 Mtrs |
| 72 | Internal Road L- Block Shastri Nagar | L -260 to L- 251 Shastri Nagar | 4.5 Mtrs |
| 73 | Internal Road B- Block Shastri Nagar | B -1020 to B- 1004 Shastri Nagar | 4.5 Mtrs |
| 74 | Internal Road B- Block Shastri Nagar | B -975 to B- 966 Shastri Nagar | 4.5 Mtrs |
| 75 | Internal Road B- Block Shastri Nagar | B -954 to B- 966 Shastri Nagar | 4.5 Mtrs |
| 76 | Internal Road B- Block Shastri Nagar | B -1155 to B- 1139 Shastri Nagar | 4.5 Mtrs |
| 77 | Internal Road B- Block Shastri Nagar | B -1138 to B- 1124 Shastri Nagar | 4.5 Mtrs |
| 78 | Internal Road B- Block Shastri Nagar | B -1107 to B- 1123 Shastri Nagar | 4.5 Mtrs |
| 79 | Internal Road B- Block Shastri Nagar | B -1072 to B- 1059/1 Shastri Nagar | 4.5 Mtrs |
| 80 | Internal Road B- Block Shastri Nagar | B -1020 to B- 923 Shastri Nagar | 4.5 Mtrs |
| 81 | Internal Road B- Block Shastri Nagar | B -1450 to B- 1459 Shastri Nagar | 4.5 Mtrs |
| 82 | Internal Road B- Block Shastri Nagar | B -1198 to E block Durga Mandir, Shastri Nagar | 4.5 Mtrs |
| 83 | Internal Road B- Block Shastri Nagar | B -1295 to B- 1239 Shastri Nagar | 4.5 Mtrs |
| 84 | Internal Road B- Block Shastri Nagar | B -1279 to B- 1295 Shastri Nagar | 4.5 Mtrs |
| 85 | Internal Road B- Block Shastri Nagar | B -1297 to B- 1209 Shastri Nagar | 4.5 Mtrs |
| 86 | Internal Road B- Block Shastri Nagar | B -1520 to B- 1513 Shastri Nagar | 4.5 Mtrs |
| 87 | Internal Road B- Block Shastri Nagar | B -1554 to B- 1546 Shastri Nagar | 4.5 Mtrs |
| 88 | Internal Road B- Block Shastri Nagar | B -1545 to B- 1537/1 Shastri Nagar | 4.5 Mtrs |
| 89 | Internal Road B- Block Shastri Nagar | B -1529 to B- 1536 Shastri Nagar | 4.5 Mtrs |
| 90 | Internal Road A- Block Shastri Nagar | A -71 to A- 84 Shastri Nagar | 4.5 Mtrs |
| 91 | Internal Road A- Block Shastri Nagar | A -1152 to A- 129 Shastri Nagar | 4.5 Mtrs |
| 92 | Internal Road A- Block Shastri Nagar | A -187 to A- 196 Shastri Nagar | 4.5 Mtrs |
| 93 | Internal Road A- Block Shastri Nagar | A -181 to A- 177 Shastri Nagar | 4.5 Mtrs |
| 94 | Internal Road B- Block Shastri Nagar | B -1505 to B- 1913 Shastri Nagar | 4.5 Mtrs |
| 95 | Internal Road B- Block Shastri Nagar | B -1584 to B- 1591 Shastri Nagar | 4.5 Mtrs |
| 96 | Internal Road B- Block Shastri Nagar | B -1611 to B- 1607 Shastri Nagar | 4.5 Mtrs |
| 97 | Internal Road B- Block Shastri Nagar | B -1637 to B- 1630 Shastri Nagar | 4.5 Mtrs |
| 98 | Internal Road B- Block Shastri Nagar | B -1652 to B- 1648 Shastri Nagar | 4.5 Mtrs |
| 99 | Internal Road B- Block Shastri Nagar | B -1796 to B- 1781 Shastri Nagar | 4.5 Mtrs |
| 100 | Internal Road B- Block Shastri Nagar | B -1782 to B- 1781/10/2 Shastri Nagar | 4.5 Mtrs |
| 101 | Internal Road B- Block Shastri Nagar | B -1781/12 to B- 1781/13 Shastri Nagar | 4.5 Mtrs |
| 102 | Internal Road B- Block Shastri Nagar | B -1757/2 to B- 1764/2 Shastri Nagar | 4.5 Mtrs |
| 103 | Internal Road B- Block Shastri Nagar | B -1757/7 to B- 1757/10 Shastri Nagar | 4.5 Mtrs |
| 104 | Internal Road B- Block Shastri Nagar | B -1757/1 8 to B-1757/26 Shastri Nagar | 4.5 Mtrs |
| 105 | Internal Road F- Block Shastri Nagar | F-50 to F-16, Shastri Nagar | 4.5 Mtrs |
| 106 | Internal Road F- Block Shastri Nagar | F-8 to F-14, Shastri Nagar | 4.5 Mtrs |

Source: As per GNCTD Notification No. 149 dated 15.09.2006

Note:   In case there is difference between ROW mentioned in the list given by GNCTD and the Zonal plan, the ROW given in the Zonal Plan/approved Layout Plan shall prevail.

In case of any clarification, GNCTD Notification may be referred.

Annexure – 8

Commercial Streets as per GNCTD Notification No. F13/146/2006/UD/5754 dated 7.2.2007

Commercial Streets in Civil line Zone as/ MCD

| S.No | Name of the Road/ Street | Stretch of Road |  | ROW (in mtrs) |
| --- | --- | --- | --- | --- |
|  |  | From | To |  |
| 1 | A- block Shukar Bazar Road, (from main Road to Mahendra Park one side) | A -1820 | A-180 | 9 Mtrs |
| 2. | Internal Road Sindhora Kalan | S.K 44 | SK 54 A (front lane one side only) | 9 Mtrs |
| 3. | Main Road of Sindhora Kalan | Park | Dharmshala | 9 Mtrs |
| 4 | Internal Road, Sri Nagar | 51/8 (on Kalidas Marg) | Gurudwara | 12 Mtrs |
| 5 | Internal Road, Sri Nagar | 51/8 | 55- A near park one side | 12 Mtrs |
| 6 | Internal Road A-block , Shastri Nagar | A-509 | Nimri Dharanshala | 9 Mtrs |
| 7 | Internal Road A-block , Shastri Nagar | A-209 | A-218 | 10 Mtrs |
| 8 | Internal Road B-block , Shastri Nagar | B-1245 | B-1318 & L -53 | 8.5 Mtrs |
| 9 | Internal Road E-block , Shastri Nagar | E -52 | E- 65 | 9 Mtrs |

Source: As per Notification No. F13/146/2006/UD/5754 dt. 05.02.2007

Annexure – 9

Commercial Streets in Rohini Zone as / MCD

| S.No | Name of the Road/ Street | Stretch of Road | Row |
| --- | --- | --- | --- |
| 1 | Choudhary Kanhiya Lal Khari Marg | Ramapura Village Railway Xing Road to MIG Flat, Ramapura | 15 Mtrs |
| 2 | Fountain Park Chowk Market Road | Road No. 44 Xing to Rani Bagh Main Market Xing. | 6 Mtrs |
| 3 | Shivaji Market, Road Pitampura Village | Rama Market Road to H.No. 334/8 near Honda Service Station | 6 Mtrs |

Source: GNCTD Notification No. F13/146/2006/UD/5754 dt. 07.02.2007

Annexure – 10

Mixed Land Use Streets as per Notification No. F13/146/2006/UD/5754 dt. 07.02.2007

Mixed Land Use Streets in Civil Line Zone as/ MCD

| S.No | Name of the Road/ Street | Stretch of Road |  | Row |
| --- | --- | --- | --- | --- |
| 1 | Internal Road, Sindhora Kalan | SK-38 | SK -35 | 9 Mtrs |
| 2 | Internal Road, Sri Nagar | 8 A | 14 A (Near Park) | 9.14 Mtrs |
| 3 | Internal Road, Sri Nagar | H.No. 36 | H.No.32 | 9.14 Mtrs |
| 4 | Internal Road, Sri Nagar | H.No. 62 | H.No.55 (one side) | 9.14 Mtrs |
| 5 | Internal Road A-block , Shastri Nagar | A-795 | A-800 | 7.5 Mtrs |
| 6 | Internal Road A-block , Shastri Nagar | A-791 | A-827 | 7.8 Mtrs |
| 7 | Internal Road A-block , Shastri Nagar | A-657 | A-770 | 8.5 Mtrs |
| 8 | Internal Road A-block , Shastri Nagar | A-610 | A-622 | 7.5 Mtrs |
| 9 | Internal Road A-block , Shastri Nagar (Front Lane) | A-470 | A-485 | 9 Mtrs |
| 10 | Internal Road A-block , Shastri Nagar | A-260 | A-243 | 9.5 Mtrs |
| 11 | Internal Road A-block , Shastri Nagar | A-55 | A-59 | 7.5 Mtrs |
| 12 | Internal Road A-block , Shastri Nagar | A-1 | A-10 (RHS) | 11 Mtrs |
| 13 | Internal Road A-block , Shastri Nagar | A-196 | A-187 | 9.5 Mtrs |
| 14 | Internal Road M-block , Shastri Nagar | M-72 | M-78 (front side) | 6.5 Mtrs |
| 15 | Internal Road M-block , Shastri Nagar | M-97 | M-92(front side) | 8.5 Mtrs |
| 16 | Internal Road L-block , Shastri Nagar | L - 93 | L -85 | 8.5 Mtrs |
| 17 | Internal Road F-block , Shastri Nagar | F -18 | F-34 | 9 Mtrs |
| 18 | Internal Road F-block , Shastri Nagar | F -51 | F-65 | 9 Mtrs |
| 19 | Internal Road FK-block , Shastri Nagar | FK -52 | FK- 40 | 6.5 Mtrs |
| 20 | Internal Road E-block , Shastri Nagar | E-223 | E -244 | 9 Mtrs |
| 21 | Internal Road E-block , Shastri Nagar | E-162 | E -137 | 9 Mtrs |
| 22 | Internal Road E-block , Shastri Nagar | E-99 | E -153 | 9 Mtrs |

Source: GNCTD Notification No. F13/146/2006/UD/5754 dt. 07.02.2007

Annexure – 11

Mixed Land Use Streets in Rohini Zone as / MCD

| S.No |  | Name of the Road/ Street | Stretch of Road | Row |
| --- | --- | --- | --- | --- |
| 1 | Bhagwan Mahavir Marg, Shalimar Bagh |  | Block Ab-1 to Ab -27, AA-1 to AA-23 | 24 Mtrs |
| 2 | Maharishi Dayanand Marg , Shalimar Bagh |  | Ring Road AL-9 to AL-137 and AG-44 to 46A | 24 Mtrs |
| 3 | Internal Road of Sahipur Village |  | H. No. 16 to 29 to 60 to 64 to 17 | 6 Mtrs |
| 4 | Ring Road Mahatma Gandhi Marg along with AG Block to AN Block |  | H.No. AG -4 to AG-35 | 60 Mtrs |

Source: GNCTD Notification No. F13/146/2006/UD/5754 dt. 07.02.2007

Note:  In case there is difference between ROW mentioned in the list given by GNCTD and the Zonal Plan, the ROW given in the Zonal Plan/ approved Layput plan shall prevail.

In case of any clarification, GNCTD Notification may by referred.

Annexure -12

Pedestrain Mixed Land use street in Zone-‘H’ as per Notification No. F13/146/2006/UD/5754 dt. 07.02.2007

Pedestrain Shopping Streets in Civil Line Zone as / MCD

| S.No | Name of the Road/ Street | Stretch of Road |  | ROW (in mtrs) |
| --- | --- | --- | --- | --- |
|  |  | From | To |  |
| 1 | Internal Road A-block , Shastri Nagar | A-531 | A-519 | 4.5 Mtrs |
| 2 | Internal Road A-block , Shastri Nagar (back Lane) | A-470 | A-485 | 4.5 Mtrs |
| 3 | Internal Road A-block , Shastri Nagar | A-291 | A-307 | 4.5 Mtrs |
| 4 | Internal Road A-block , Shastri Nagar | A-219 | A-195 | 4.5 Mtrs |
| 5 | Internal Road A-block , Shastri Nagar | A-470 | A-485 | 4.5 Mtrs |
| 6 | Internal Road M-block , Shastri Nagar | M-19 | M- 27 | 4.5 Mtrs |
| 7 | Internal Road M-block , Shastri Nagar | M-97 | M-92 (back side) | 4.5 Mtrs |
| 8 | Internal Road E-2 block , Shastri Nagar | E-2/28 | E-2/1 | 5.5 Mtrs |
| 9 | Internal Road E-2 block , Shastri Nagar | E-2/16 | E-2/50 | 4.5 Mtrs |

Source: GNCTD Notification No. F13/146/2006/UD/5754 dt. 07.02.2007

### ROHINI SCHEME ( SUB ZONE- H-7,8,9 )

##### Annexure-13

IDENTIFICATION OF MIXED- USE STREETS IN ‘ H’ ZONE (ROHINI)

| Sl.No. | Name of Road/ Street | Stretch of Road from- to | R/W |
| --- | --- | --- | --- |
| 1 | Police Station Road, Mangol Puri | From J Block crossing to G Block crossing | 24.0 |
| 2 | L.P. Road, Mangolpuri | A-Block Abkhara to S Block crossing | 18.0 |
| 3 | Katran Road, Mangolpuri | Q-Block crossing to ST Block crossing | 18.0 |
| 4 | F Block, Mangolpuri | Masid to Outer Ring Road | 9.0 |
| 5 | Sector-7,Internal Road | From D-11/60 to D11/405 | 9.0 |
| 6 | Sector-9,Rohini Internal Road | From Bharat Chowk to PWD road | 9.0 |
| 7 | Sector-16,Rohini Internal Road | From G-6/225 to G-5/330 | 13.5 |
| 8 | Sector-18,Rohini Internal Road | From B-1/1 to B-11/1 | 13.5 |
| 9 | Sector-5,Rohini Internal Road | From B-2 /104 to B-9/118 | 18.0 |
| 10 | Sector-5,Rohini Internal Road | From C-6/141 to Telephone Exchange | 18.0 |
| 11 | Internal Road Sector-1 | From D-82 to D-559 | 9.0 |
| 12 | Internal Road Sector-1 | From C-253 to D-336 | 9.0 |
| 13. | Internal Road Sector-1 | From D-387 to D-784 | 9.0 |
| 14. | Internal Raod-Sector-1 | From B-286 to B-384 | 9.0 |
| 15 | Internal Road Sector-1 | From B-1122 to B-1039 | 9.0 |
| 16 | Internal Road Sector-3 | From B5/60 to B-9,,DDA park | 13.5 |
| 17 | Internal Road Sector-3 | From C Shopping to Open land | 13.5 |
| 18. | Internal Road Sector-3 | From D-17/159, C Shopping Centre to Park Corner | 13.5 |
| 19. | Internal Road Sector-4 | From A-4/226 to A-4/291 | 13.5 |
| 20 | Internal Road Sector-3 | From E-20/30 to D-14/226 | 18.0 |
| 21 | Jaipur Golden Hospital Road | From Pkt.A-1 Sector-3 to Pkt.C-12(MIG flat Sec.3) | 24.0 |
| 22 | Sector-5,Rohini Internal Road | From B-5/91 to B-5/143 | 9.0 |
| 23 | Sector-5, Rohini Internal Road | From B-7/215 to B-9/60 | 9.0 |
| 24 | Sector-7, Rohini Internal Road | From B-6/159 to DAV School | 9.0 |
| 25 | Sector-7,Rohini internal road | From D-14/36 to D-14/196 | 9.0 |
| 26 | Sector-7 Rohini internal road | From D-12/12 to PWD | 9.0 |
| 27 | Sector-7, Rohini Internal Road | From D-12/178 to Geeta Ratan Jindal School | 9.0 |
| 28. | Sector-7, Rohini Internal Road | From D-14/12 to D-12/150 | 9.0 |
| 29 | Sector-7 Rohini Internal Road | From B-5/90 to B-4/91 | 9.0 |
| 30 | Sector-16,Rohini Internal Road | From G-4/196 to G-6/121 | 13.5 |
| 31 | Sector-5, Rohini Internal Road | From A-3/11 to B-5/405 | 18.0 |
| 32 | Sector-6, Rohini Internal Road | From A-1/650 to D-6/19 | 18.0 |
| 33 | Sector-6, Rohini Internal Road | From C-5/253 to C-4/253 | 18.0 |
| 34 | Sector-8, Rohini Internal Road | From A-3, Sector-8 to Sector-7 & 8 (Dividing Road) | 18.0 |
| 35 | Sector-8, Rohini Internal Road | From A-2/104 to PWD | 18.0 |
| 36 | Sector-11, Rohini Internal Road | From A-1/137 to A-3/92 | 18.0 |
| 37 | Sector-11,Rohini Internal Road | From A-3/53 to C-4/37 | 18.0 |
| 38 | Sector-11,Rohini Internal Road | From B-8/12 to B-2/1 | 18.0 |
| 39 | Sector-11,Rohini Internal Road | From H-2/35 to CSC-5 | 18.0 |
| 40 | Internal Road, Sector-2 | From Pkt.6-11, H.No.355 to Pkt.6-11,730 | 13.5 |
| 41 | Internal Road, Sector-2 | From Shop No.1 Pkt.7 Road No.3 | 13.5 |
| 42 | Internal Road, Sector-3 | From G-26/405 to G-30/292 | 13.5 |
| 43 | Internal Road, Sector-2 | From C,Shopping Centre to Mangolpur,Kalan Phirni | 18.0 |
| 44 | Internal Road, Sector-3 | From E-21/234 to D-15/30 | 18.0 |
| 45 | Internal Road, Sector-3 | From E-18/253 to E-19/222 | 18.0 |
| 46 | Internal Road Sector-3 | From Community Centre to Kandirya Vidhyala Corner | 18.0 |
| 47 | Prashant Vihar | From D Block Market Central to Maheshwari Apartment | 18.0 |
| 48 | Mangol Puri(Internal Road) | A-Block, Mangolpuri dispensary to D-926(both sides) | 9.0 |
| 49 | Mangolpuri(Internal Road | i)F-Block Mother Diary Booth to Sr.Sec.School(one side) ii)Govt.Sec.School to G-Block Mandir(one side) | 13.0 13.0 |
| 50 | Mangol Puri (Internal Road) | F-Block,Mangolpuri, Madrasi Mandir to F204(both side) | 9.0 |
| 51 | Mangol Puri (Internal Road) | G-Block,Hr.Sec.School to G-900(both sides) | 9.0 |
| 52 | Mangol Puri (Internal Road) | B-151 to Nirankari Bhawan, Mangolpuri(both side) | 9.0 |
| 53 | Mangol Puri (Internal Road) | C-151 to C-Block, Primary School, Mangolpuri(both sides) | 9.0 |
| 54 | Mangol Puri (Internal Road) | J-1105 to J-Block Market Mangolpuri((both side) | 9.0 |
| 55 | Mangolpuri (Internal Road) | J-24 to Y Block, Jhuggi Mangolpuri(both sides) | 9.0 |
| 56 | Mangol Puri (Internal Road) | R-24 to R-578 Mangolpuri,(LHS) | 9.0 |
| 57 | Q Block Road Mangolpuri | Q-11/96 to P-1/96 Mangolpuri(one side) | 9.0 |
| 58 | P Block Road,Mangolpuri | Q-11/1 to P-1/1 Mangolpuri(LHS) | 18.0 |
| 59. | U-T Road Mangolpuri | T-504 to 1392 Mangolpuri(Both sides) | 9.0 |
| 60 | Mangolpuri internal road | 0-24 to N-24, Mangolpuri (LHS) | 18.0 |
| 61. | L-Block road, | L-1229 to L-1, Mangolpuri (LHS) | 13.0 |
| 62. | Sai Baba Mandir Marg. | H.No.292 Pkt.D-11, Sector-7, to H.No.405, D-16, Sector-7. | 13.5 |
| 63. | Internal road Sector-8. | H.No.91, Pkt.B-6 to H.No.60, Pkt.B-4. | 13.5 |
| 64. | Manav Chowk towards Sector-15. | Manav Chowk to H.No.27, Pkt.C-10,Sector-15. | 13.5 |
| 65. | Internal road Sector-15. | H.No.21 Pkt.F-15/15 to H.No.62, Pkt.F-21/15. | 13.5. |
| 66. | Internal road of Sector-5. | H.No.60, Pkt.B-5 to H.No.290,Pkt.B-9. | 13.5 |
| 67. | Internal road Sector-11. | H.No.69, Pkt.G-2 to H.No.31 Pkt.G-8. | 18.0 |
| 68. | Internal road of Sector-16. | H.No.I, Pkt.E-1 to H.No.1, Pkt.F-3. | 18.0 |
| 69. | Main Shalimar Haiderpur Road | From Haiderpur Bus Stand to CC Colony | 13.5 |
| 70. | Phirm Road of Badli Road | From Tikona park to Bus Stand near Railway Crossing Badli Village. | 9.00 |

Source: As per Notification No. S.O.1456(E) dated 7.09.2006

Note:  In case there is difference between ROW mentioned in the list given by GNCTD    and the Zonal Plan, the ROW given in the Zonal Plan/ approved Layput plan shall  prevail.

In case of any clarification, GNCTD Notification may by referred.

Annexure-14

IDENTIFICATION OF COMMERCIAL STREETS IN ‘H’ ZONE (ROHINI)

| S.No. | Name of Road/Street | Stretch of Road From – To | R/W(M) |
| --- | --- | --- | --- |
| 1 | S.T. Block, Mangolpuri | From PWD road to S Block crossing | 24 .00 |
| 2 | A-Block Market road,Mangolpuri | A-Block M.C. Primary School to Hanuman Mandir | 13.5 |
| 3 | I.G. Block Road, Mangolpuri | Indira Gandhi Park to Y Block Barat Ghar | 9.0 |
| 4 | H.Y. Block Road, Mangolpuri | Police Station Road to Y Block Barat Ghar | 9.0 |
| 5 | N Block Road, Mangolpuri | N Block Barat Ghar to O Block Dispensary | 9.0 |
| 6 | DE road, Mangolpuri | PS road to E Block Laboratory Block | 9.0 |
| 7 | PQ Block Road | Q-11/49 to P.1/49 Mangolpuri | 9.0 |
| 8 | R.S. Block Road | R-1369 to R-624, Mangolpuri | 9.0 |
| 9 | K.L. Block Road | K-1021 to K-24, Mangolpuri | 9.0 |
| 10 | PWD Road diving of sector-7 &8 | Sai Baba Chowk to Phawara Chowk(opposite M2K Cinema,Sector-3 | 30.0 |
| 11 | PWD Road dividing of Sector-6 &5 | Rajiv Gandhi Cancer Hospital to Vishram Chowk | 30.0 |
| 12 | PWD road No.41 | Madhuban Chowk to Rithala Metro Station | 45.0 |
| 13 | Sector-7,Rohini Internal Road | From D-12/61 to PWD | 9.0 |
| 14 | Sector-16,Rohini Internal Road | From H-3/172 to H1/1 | 13.5 |
| 15 | Sector-6,Rohini Internal Road | From D-8/60 to D-6/399 | 18.0 |
| 16 | Sector-7,Rohini Internal Road | From B-4 to DDA market | 18.0 |
| 17 | Sector-16,Rohini Internal Road | From E-4/1 to F-7 to DDA Market | 18.0 |
| 18 | Sector-6 & 7 Diving Road | From Ayodhaya Chowk to Ambedkar Hospital | 24.0 |
| 19 | C & D Block,Sector-15,Rohini | Manav Chowk to Sector-15 & 16 dividing road | 13.5 |
| 20 | Deepali Chowk to Vijay Vihar | DDA Office Sector-3 to Vijay Vihar | 30.0 |
| 21 | Dividing Road of Sector-2 & 3(PWD Road) | Outer Ring Road to Avantika Chowk | 30.0 |
| 22 | Avantika Chowk to Vishram Chowk(PWD Road) | Avantika Chowk to Vishram Chowk | 30.0 |
| 23 | Sector-16, Rohini Internal Road | From G-1/73 to G.7/260 | 13.5 |
| 24 | Sector-16,Rohini Internal Road | From F-2/1 to F-6/1 | 18.0 |
| 25 | Sector-16, Rohini Internal Road | From A-2/124 to corner of school | 18.0 |
| 26 | Sector-18, Rohini Internal Road | From B-8/64 to B-2/1 | 13.5 |
| 27 | Main Avantika Market Road | Avantika Chowk to Guru Golwalkar Marg | 24.0 |
| 28 | Guru Golwalkar Marg(Road No.3) | Outer Ring Road to Nangloi Drain | 30.0 |
| 29 | Parshant Vihar Internal Road | From A-1/19 Lancer Convent to Milansar Apartment Sports Complex | 24.0 |
| 30 | Kabir Das Marg | Eklavya Marg to Kk Block Power Grid | 4.0 |
| 31. | Sector-16 Rohini, Internal Road | From G5/225 to G5/125 | 13.5 |

Source: As per Notification No. S.O.1456(E) dated 7.09.2006

Note:  In case there is difference between ROW mentioned in the list given by GNCTD    and the Zonal Plan, the ROW given in the Zonal Plan/ approved Layput plan shall  prevail.

In case of any clarification, GNCTD Notification may by referred

Annexure -15

WATER BODIES IN ZONE-H (1 TO 6)

Kanjhawala Block

| S.No | Name of the Village / Census Town (CT) | Land ownership | Surface Area (Sq. Mtrs) |
| --- | --- | --- | --- |
| 1 | Haider Pur | BDO | 2,000 |
| 2 | Pitam Pura | DDA | 5,00 |

(Alipur Block) Village Ponds

| 1 | Pitam Pura | DDA | 5,00 |
| --- | --- | --- | --- |

As per MPD -2021, water bodies which are more than 1 hac. Need to be conserved

Source: MCD

Annexure –16

Water Bodies in Rohini –(H-7 to H-9)

| S.No | Name of the Village / Census Town (CT) | Land ownership | Surface Area (Sq. Mtrs) |
| --- | --- | --- | --- |
| 1 | Mangolpur Kalan | BDO | 15,000 |
| 2 | Mangolpur Khurd | BDO | 3,500 |
| 3 | Distt. Park, Rohini | DDA | 10,000 |
| 4 | Rithala | BDO | 10,000 |

As per MPD -2021, water bodies which are more than 1 hac. Need to be conserved

Source: MCD

Annexure -17

Centrally Protected Monument under Delhi Circle, Archaeological Survey of India

| S.NO. | Monument and Locality | Name of the Monument as appeared in Gazette Notification | Gazette Notification No. & Date |
| --- | --- | --- | --- |
| 1 | Sheeh Mahal Haider Pur Village | Shalimar Garden | S.O. 445 dt. 10.01.1983 S.O. 4774 dt. 06.12.1983 |

Annexure -18

Tentative list of Unauthorised Colony in Zone ‘H’in the process of regularsation as per polices

A. Zone (H-1 to H-6)

| S.No | Name of Unauthorized Colony | Sl.No given by GMCTD | Regn.No.Given by GMCTD |
| --- | --- | --- | --- |
| 1 | Ambedkar Nager (Extn) Haiderpur,Delhi-110088 | 78 | 575 |
| 2 | Ambedkar Nager(Extn Haiderpur,Delhi-110088 (Extn) | 944 | 1391 |
| 3 | Rani Garden (Ambedkar park) Shastri Nager,Delhi-110034 | 1080 | 835 |
| 4 | Wazipur Extn.(Village) Ashok | 1134 | 346 |

.

B.Rohini  (H-7, 8 and 9)

| S.No | Name of Unauthorized Colony | Sl.No given by GMCTD | Regn.No.Given by GMCTD |
| --- | --- | --- | --- |
| 1 | Badli Extn, Delhi-110042 | 276 | 541 |
| 2 | Gopal Vihar Colony , Delhi-110041 | 512 | 263 |
| 3 | Guru Sabha Rajupur (Extn) Colony, Delhi | 707 | 1076 |
| 4 | Guru Nankn Industrial Area, Bewana Road, Samaipur, Delhi | 1000 | 1407 |
| 5 | Pual Colony, North Rithala, Delhi -110085 | 297 | 552 |
| 6 | Pual Colony, North Rithala, Delhi -110085 | 1271 | 136 |
| 7 | Rithala (Extn) Village P.O Rithala, Delhi-110085 | 478 | 325 |
| 8 | Shivipur Near Badli Railway Station, Delhi-110085 | 251 | 468 |
| 9 | Suraj park Colony, B-3, Badli Industrial Area, Delhhi-110042 | 294 | 523 |
| 10 | Vijay Vihar, phase-I, Near Rithala | 644 | 347 |
| 11 | Vijay Vihar, phase-II,(Rithala) Near Rohini, Sector-4, Delhi 110085 | 438 | 382 |
| 12 | Surajpark, Samaipur, Near Sector-18, Rohini | 985 | 1306 |