ZONAL DEVELOPMENT PLAN FOR

ZONE   P-1, NARELA

(NARELA SUB-CITY)

Approved by the Delhi Development Authority

vide item no. 85/2007 dated 30.10.2007 for

inviting objections/suggestions

### DELHI DEVELOPMENT AUTHORITY

Price :- Rs. 70/-

CONTENTS

Page No.

1.0	Introduction	1

2.0	Statutory Provisions	1

3.0	Location, Boundaries and Area	2

4.0	Population, Employment and Work Force	5

5.0	Plan Provisions	5

6.0	Existing Profile	8

7.0	Objectives	9

8.0     Destination & Development Catalysts Targets	10

9.0	Proposals	12

10.0	Land use Plan	13

11.0	Proposed Change of Land-use	27

# LIST OF DRAWINGS

1.0	Location plan of Narela Sub-city (Zone P-1)	3

2.0	National Capital Territory of Delhi Zones (Division)	4

3.0	Zonal Development Plan

4.0	Pocket wise details of specific landuse

## LIST OF TABLES

1.0	Proposed Land-use Break up	14

## ANNEXURES

ANNEXURE-I	Land use Break up as per approved 	28

Zonal Plan of Sub Zone-I-1 (Narela Town)

ANNEXURE-II	Villages / Census Towns in Narala Subcity	29

ANNEXURE-III	List of unauthorized colony falling in 	        30

Narela Sub-city

ANNEXURE-III-A	List of unauthorized regularized colonies	31

(Tentative)

ANNEXURE-IV	Details of Facility Areas	32

ANNEXURE-V	List of village ponds – Alipur block	35

ANNEXURE-VI	Pockets wise details of specific landuses      36	as per zonal development plan and propo-                                         sed modifications

ANNEXURE-VII	Identification of streets for commercial use   39                                           	in Narela Zone & Identification of comer-                                     cial streets in Narela Zone

ANNEXURE-VIII	   Identification of Mixed landuse street in 	      40

Narela Zone

PREAMBLE

In accordance with Authority’s Resolution in its meeting held on 6.9.2007, genuine pre-existing institutions, i.e. before 1.1.2006, rendering cultural, religious (including spiritual) health care and educational services to the people, but which do not form part of the ridge or Gram Sabha or public land, shall be incorporated vis-a-vis their current land uses in the respective Zonal Plan-2021. Such institutions will, however, be subject to reasonable policy and procedural stipulations regarding factual verification, FAR, Development Charges, Land-use, etc.

ZONAL PLAN OF ZONE P-1 (NARELA SUB-CITY)

- INTRODUCTION

  - The NCT of Delhi has been divided in 15 nos. of planning zones (division) designated from ‘A’ to ‘P’ (except Zone-I) in the Master Plan for Delhi-2021, P-Zone is further sub divided in to two zones designated on P-1 & P-II zones. P-1 zone covers an area of       9866 hact.

  - As per MPD-2001 provisions, this zone covers Narela sub-city that comprises of parts of Zones – M, N and P. It formed part of the urban extension plan of MPD-2001 approved by the Authority vide Resolution No. 79 dated 30.6.1987.  Hereinafter this zonal plan is referred to as Zonal Plan of Narela, Zone P-1.  This zone has been developing since pre-independence era and through the MPD-1962, MPD-2001 and now MPD-2021.  As the zone has a heterogeneous character having the unplanned areas and planned areas development norms of zonal plan.

- STATUTORY PROVISIONS

  - Zonal Development Plan means a plan for a Zone (Division) of the National Capital Territory of Delhi. The Zonal Plan (Divisional Plan) details out the policy of the Master Plan and acts as a link between the layout plan and the Master Plan. The Development Schemes / lay out plan indicating use premises should conform to the Master Plan / Zonal Plan.

  - The Zonal Plan of this area stands approved vide MOUD letter no. K-13011/7/06/DDIB dt.26.052006. However, this has now been reviewed under the framework of MPD-2021.

  - MPD-2021 stipulates that already approved sub-zonal plans in conformity with master plan shall continue to be operative. The proposed zonal plan after approval shall supersede the earlier approved sub-zonal / zonal plans.

  - Section-8 of Delhi Development Act provides for preparation of Zonal Development Plan simultaneous with the preparation of Master Plan or as soon as may be, the Authority shall proceed with the preparation of Zonal Development Plan for each of the zone into which Delhi may be divided. However, a Zonal Development Plan may contain a site plan and land use plan with approximate location and extent of land uses such as public and semi-public buildings / work centers / utilities, roads, housing recreation, industries, business, markets, schools, hospitals, open spaces etc. It may also specify standards of population density and various components of development of the zones.

The procedure to be followed for the preparation and approval of the Zonal Development Plan is laid down in Section – 10 of the Act.

- LOCATION, BOUNDARIES AND AREA

  - P-1 zone is located in the North-West of NCTD & the boundaries are as under :

North:	NCTD Boundary

South:	Boundaries of Sub-zones C and H

East:	G. T. Karnal Road and its bypass (NH-1).

West:	Western Yamuna Canal.

  - The total area of the zone is 9866 Hact. of which urbanisable area of the zone is 7365 Hectares remaining 2501 Hectares is under Rural / Green Belt towards Northern border of NCTD as per MPD-2021 land use plan.

- POPULATION AND WORK FORCE

4.1	Population - As per MPD-2021 the urban extension areas are proposed to be planned with an overall city level density of 250 persons per hectare for urban extensions and based on the area under consideration the proposed population is to 1.62 million including that of the existing settlements.

  - Work Force - MPD-2021 has anticipated a participation rate of 38.1% and thereby the work-force in the Narela Sub-city works-out to about 6.2 lakh, due to the location of Industries in the sub-city, the component of work force works out to be roughly 35-40 %, i.e. in all a generation of about 6.0 lakh workers.

- PLAN PROVISIONS

5.1	Master Plan for Delhi - 1962

The MPD-62 recommended to develop Narela as one of the ring towns around Delhi Metropolitan Area (DMA) owing to its location and development potential. It proposed a township for 60000 people with government offices and industry to make the town a self contained unit. The proposal included the adjoining developments of Mamurpur village and the development undertaken by the Ministry of Rehabilitation. Broadly the land use plan for 1753 acres was detailed out with a gross residential density of 50-60 persons / acre. As per the provisions of MPD-62 a Zonal Development Plan for Narela ring Town / (Zone I-1) was prepared and approved by the Central Government. Ministry of works and Housing vide letter No. 11-2/69-UDI dated 10th January 1974. The land use break up as per approved zonal plan of zone I-1 is annexed at annexure-1.

  - Master Plan for Delhi – 2001

As per MPD-2001, urban area was proposed to be extended about 18000 to 24000 ha. which includes Narela sub-city also.

Zonal Plan of Narela Subcity

Based on the MPD-2001 the zonal plan of this zone (comprising of part zone M, N & P) covering an area of 9866 Hact. was approved vide letter No. 13011/7/06/DDIB dated 26.05.2006. The salient features of the approved zonal plan are:

- 3165 ha. of land earmarked for residential use to accommodate 12 lakh population

- For integration of goods movement by road and rail freight complex is proposed this consist of wholesale market, chemical market, ware housing, road (truck) and rail transport terminal, ISBT etc.

- To cater to the inter city passenger movement a metropolitan passenger terminal is provided in the zonal plan.

- About 1500 ha. land in this zone has been developed in the form of various housing category i.e. EWS, LIG, MIG & SFS flats. Approx. 850 nos. of alternative plots for the land owner where land is acquired.  Relocation of slum & JJ squatters from other parts of Delhi. Macro/Micro level facilities such as hospital, water & sewerage pumping station, cremation ground, convenient & local shopping centre, development of roads and about 300 ha. of Indl. Areas.

5.3	MPD-2021 provisions

MPD-2021 stipulates the holding capacity of the zone to be about 16.2 lakhs.

MPD 2021 has proposed a series of innovative concepts such as: Local area planning, Redevelopment of villages, unauthorized colonies and Built up areas, Restructuring around major transport corridors, metro corridor, a network of green & sports infrastructure etc. which will be suitably incorporated while preparing the detailed schemes.

As far as the proposed Green Belt on the periphery of NCTD is concerned, the plan will have to be developed in such a manner that the villages/abadi falling in this belt can be suitably marked and comprehensive rural development/agriculture development could be planned in the surrounding areas.

5.4 	NCR Plan-2021 Provisions

The Regional Plan for NCR-2021 notified in September, 2005 formulated a set of policies for the control of land uses and development of infrastructure in the region. The Functional Plan on Transport prepared and approved by the NCR Planning Board in 1996 envisages an organized transport network to improve accessibility and the movement of goods and passengers within the region. The Functional Plan of Transport has suggested the proposals for construction of road and rail linkages along the high-density routes in and around the Capital and also in the National Capital Region. It is proposed to develop activities keeping in view rail and road linkages with Delhi Metropolitan area and at the same time relieving pressure on the existing transport routes converging at Delhi.

- Integrated Mass Rapid Transport System (MRTS), Regional Rapid Transport System (RRTS), expressways, bypasses as proposed in the NCR Plan-2021 need to be executed.

- EXISTING PROFILE AND PHYSICAL CHARACTERSTICS

6.1	The zone P-1 is very well connected by major roads and highways & ideally suited for the urban development.  The entire zone area is a vest expanse of more or less flat terrain with land gradually slopping the south. The area has very large green tree coverage in the form of orchards, forest (declared as reserved forest) and farmlands. The sub-city area has good soil for construction. The whole of sub-city has good natural drainage system and the area is divided into two drainage basins (i) catchments of Bawana escape and (ii) catchments of Drain No. 6. Both these drains have number of subsidiaries spread over the entire sub-city area holding mainly the rural discharge.

6.2 	Other Characteristics:

The significant existing developments in the sub-city are as under:

  - The abadis of 16 villages and two census towns (Alipur and Narela) fall in the zone. A list of the villages falling in the zone with their population as per census of India 2001 is annexed as Annexure-II.

  - The large number of farmhouses exist in this area. A number of unauthorized colonies and other developments have also grown within the zone.

  - The major portion of planned development taken up by DDA is near the Narela Town. On acquired land of about 516 hact. initially for various uses.

  - The total area of 483 hact. has been identified for industrial use zone out of which 247 hact. has already been developed by DSIDC and is operational.

  - Land for utilities like water treatment plant, roads, ESS, part integrated freight complex has been acquired and are being developed.

  - Integrated Freight Complex (IFC) covering an area of 397.81 hact. is a major economic and employment generation centre.

- OBJECTIVES

The objectives of the zonal development plan are as under :-

- As per MPD-2021 this area will be developed as an integrated part of urban extension with self sufficient and high level of the services.

- This area will cater to all sections of the society in term of housing and employment.

- To provide a comprehensive framework of land/uses as envisaged in the urban extension plan of MPD-2021.

- To retain the agricultural green belt as shown in the MPD-2021 land-use plan.

- DESTINATION & DEVELOPMENT CATALYSTS TARGETS

  - Destination Targets:

- Narela Industrial Area

- Bawana Industrial Area

- Metropolitan Passenger Terminal (MPT)

- Integrated Freight Complex (IFC)

- Interstate Bus Terminal (ISBT)

- District Center and community Centers, Food & Craft Bazaar

- District Sports Center

- Integrated Schools, hospitals & Socio-cultural Centers

- International Exhibition Center/Trade Conference Center

- Hi-Tech Park

- University Center for research, higher education & colleges

  - Development Catalysts:

- Construction of 100 m. UER-II, 80 m. UER-I & III and 60 m. R/W roads for connectivity and opening of the area.

- Extension of MRTS routes from Barwala to Bawana and Holambi Kalan station.

- Increase in the frequency of shuttle train service between Delhi and Narela.

- Introducing new routes and increasing frequency of existing bus routes

- Relocation of existing grain go-downs to the designated warehousing at IFC

- Relocation of Chemical Market and wholesale markets in IFC.

- Development of industrial areas.

The above would involve certain modifications in the already approved zonal plan: -

- To provide a University Center for higher education and research.

- To provide District Sport Centre and Divisional Sport Centre.

- To provide an International Exhibition Fair ground along with trade cum conference center

- To explore possibilities of extending MRTS route up to the sub-city.

Following are the approved schemes in Narela Sub-city:

- Sector Plan of A1-A4 for 320 hectares has been approved by the Screening Committee as high value residential development to be developed on group housing basis.  The above proposal was approved by the Screening Committee in its meeting held in Oct.’2004 which has necessitated relocation of Jail & court site, service center site and readjustment of landuse in the zonal Plan of Narela Sub-city as the land under 3 HT lines passing through this residential sector which has been proposed as zonal green.

- For water treatment plant to be allotted to DJB. Tech. Committee in its meeting held dt. 25.10.04 decided to earmark 35 hact. of land at Narela.

- An area of 175 hect. has been notified for “manufacturing” use vide MOUD notification dt. 01.03.05 and DSIDC is developing it.

- The realignment of 80 mt. Urban Extension Road-I (UER-I) near FCI godown was approved by the Technical Committee in its meeting held on 12/05/2004. Subsequently the details alignment of this 80 mt. ROW and 100 mt. ROW approved from Technical Committee meeting dt.1/02/2007.

- Sector plan of G7-G8 covering an area of 299 ha. approved by Screening Committee on 7.1.2007.

- Landuse of an area measuring 290.07 hact. has been notified by the MOUD on dt. 25/01/05 from rural use zone to commercial and Transportation.

- The landuse of an area measuring 1506.89 hact. in the west of Delhi Karnal railway line upto western Yamuna canal has been notified by MOUD on dt. 03/01/05 from “Rural use zone to Residential/ Commercial/ Manufacturing/ PSP/ Recreational/ Utility/ Transportation

- PROPOSALS:

- An Education & Research Centre including University has been proposed at the northern part of the sub-city.

- As per MPD-2021 norms, the required number of Petrol Pump sites/CNG stations in the Narela sub-city works out to be 155. There are 12 existing sites which includes one CNG Mega station in Narela project, 6 of the existing sites are located on GT Karnal Road (NH-1) near Singhu Border along the G.T.K. Road, which are located in mandatory green belt need to be dealt as per NCR Plan 2021 provision. The remaining sites are proposed to be located at layout level as per prevailing MPD / Guidelines.

- The residential pockets for various housing categories are proposed for an area of 82.0 hact. in Sector A1 – A4. Keeping this in view, the jail, court complex site, govt. office site measuring 56 ha. earlier proposed in the approved zonal plan be shifted in Sector B-3 &  B-8 in the north & south of 60m R/W road near the AIR Khampur with area 16 hact for jail and 16 hact. for court complex & govt. offices.

- The landuse along UER-I, II & III have been readjusted as per MPD-2021 provision so that facility corridor comprising of ‘Public-Semipublic’ Facility/city level utilization & commercial use are located along major roads.

- Development of villages shall be integrated with the development scheme of the area. Around these settlements, education, health and recreational facilities and other uses would be developed for the benefit of the local population.

10.0	LAND USE PLAN

10.1	Following is the proposed land use break-up of this zone:

Table 1.0: Proposed Land-use Break Up

| Land use | Area (Ha) | %age |
| --- | --- | --- |
| Residential Commercial Public & Semi-public facilities Government Utilities Industrial Recreational Transportation/ Circulation | 3153.0 391.0 875.0 16.0 327.0 483.0 1029.0 1091.0 | 42.80 5.30 11.90 0.20 4.50 6.50 14.00 14.80 |
| Total | 7365.0 | 100.0 |

Note:

Area under “Recreational use” includes 242.4 Ha of AIR land (AIR Khampur + AIR Signals).

  - Residential Development

    - Existing Development

The residential development in Narela Sub-city has taken place so far in the form of various types of DDA Housing i.e. EWS, LIG, MIG /self financing. A new concept of incremental housing was also adopted keeping in view of the growing needs of a family. Beside this the unorganized growth of residential development has taken placed in the form of unauthorized colonies, extension of villages etc. The list of villages and unauthorized colonies/ unauthorized regularized colonies is annexed at annexure II, III & IIIA respectively. A large no. of slum and J.J. population has also been relocated from the mother city. One pocket in sector B-2 has also been developed as turn key project for LIG housing.

- Existing villages & Census Town- The villages and census towns are also considered at par with these as they also have the same traits. The socio-economic changes in these old unplanned areas, especially in villages have been substantial. The redevelopments plans should ensure that the permissibility of mixed use zoning at property or within the premise level is compatible to the predominant residential areas.

- Unauthorised colonies – Unauthorised colonies in Delhi pose a serious problem as a huge population is living in these colonies.

The present method of regularization of unauthorized colonies is by the provision of basic infrastructure to improve the environment. However regularization has not really brought in any tangible improvement. Effectively, the process has only led to de-facto tenure rights on the land and access to services.

In all unauthorized colonies whether on private or public land, regularization should be done as per the govt. orders issued from time to time. It must be ensured that for improvement of physical and social infrastructure, the minimum necessary/ feasible level of services and community facilities are provided.

    - New Housing Areas:

The proposed housing strategy incorporates approaches for development of new housing area, upgradation and re-densification through re-development of existing housing area including unauthorized colonies, in view of the limited availability of land and increased requirement of housing, plotted residential development shall be discouraged. The sub-city when fully developed will provide housing facilities to over 3.60 lakh families in all income groups. Involvement of private sector in development of housing will be considered as per MPD-2021 provision.

10.2.3	As per MPD-2021, the identification of mixed uses area / street shall be done as per mixed use regulation. The mixed use would be permissible on streets/stretches already notified by the competent authority. The mixed use areas will be shown in the local area plans. The list of commercial and mixed land-use streets in Narela zone as per notification of GNCTD dated 15.9.07 & 12.4.2007 is annexed at annexure – VII & VIII respectively. Keeping in view that the development of Narela sub-city is yet to come up, MCD may consider with taking forward the list of mixed use & commercial streets.

  - Commercial

Shopping and commercial areas reflect the economy and the image of the city.  As required three District Centers has been proposed in zone P-I and one remaining site under District Centers in IFC will be utilized for sub-city level whole sale and service market.

The space for informal bazaar and service market will be reserved in the District Centers, Community Centers and LSS/CSC. Integrated Freight Complex (IFC) covering an area of 397.81 hact. is major economic activity and employment generation centre.

  - Public and Semi-Public Facilities

The requirement of Public and Semi-public facilities (Social Infrastructure) up to the Zonal Plan level have been identified and located in various facility areas as per the norms. Each public and semi-public facility pocket provides a cluster of various community and above level facilities worked out as per the population of the sub-zonal pockets. Some of the major facilities proposed as:

University Centre (1 Nos.), Medical College (2 Nos.), Divisional Sports Centre (1 Nos.), District Sport Centre (3 Nos.), International Exhibition cum Fair Ground (1 Nos.), District Jail (1 Nos.).

  - Government Offices

An integrated office complex has been proposed in Sector B-3       (16 ha.) which will comprises the offices of GNCTD, Local bodies district court etc.

  - Utilities / Physical Infrastructure

    - Water Supply

The total water requirement of the sub-city works out to the approximately 120 MGD @ 60 gallons capita per day as per MPD-2021 norms. Water Treatment Plants with a capacity of 300 MGD has been planned which would serve Rohini as well. The water supply system will consist of underground reservoirs / command tanks (each for 1.5 to 2 lakh population) inter-connected by water mains. Each Command Tank will have its own command area. Land for W.T.P. along Western Yamuna Canal has been identified and transferred to D.J.B.

10.6.2	Sewerage System

Total Sewerage is expected to be generated as 90 MGD. Two STP’s have been planned. STP-I with a capacity of 20 MGD and STP-II with a capacity of 70 MGD system to consist of Gravity main sewage pumping station and rising mains. The sewerage treatment plants are towards the east of G.T. Karnal Road and are in the process of development and would take care of requirement of Narela sub-city.

10.6.3 	Storm water Drainage

Total discharge is of about 7000 cusecs. The total area falls under the catchment of Bawana escape and Drain No. 6. Also there are number of subsidiary drains such as Bakaner Drain, Sanoth Drain, Gogha Drain etc which will form major network for carrying the storm water.

10.6.4	Power

Total requirement: 1400 M.W.

System may consist of 400 KV ESS (1 No. existing) and 220 KV ESS (5 Nos., 1 already existing) and 66 KV ESS (20 Nos.). The detailed locations and numbers would be decided in consultation with concerned agencies at the time of preparation of detail plan of the sub-city.

10.6.5	Solid Waste Management

Total Solid Waste : 840 tons / day. The process of disposal to be through Sanitary landfills, composting and incineration depending upon the quality of the refuse. In future emphasis would be laid on solid waste disposal by more scientific method, where the land requirement gets reduced to minimum in consultation with the concerned agencies.

A site of about 60 hact. for solid waste has been earmarked near Narela - Bawana industrial area Ph-II.

There is a scope to improve the service delivery by proper management and coordination. The local bodies and service agencies may take into account the proposals of zonal plans for making advance preparation for the provision of services in the proposed Urban Extension. Alternatives, such as, public private partnership / joint sector /SPV and privatization may also be considered with the view to harness all the resources for making available services in a time bound manner. Decentralization of services, adoption of new technology and ecological concepts, such as, watershed planning and better ground water management, zero-run off drainage, roof top rain water harvesting, recycling, renewable energy, leakage and waste minimization, etc should be the basis of services planning and development.

  - Industry

An area of 483 Hect. has been proposed for industrial use which consist of 3 industrial clusters namely (i) Narela Industrial Area in Ph-1. (ii) Bawana Industrial Area Ph-II. (iii) Bhorgarh Industrial Area. The planning & development of these industrial areas is dealt by GNCTD. Besides these, two service centers have been proposed.

  - Recreational

    - About 1029 Hectare of land has been proposed for “Recreational’ use which comprises of community, district and sub-city level parks and play grounds as per the provision of MPD-2021. Land under forests, drains, HT lines have been proposed for ‘Recreational’ uses apart from linear green (Dist. Park) would provide for continuity of open space enhancing the visual and functional quality of built environment as well as would provide for segregated cycle / walking corridors.  About 14% of land has been proposed in the zonal plan apart from 2501 hect. green belt in the northern fringe of the sub-city. Further local / neighborhood park / play grounds will be part of the layout plan.

Multi-purpose grounds & network of the multi-purpose grounds, a category of park/ open grounds for holding marriages and other special functions has been proposed. Already approved party/marriages by the local body/MCD are designated as multi-purpose ground.

    - Environment, Green Belt and Water Bodies

Environment

One of the major objectives of the plan is creation of a sustainable physical and social environment for improving quality of life. There are two components of urban management – the environment per se, or the habitat and service management. The three told approval and strategy has been adopted:

- Management of Natural Resources and the related environment infrastructure and services in a manner that would lead to optimization of use of natural resources, and reduction/ abatement of pollution.

- Conservation and Development of the Natural features with a view to enhancing their environment value; and

- Development and preservation of open spaces, green and landscape / recreational areas.

  - Water (surface and ground)

The zone has been divided in to two drainage basins (i) Catchments of Bawana escape and (ii) Catchments of Day no.-6 both this drains have number of subsidiaries spreads over the entire sub-city area holding mainly the rural discharge.

  - Green / Recreational Area

The zone has very large green coverage in the form of orchards, forest (declare as reserved forest) and farms lands. This sub-city covers a green belt of an area of 2501 hact. all along the northern boundary of the Narela subcity. The other area under recreational / green use i.e. 1029 hact. is in the form of district parks, city parks, community parks, roads side green etc. comprising along 14% of the total urban land area.

Two major drains i.e. Drain No-6 and Bawana escape which are passing through the subcity and this have been proposed to be developed as a green corridor. Existing water bodies (1 hact. and above) are proposed to preserved and developed as a landscape feature. The list of village ponds – Alipur block is annexed at annexure- ‘V’.

10.8.3 	Conservation of Heritage

An old historical tank (mugal tank) and wall masjid situated in the revenue estate of village Kuraini has been identified as historical monuments by INTACH which is to be preserved and a conservation plan is to be prepared.

  - Transportation / Circulation

10.9.1	The present circulation system of the sub-city is based on mainly surface transport i.e. rail and road. The strengthening of the rail network supported by MRTS would provide major public transport to the sub-city from the mother city as per details given below:

- Railway Network:

The rail corridor (Broad Gauge Railway Line to Amritsar) running through the sub-city provides for goods and passenger movements. The existing railway corridor has already been electrified for the phased movement of the commuters as well as the goods from the nearby areas. In consultation with railways it is proposed to expand the main corridor to 4 tracks with a metropolitan passenger terminal near Holambi Kalan.

- Road Network:

A hierarchy of road network (100 mt., 80 mt., 60 mtr. and 40 mt. road right of Way) has been proposed. Three major arterial roads viz Urban Extension Roads (UER) – I, II & III of 80 mtr, 100 mtr & 80 mtr R/W respectively are proposed between G. T, Karnal Road (NH-1) to Rohtak Road (NH-10) and NH-8 passing through Narela, Rohini & Dwarka projects. This road system would be further connected by a hierarchy of collector local roads at the level of layout plans.

It is envisaged to segregate the different traffic modes and roads with green buffer would also combat the air and noise pollution in the area. Attempts would be made for pedestrian and bicycle friendly by using modern traffic calming design to keep the speeds within limits as per the design to provide dedicated cycle tracks for the movement within the sub-city.

- Bus Depots

Apart from the modern rail system, buses will be continued to be other major transport in the city. As per the MPD-2021 norms, the details requirement of Bus terminal & Bus depots are provided / will be provided with the development of different phases of development of the city. The new Bus Terminals need to be planned and will be developed in strategic location to make the use of MRTS and Metro station convenient for all comments.

- MRTS

As recommended in MPD-2021 a link of MRTS has been proposed from “Sanjay Gandhi Transport Terminal to Narela”. However, exact alignment of the same is to be prepared in consultation with DMRC. It may be envisaged that the proposed MRTS corridor from Rethala to Holambi Kalan could be extended further to Jhangir Puri passing through Zone P-II.

Influence Zone

It is proposed that the while preparing Local Area Plan maximum up to 500 m wide belt on both the sides of the central line of MRTS/ major transport corridors be identified in consultation with the local body and shall be designated as ‘Influence Zone’. The development of Influence Zone shall be taken up as per feasibility/availability of land and applicability of development control norms shall be as peruse zone and norms as approved by the concerned local body. These are to be in accordance with Master Plan and part of comprehensive scheme. No piecemeal permission shall be given on individual plots without a comprehensive scheme. However residential height of plots shall not exceed 15 m.

- Integrated Multi Model Transport System

The future transport system consist of mix of rail road based system (IRBT/RRTS corridors as identified in NCR Plan 2021), Bus Rapid transport System and other mass transit modes on select corridors will be identified as per the needs from time to time in accordance to prevailing policy.

    - Petrol Pumps/CNG Station & Gas godowns:

As per Master plan-2021 the provision of Petrol Pump / CNG Station & Gas Godowns are as follows:

| 1. | Two fuel stations (one petrol pump & one CNG station) per 150 ha of gross residential area : 3165 hac. | 42 |
| --- | --- | --- |
| 2. | Two fuel stations (one petrol pump & one CNG station) per 40 ha of gross industrial area; 483 hac. | 25 |
| 3. | Four fuel stations (two petrol pump & two CNG station) in each freight complex | 4 |
| 4. | Four fuel stations (two petrol pump & two CNG station) in each district center : (4 Nos.) | 12 |
| 5. | Two fuel stations (one petrol pump & one CNG station) in each community center | 26 |
| 6. | Two fuel stations (one petrol pump & one CNG station) in each PSP facility area (21 Nos.) | 42 |

The total no. of 48 Gas godowns are required at the rate of three Gas godowns for 1,00,000 population. The gas godown sites will be suitably located in the layout plans.

  - Development Strategy

- At present, the Govt. of India policy of large scale land acquisition, development and disposal prevails in Delhi.

- The change of user of land permitted by the plan (Master Plan/Zonal Plan) is only enabling in nature and only lifts a restriction, which otherwise is to be used as per the use of the property as per ownership documents.

- The land has to be used as per the approved layout plan (LOP) and no change of the use premises can be made contrary to the land ownership together with land use at premise land even if the plans (MPD/ZDP) permit suchc user.  There is no automatic conversion of the ‘rural use’ to residential/or any other urban activity.

- After a decision is taken by the Govt. to allow the betterment/development of private lands, the conversion charges are to be paid which are leviable on the basis of rates prevalent on the date of application.

- As per Development Code of MPD-2021, the National Capital Territory of Delhi is divided into 9 landuse categories and under Clause 3.0 of MPD-2021 (Page 123) each category is assigned number of use zones, which shall be further sub-divided into required number of use premises with or without conditions as part of the layout plans.

- Clause 2(4) of MPD-2021 defines a layout plan indicating configuration and size of all use premises.  As such, without an approved layout plan, the landuse can not be established for a premise.

11.0	PROPOSED CHANGE OF LAND USE

The MPD-2021 has been notified by MOUD on 7.2.2007. To accommodate the policy and modified requirements some readjustments of landuses has been carried out to optimum utilization of landuses along the major transport corridors. A care has been taken not to revise /modify the original land-use distribution stands approved. The modifications proposed along with its areas processing under section 11A of DD, Act are placed at Annexure ‘VI’.

ANNEXURE I

LAND USE BREAK UP AS PER APPROVED ZONAL

PLAN OF SUB ZONE I-1 (Narela Town)

| S. No. | Description | Phase I (Area in Acres) | Hectares |
| --- | --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. 11. 12. 13. 14. 15. | Residential Area Industrial Area District Centre Existing Mandi Master Plan Roads Transport Center Railway Station Agriculture land Existing Tank Hospital Police Station College Existing Canal Community Center Master Plan Green | 518.00 501.2 34.0 10.0 25.2 18.0 16.0 727.5 14.0 20.0 4.0 34.0 2.0 22.0 70.0 | 209.63 202.84 13.76 4.04 10.20 7.28 6.48 294.41 5.66 8.09 1.62 13.76 0.81 8.00 28.33 |
|  | Total | 2015.00 | 815.81 |

ANNEXURE - II

VILLAGES/CENSUS TOWNS IN NARELA SUBCITY

| S. No. | Name of the Village / Towns | Population (2001) |
| --- | --- | --- |
| 1. | Alipur (Cesus Town) | 16631 |
| 2. | Bakaner | 21089 |
| 3. | Bhorgarh | 5562 |
| 4. | Budhpur – Bijapur | 1269 |
| 5. | Holambi Kalan | 3876 |
| 6. | Holambi Khurd | 2705 |
| 7. | Iradat Nagar Alias Naya bans | 2509 |
| 8. | Khera Kalan | 4790 |
| 9. | Khera Khurd | 8817 |
| 10. | Kureni | 2586 |
| 11. | Mamurpur | 3465 |
| 12. | Narela Census Town | 38848 |
| 13. | Sanoth | 2909 |
| 14. | Shahpur Garhi | 959 |
| 15. | Singhola | 1878 |
| 16. | Sirsapur | 15043 |
| 17. | Tikri Khurd | 8637 |
| 18. | Razapur Kalan | 0 |

ANNEXURE - III

Tentative list of Unauthorized Colonies falling in Narela Subcity

| Sl. No. | Sl. No. as per list | Name of unauthorized colony | Sl. no. given by GNCTD | DDA dev. area nos. | M | Zone N | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1016 | Rajiv Colony Narela Delhi-40. | 50 | - | - | - | P |
| 2 | 1012 | Rajeev Colony Extn. Pio Maniari Road Narela Delhi-40. | 256 | - | - | - | P |
| 3 | 1200 | Shiv Mandir Colony, Purani Basti, Saziyabad Border, Narela, Delhi-40. | 140 | - | - | - | P |
| 4 | 401 | Gautam Colony Extn. (Arya Nagar), Safiabad Road, Narela, Delhi-40. | 463 | - | - | - | P |
| 5 | 1140 | Sanjay Colony, S-235, Safiabad, Road, Narela, Delhi-40. | 334 | - | - | - | P |
| 6 | 1139 | Sanjay Colony, Near Sabji Mandi, Arya Samaj Road, Narela, Delhi-40. | 301 | - | - | - | P |
| 7 | 402 | Gautam Vihar Colony, Narela, Delhi-40. | 475 | - | - | - | P |
| 8 | 125 | Bunkner Basti, Near Bliarad Pana, Narela, Delhi-40 | 382 | - | - | - | P |
| 9 | 129 | Basti Bunkner (South) Colony, Lampur Road, Narela, Delhi-40. | 230 | - | - | - | P |
| 10 | 60 | Ambedkar Colony, Bunkner Extn., Village Bakner, Narela, Delhi-40. | 321 | - | - | - | P |
| 11 | 59 | Ambedkar Colony Extn., Part-II, Bankner. | 736 | 175 | M | - | - |
| 12 | 890 | OM Vihar Colony, Lampur Road, Narela, Delhi-40. | 1196 | - | - | - | P |
| 13 | 853 | New Basti Lampur, Narela, Delhi-40. | 72 | - | - | - | P |
| 14 | 1162 | Swatentra Nagar Extn., West Gonda Road, Narela, Delhi-40. | 3 | - | - | - | P |
| 15 | 1302 | Swatentra Nagar Ph-II (left out portion) Narela. | 624 | - | - | - | P |
| 16 | 649 | Kureni New Basti, Narela, Delhi-40 | 285 | - | - | - | P |
| 17 | 1247 | Shyam Kunj Colony, Tikri Khurd, Delhi-40 | 160 | - | - | - | P |
| 18 | 700 | Mahavir Colony, Alipur Road, Bhorgarh, Narela, Delhi-40. | 1067 | - | - | - | P |
| 19 | 854 | New Colony, Sunnod Ghonga More, Narela Bawana. | 1180 | - | M | - | - |
| 20 | 1143 | Sannoth Extn. Part-I, II & III, Narela, Delhi-40. | 816 | Yet to be notified as DAW-175 | - | - | - |
| 21 | 1214 | Shiv Vihar, Holambi Kalan, Delhi-40. | 1430 | - | M | - | - |
| 22 | 1344 | Vashu Vihar Colony, Holambi Kalan. | 1211 | - | M | - | - |
| 23 | 77 | Ankur Vihar Colony, Holambi Kalan. | 1212 | - | M | - | - |
| 24 | 429 | Govind Colony Near Jhangola No-2, Alipur, Delhi-40. | 1093 | - | - | - | P |
| 25 | 161 | Bhgat Singh Park, Saraspur, Delhi-42. | 465 | - | - | - | P/C |
| 26 | 141 | Bhagat Singh Park Extn., Siraspur Village, Delhi-42. | 858 | - | - | - | P/C |
| 27 | 225 | Chandan Park, Near Saraspur, Narela, Delhi-42. | 431 | - | - | - | P/C |
| 28 | 564 | Jeevan Park, Siraspur, Delhi-42. | 57 | - | - | - | P |
| 29 | 345 | Extn. Rana Park Colony, Rana Park, Siraspur, Delhi-42. | 127 | - | - | - | P |
| 30 | 1052 | Rana Park, Siraspur, Delhi-42. | 108 | - | - | - | P |
| 31 | 486 | Hazari Park, Holambi Kalan, Delhi-82. | 73 | - | - | - | M |

Source :- GNCTD, 2005

ANNEXURE – III-A

List of unauthorized regularized colonies (Tentative)

| Sl. No. | Name of the colony | Municipal Zone | Date of approval |
| --- | --- | --- | --- |
| 1. | Swatantra Nagar, Narela | Narela | 1007/Stg. Dt. 19.5.81 |
| 2. | Indra Colony | Narela | 1007/Stg. Dt. 19.5.81 |
| 3. | Narela Mandi Extn. (Part) | Narela | 1007/Stg. Dt. 19.5.81 |
| 4. | Master Colony, Narela | Narela | 2919/St. Dt. 24.1.1983 |
| 5. | Shiva Nagar | Narela | 2919/St. Dt. 24.1.1983 |
| 6. | Punjabi Bagh Colony ‘G’ Block | Narela | 2919/St. Dt. 24.1.1983 |
| 7. | Vijay Nagar | Narela | 2919/St. Dt. 24.1.1983 |

Source :- M.C.D., 2006

ANNEXURE IV

DETAILS OF FACILITY AREAS

| S. No. | Name of Facility Areas | Area (In Ha) | Facility Proposed |
| --- | --- | --- | --- |
| 1. | FA-1 | 10.0 | 2 Proposed College, Hospital, Medical College |
| 2. | FA-2 | 49.0 | Hospital ‘A’, Police Station, Working Women Hostel |
| 3. | FA-3 | 9.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hospital. ‘A’, Fire Station |
| 4. | FA-4 | 4.0 |  |
| 5. | FA-5 | 40.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Vet (h), Vocational Traning, Police Station, Veterinary Institute |
| 6. | FA-6 | 32.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hos. ‘B’, Vocational Traning, Fire Station, Care Centre for physically challenged orphanage, Children Centre, Old Age Home, Religious Centre |
| 7. | FA-7 | 21.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV |
| 8. | FA-8 | 51.0 | All India Radio |
| 9. | FA-9 | 54.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hospital ‘A’, Vocational Traning, Fire Station, Care Centre for physically challenged, Working Women Hostel, Head Post Office, Telephone Exchange, Old Age Home |
| 10. | FA-10 | 56.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Religious Centre, Science Centre |
| 11 | FA-11 | 26.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hospital ‘B’, Police Station, Fire Station |
| 12 | FA-12 | 9.7 | All India Radio |
| 13 | FA-13 | 18.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Fire Station |
| 14 | FA-14 | 50.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hospital ‘B’, Professional Collages, Police Station |
| 15 | FA-15 | 37.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Vet (h), Fire Station |
| 16 | FA-16 | 42.3 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Police Station |
| 17. | FA-17 | 60.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hospital ‘A’, Fire Station, Care Centre for physically challenged, International Convention Centre, Exhibition Cum Fair Ground |
| 18. | FA-18 | 20.0 |  |
| 19. | FA-19 | 20.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV |
| 20. | FA-20 | 39.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hospital ‘B’, Fire Station, Working Women Hostel Orphanage, Children Centre, telephone Exchange, Old Age Home |
| 21. | FA-21 | 29.0 |  |
| 22. | FA-22 | 115.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Fire Station, University Campus, Medical College, Distt. Office & Battalion, Police Lines |
| 23. | FA-23 | 16.0 | JAIL |
| 24. | FA-24 | 29.0 | Hospital ‘C’, Hospital ‘D’, 2 (Maternity Hospital, Nursing Home), 1 Community Hall, 1 ESS 66 KV, Hospital ‘B’, Vet (h), Police Station, Fire Station, Orphanage, Children Centre, Fire Training Institute, Disaster Management |
| 25 | FA-25 | 23.0 | Divisional Sports Complex |

ANNEXURE – V

LIST OF VILLAGE PONDS – ALIPUR BLOCK

| Sl. No. | Name of the village / Census town (CT) | No. of Ponds | Land Ownership | Surface Area (Sq.Mt.) |
| --- | --- | --- | --- | --- |
| 1 | Bankner | A | BDO | 9,000 |
|  |  | B | BDO | 7,000 |
| 2 | Lampur | A | BDO | 3,500 |
| 3 | Sanoth | A | BDO | 25,000 |
|  |  | B | BDO | 25,000 |
|  |  | C | BDO | 2,500 |
| 4 | Holambi Khurd | A | BDO | 2,500 |
|  |  | B | BDO | 4,500 |
| 5 | Naya Bans | A | BDO | 1,800 |
|  |  | B | BDO | 2,500 |
| 6 | Narela | A | BDO | 2,500 |
|  |  | B | BDO | 3,000 |
|  | Mughal Tank | C | BDO | 4,000 |

Source :- MCD

ANNEXURE – VI

Modification / changes in approved Zonal Plan

CATEGORY I- Already notified through MPD-2021 dated. 7.02.2007

| Sl. No. | Pocket No. | Land-Use |  | Area in hact. | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  | As per approved zonal development plan of Narela | Proposed modified landuse |  |  |
| 1. | 3 A. B. C. D. E. F. | Residential Govt. office Govt. office PSP facilities Industrial Residential | Recreational Recreational Residential Residential PSP facilities PSP facilities | 42.00 4.00 12.00 40.00 6.00 4.00 | All these changes have been made in sector A1-A4 due to shifting of Jail site and utilization of land under high tension line as “green” (recreational). |
| 2. | 4 | Residential | PSP facilities | 1.00 | As per the request received from local residents for cremation ground and graveyard. |
| 3. | 5 | PSP facilities | Govt. Office | 16.00 | This change is due to shifting of govt. Offices from sector A1-A4 (details given against pkt. 3/b and 3/c) as explained in Pkt.3. |
| 4. | 6 | Recreational | PSP facilities | 16.00 | This change is due to shifting of jail from sector A1-A4 (details given against pkt.3/A). |
| 5. | 7 | Recreational | Utility | 10.50 | This site was already under the Jal Board. Land has been shown as “utility”. |
| 6. | 23 | PSP facilities | Recreational | 204 | These two pockets have huge open space area with transmission tower. Keeping part of the land for “PSP” facility, rest of the open area is proposed for “recreational”. |
| 7. | 24 | PSP facilities | Recreational | 38.40 |  |
| 8. | 25 | Circulation | Residential | 6.0 | 60M R/W reduced to 30m R/W, considering the overall road network plan of Narela Subcity. |

CATEGORY-II-  Being notified under section 11-A of DD, Act under the modified zonal                              Plan (P-1).

| Sl. No. | Pocket No. | Land-Use |  |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  | As per approved zonal development plan of Narela | Proposed modified landuse | Area in hact. |  |
| 1 | 1 | Recreational | Residential | 27.00 | This modification was approved by the TC on 16.11.05. |
| 2 | 2 | Recreational | PSP facilities | 115.00 | This modification was approved by the TC on 16.11.05. |
| 3 | 8 | Recreational | PSP facilities | 21.00 | All these modification / Adjustments were approved by the TC on 16.11.05. |
| 4 | 9 | Residential | PSP facilities | 13.80 |  |
| 5 | 10 | Residential | Industrial | 6.00 |  |
| 6 | 11 | Residential | PSP facilities | 25.02 |  |
| 7 | 12 | Utility (water treatment plant) | Recreational | 10.50 | This is in view of pocket-7. This modification was approved by the TC on 16.11.05. |
| 8 | 13 | Recreational | Residential | 26.00 | All these modification / Adjustments were approved by the TC on 16.11.05. |
| 9 | 14 | PSP facilities | Residential | 17.56 | All these modification / Adjustments were approved by the TC on 16.11.05. |
| 10 | 16 | Residential | PSP facilities | 19.60 |  |
| 11 | 17 | Residential | Recreational | 27.00 | All these modification / Adjustments were approved by the TC on 16.11.05. |
| 12 | 18 | Residential | PSP facilities | 32.00 |  |
| 13 | 19 | Residential | PSP facilities | 17.48 |  |
| 14 | 22 | PSP facilities | Residential | 25.40 |  |

Note : All these changes in fourteen pockets are proposed to :

- Adjust the above (Category-I) 8 changes incorporated in MPD-2021.

- Intensity the urban extn. Corridors (including proposed metro corridors UER- II) with non-residential land use.

- Pocket No. 15, 20 & 21 were earlier considered for T.C. but due to realignment of UER-III the change is not required.

CATEGORY-III-  Being notified under section 11-A of DD, Act under the modified zonal                              Plan (P-1) due to realignment of UER-III for approval of T.C.

| Sl. No. | Pocket No. | Land-Use |  |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  | As per approved zonal development plan of Narela | Proposed modified landuse | Area in hact. |  |
| 1 | 26 | Circulation | - Residential - PSP Facility - Commercial - Residential - Residential | 7.4 5.4 3.7 8.0 11.8 | Shifting the alignment of UER-III. |
| 2 | 27 | Recreational (Green along UER-III) | - Resedential - PSP Facility - Commercial - Residential - Residential | 7.4 5.4 3.7 8.0 11.8 | Shifting of the Green along UER-III. |
| 3 | 28 | A1. PSP Facility B1. Residential C1. Commercial D1. Residential E1. Residential | Circulation Proposed (UER-III) | 6.8 3.0 2.0 7.5 11.0 | New alignment of UER-III. |
| 4 | 29 | a1. PSP Facility b1. Residential c1. Commercial d1. Residential e1. Residential | Recreational Recreational Recreational Recreational Recreational | 6.8 3.0 2.0 7.5 11.0 | New alignment of UER-III. |
| 5 | 30 | PSP Facility | Residential | 23.5 | Shifting of landuse along UER-III. |
| 6 | 31 | Commercial | Residential | 5.6 | Shifting of landuse along UER-III. |
| 7 | 32 | PSP Facility | Residential | 25.2 | Shifting of landuse along UER-III. |
| 8 | 33 | Residential | Transportation | 6.0 | Adjustment of area plan UER-III. |
| 9 | 34 | Residential | PSP Facility | 23.5 | Shifting of landuse along UER-III. |
| 10 | 35 | Residential | Commercial | 5.6 | Shifting of landuse along UER-III. |
| 11 | 36 | Residential | PSP Facility | 25.2 | Shifting of landuse along UER-III. |
| 12 | 37 | Residential | PSP Facility | 15.0 | Shifting of landuse along UER-III. |
| 13 | 38 | Residential | Commercial | 6.3 | Shifting of landuse along UER-III. |
| 14 | 39 | Residential | PSP Facility | 12.5 | Shifting of landuse along UER-III. |
| 15 | 40 | Recreational | PSP Facility | 23.0 | Change proposed in MPD-2021 (Divisional Sports Complex). |
| 16 | 41 | Residential | Circulation | 18.0 | Proposal of 60m R/W road in south of UER-III near Siras Pur & shifting or road along Rly. Line. |

ANNEXURE – VII

IDENTIFICATION OF STREETS FOR COMMERCIAL USE IN      NARELA ZONE (as per notification dt. 15.9.2006)

| Sl. No. | Name of the Road/Street | Stretch of Road From – To | ROW (Mtrs) |
| --- | --- | --- | --- |
| 1. | Arya Samaj Road | Ram Dev Chowk to Y Turn | 18 |
| 2. | Narela – Lampur Road | t. Point to Railway X-ing | 24 |

IDENTIFICATION OF COMMERCIAL STREETS IN NARELA ZONE      (as per notification of GNCTD dt. 12.4.2007)

| Sl. No. | Name of the Road/Street | Stretch |  | R.O.W (In Mtrs) |
| --- | --- | --- | --- | --- |
|  |  | From | To |  |
| 1. | Main Swatantar Nagar Road | Bawana Narela Road | Narela Lampur Road | 9.00 |
| 2. | Narela Bawana Road | Plot No.68 | Plot No. 2152 only northern side | 30.00 |
|  |  | H. No. 2142 | Railway Boundary | 30.00 |
|  |  | Ram Dev Chowk | Railway crossing Bawana Road | 30.00 |
|  |  | Swami Swaroopanan | Railway crossing | 30.00 |
|  | Chaudhary Meer Sing Marg | Narela Bawana Road | Bankner Railway crossing | 13.00 |
| 3. | Punjabi Colony Road | Physical Centre | Govt. Girls Sr. Sec. School | 7.00 |

ANNEXURE – VIII

IDENTIFICATION OF MIXED LANDUSE STREETS IN NARELA ZONE      (as per notification of GNCTD dt. 12.4.2007)

| Sl. No. | Name of the Road/Street | Stretch |  | R.O.W (In Mtrs) |
| --- | --- | --- | --- | --- |
|  |  | From | To |  |
| 1. | Hawaldar Mukhtyar Singh | End of Park | Main Auchandi Road Bawana | 6.00 |
| 2. | Shivaji Nagar Road | MCD Park | L Point of Plot No. 73 only eastern side | 6.00 |