DRAFT ZONAL DEVELOPMENT PLAN

ZONE   G    ( WEST DELHI-1)

DELHI DEVELOPMENT  AUTHORITY

CONTENTS

|  | CONTENTS | PAGE |
| --- | --- | --- |
| 1.0 | Introduction | 1 |
| 1.1 | Location, Boundaries and Area | 1 |
| 1.2 | Statutory Provisions and Objectives | 1 |
| 1.3 | Earlier approved Zonal Development Plan Proposals | 1 |
| 1.4 1.4.1 1.4.2. 1.4.3 1.4.4 | Special characteristic of the Zone Green belt Cantonment area Airport Area Tihar Jail | 2 3 3 5 |
| 2.0 | Zonal Development Plan Proposals | 5 |
| 2.1 | Population & Employment | 5 |
| 2.2 | Residential Development | 6 |
| 2.2.1 | Urban Extension | 6 |
| 2.2.2 | Restructuring and upgrading/Redevelopment and Existing areas | 7 |
| 2.2.3 | Un planned Areas villages and unauthorized colonies, Regularised unauthorized colonies | 7 |
| 2.2.4 | Local Area Plans | 8 |
| 2.2.5 | Mixed Use | 8 |
| 2.3 | Work Centres | 9 |
| 2.3.1 | Industry | 9 |
| 2.3.2 2.3.3 | Redevelopment Scheme Strategy | 9 9 |
| 2.3.4 | Redevelopment of Unplanned Area | 10 |
| 2.3.5 | Trade & Commerce | 11 |
| 2.3.5 | District Centres/Sub-District Centre | 11-12 |
| 2.3.6 | Community Centres | 12 |
| 2.3.7 | Local Shopping Centres (LSCs)/ Convenience Shopping Centres | 13 |
| 2.3.8 | Service Market | 14 |
| 2.3.9 | Informal Trade | 14 |
| 2.3.10 | Wholesa1e Market & Warehousing | 15 |
| 2.3.11 | Government Offices | 15 |
| 2.4 | Social Infrastructure | 15 |
| 2.4.1 | Health | 16 |
| 2.4.2 | Education : Colleges, Sr. Sec. School | 16-17 |
| 2.4.3 | Sports Facilities | 18 |
| 2.4.4 | Communications | 19 |
| 2.4.5 | Security and Safety : - Police - Fire | 19 18 18 |
| 2.4.6 | Disaster Management | 19 |
| 2.4.7 | Distributive facilities | 2019 |
| 2.4.8 | Other community facilities | 20 |
| 2.4.9 | Cremation ground/Burial ground and cemetery | 21 |
| 2.4.10 | Socio cultural facilities | 21 |
| 2.5 | Transportation | 21 |
| 2.5.1 | Roads | 21 |
| 2.5.2 | Rail | 21 |
| 2.5.3 2.5.4 | Air Existing/Proposed Grade Separators and Existing Bridges Important Intersections | 22 22 23 |
| 2.5.5 2.5.6 | MRTS Corridor Influence Zone | 24 24 |
| 2.5.7 | BRTS Corridor | 24 |
| 2.5.8 | Bus Depot | 25 |
| 2.5.9 | Bus Depots &Terminals | 25 |
| 2.5.10 2.5.11 | Fuel Stations Petrol Pumps | 25 26 |
| 2.5.12 | Bicycle/Cycle Rickshaw | 26 |
| 2.5.13 | Railway stations/yards | 26 |
| 2.5.14 | Parking | 27 |
|  | - Proposed surface parking - Night Parking | 27 27 |
|  | - Multi level parking - Other parking areas - Park and Ride | 27 28 28 |
| 2.6 | Environment | 28 |
| 2.6.1 | Environment | 28 |
| 2.6.2 | Recreational use, Recreational Area | 29 |
| 2.6.4 | District Parks & Sports Stadium | 30 |
| 2.6.5 | Neighbourhood Parks/Play Areas (NHP) | 30 |
| 2.6.6 2.6.7 | Pollution Control Multipurpose Grounds | 30 31 |
| 2.7 | Urban Design | 31 |
| 2.8 | Conservation of Built Heritage/Archaeological sites the zone | 31 |
| 2.9 | Physical Infrastructure | 31 |
| 2.9.1 2.9.2 | Water Supply Sewerage | 31 32 |
| 2.9.3 | Drainage | 32 |
| 2.9.4 | Electricity | 32 |
| 2.9.5 | Solid Wastes | 33 |
| 2.10 2.11.1 2.11.2 | Proposed landuse Change of landuse Phasing & Implementation | 33 34 35 |
| 2.12 | Plan Review & Monitoring | 35 |

TABLES

| Table - 1.3 | Sub-Zones | 2 |
| --- | --- | --- |
| Table – 2.1 | Sub-Zone wise population and holding capacity | 5 |
| Table-2.3.6 | Community Centres | 13 |
| Table – 2.3.7 | Local Shopping Centres | 14 |
| Table – 2.4.1 | Hospitals | 16 |
| Table – 2.4.2(1) | Colleges | 17 |
| Table – 2.4.2(2) | Sr. Sec. Schools | 18 |
| Table – 2.5.9 | Bus Depot/Terminal | 25 |
| Table – 2.6.3 | Recreational Area | 29 |
| Table – 2.10 | Landuse distribution for Zone ‘G’ | 33 |
| Table – 2.10(1) | Distribution of land use for sub-zone G-18 | 34 |
| Table – 2.11.1 | Cases involving change of landuse | 34 |
| Table – 2.12.1 | Distribution of facilities of zonal plan | 35 |
| Table – 2.12.2 | Requirement of facilities as per district level in Zone ‘G’ | 36 |
| Table – 2.12.3 | Requirement of facilities as per zonal/ Sub-city level in zone ‘G’ | 37 |

ANNEXURES								Page No.

Annexure A			Major  Roads					39

Annexure B			Urban Villages Falling in Zone G		40

(Excluding Cantonment area)

Annexure C			Tentative list of unauthorized /			41

Regularized colonies in zone ‘G’

Annexure D			List of Resettlement Colonies in Zone G	42

Annexure E			Rehabilitation Colonies in Zone G		42

Annexure F			Commercial/Mixed Use Streets		      43-55

ZONAL DEVELOPMENT  PLAN  OF ZONE –‘G’ ( WEST DELHI :I )

- INTRODUCTION

1.1       LOCATION, BOUNDARIES AND AREA

As per  the Master Plan for Delhi – 2021, the National Capital Territory of Delhi has been divided in to 15 zones (Divisions), designated ‘A’ to ‘P’ (except zone “I” ). The Zonal Plans of eleven zones including Zone G under MPD  2001 have been approved and notified .

Zone ‘G’ (West Delhi – I )   is located in the West Delhi covering an area of about 11,865 Ha. and consists of 18 sub-zones. The zone is surrounded by Delhi Rohtak Railway Line in the North, Delhi Rewari railway line in the South -East, Pankha road and Najafgarh drain in the South-West and   UT Boundary.

The zone has been developing since pre-independence era and through MPD-1962,. MPD-2001 and now MPD-2021.  As such, the zone has a heterogeneous characters, where the unplanned areas and planned areas developed under the norms of various plans co-exist.

1.2	STATUTORY PROVISIONS AND OBJECTIVES :

The Zonal plans  are  prepared under section 8 and processed under section 10 and simultaneously the modifications of land uses shall be processed under section 11 (A) DD Act 1957.

1.3	EARLIER APPROVED ZONAL DEVELOPMENT PLAN PROPOSALS

Zonal Development Plan of Zone-G as per MPD-2001 was approved by the Ministry of UD  vide letter no. K-13011/7/2006 DDIB dated 26.05.06.

Zone ‘G’ is divided into 18 sub zones, as per the details given below

Table 1.3

| S. No. | Sub Zone | Name | Area in Hact. |
| --- | --- | --- | --- |
| 1. | G-I | Najafgarh Road Industrial Area | 246.00 |
| 2. | G-2 | Ramesh Nagar | 372.00 |
| 3. | G-3 | Moti Nagar | 183.00 |
| 4. | G-4 | Jai Dev park Area | 169.00 |
| 5. | G-5,6,7,11,12 &15 | Cantonment Area and Airport Area | 4934.00 |
| 6. | G-8 | Rajouri Garden | 866.00 |
| 7. | G-9 | Tatar Pur Area | 320.00 |
| 8. | G-10 | Madipur Area | 535.00 |
| 9 | G- 13 | Janakpuri Area | 681.00 |
| 10 | G-14 | Tilak Nagar | 507.00 |
| 11. | G-16 | Vikas Puri/ Bodella | 514.00 |
| 12. | G-17 | Jwalaheri/ Paschim Vihar | 1063.00 |
| 13. | G-18 | West of NH-8 Area | 1475.00 |
| Total |  | Sub Zone 1 to 18 | 11,865.00 |

1.4	 Special Characteristics of the zone  are Rohtak Road, Najafgarh Drain, Najafgarh Road, NH-8, and Ring Road. Important features  of the zone and their  Planning provisions    are as follows:

1.4.1    GREEN BELT

Southern part of the zone along the border of NCTD earlier designated as Rural / Agriculture land is shown as Green Belt, in synergy with the provision of Regional plan 2021 of NCR/MPD-2021. The belt extends from the NCTD boundary up to a depth of one peripheral revenue village boundary, wherever possible.  There are  two villages namely,  Bijwasan and Kapeshera village which are  falling within the Green Belt.

Part of sub-zone  G-18 which was also earlier designated as Rural use zone is considered for new planned urban   extension area .  The area is bounded by Rewari Railway Line in the West. Airport on the North, NH-8 on the East and U.T. boundary line on the South. In this area land is proposed to be broadly used for Residential, Commercial, Industrial, Recreational, Public and Semi-public facilities and Circulation etc.

Permissibility/Activities and development in the Green Belt are to be in accordance with the provisions stipulated in the Master Plan.

1.4.2	CANTONMENT AREA

The Cantonment Area is consisting of five sub-zones viz G-5, 6,7,11 and 12. The development of these sub-zones will be taken up as per the provision of the Master plan  by the concerned land owning agency.

1.4.3	AIRPORT AREA

Sub-zone G-15, comprising of Airport area, approximately about 2260 hac.  is bounded by Rewari Railway Line on the West, Cantt. Area on the north, NH-8 in the East and  sub-zone G-18 on its South

The international and domestic  Passenger movement in Delhi is catered by Indira Gandhi International Airport and Palam Airport                                                                                                                                                                                respectively. Both the Airports have been linked to other part of the city and urban extension through the transport network to facilitate fast movement. Development / Redevelopment proposals are to be prepared by Airport Authority within the overall frame work of the Master Plan.

The total area earmarked for the Airport is about 2260 ha. vide Ministry of Civil Aviation letter dt.7.1.98, “AAI should prepare MP of airport and have an arrangement with local bodies so that MP of the airport does not violate the MP of local bodies such as DDA.”

Further it is suggested that while working out the development plan of the Airport, the following is ensured:

- Connecting International Airport and Domestic Airport Terminals by internal road network.

- To connect the Airport with the city by Metro in consultation with DMRC.

- Traffic Management improvement and widening of the roads within and surrounding the airport. It may be noted that the NH-8 is to be developed to 90 M. Right of way.

- Augmentation of parking facilities, including buses and taxies.

- Integrating the airport with the proposed Metropolitan Railway Passenger Terminal at Dwarka, ISBT, Metro corridor and other public transport.

As compared to the present volume of traffic to the Airport to the tune of

16.2 million passenger per annum, the forecast for 2007 is 20.4 million passenger which will enhanced to 66.4 million passenger per annum in the year 2021.  To cater to such a heavy volume of traffic, adequate road network will have to be developed alongwith parking. This requires provisions of proper circulation network within the Airport and its periphery. The development of commercial areas/hotels within the airport area, needs to  be seen in view of its implications on traffic and transportation network of surrounding roads/NH-8.

The development of Airport will require augmentation of utilities

particularly power, water supply and drainage facilities. Accordingly, Service plans may be prepared in consultation with the Service Agencies/DJB/MCD.  It is proposed that existing villages/abadi be shifted outside the airport complex and the area thus available be utilized for transportation and parking requirement.

1.4.4	TIHAR JAIL

The Central Jail is located in sub-zone G-13. This is a high security central jail and is the only of its kind not only in Delhi. Programmes for redevelopment of the Jail area are to be prepared by concerned authority/ GNCTD  within the overall framework of the Master Plan.

2.0	ZONAL DEVELOPMENT PLAN  PROPOSALS

2.1	POPULATION AND EMPLOYMENT

Master Plan  stipulates following estimated holding capacity and existing

population for zone-G

i 	Holding capacity

In MPD – 2001				- 14,90,000

ii	Existing population 2001			- 16,29,000

iii	Holding Capacity as per MPD-2021	           -  19,55,000

Table 2.1

SUB ZONE WISE POPULATION AND HOLDING CAPACITY :

| SUB-ZONE | AREA (HACT) | POPULATION |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | POPULATION 1991 | HOLDING CAPACITY 2001 | POPULATION 2001 | HOLDING CAPACITY 2021 |
| 1 | 2 | 3 | 4 | 5 | 6 |
| G-1 | 246.00 | 43,824 | 40,000 | 45,000 | 73,500.00 |
| G-2 | 372.00 | 68,231 | 74,000 | 76,075 | 108,000.00 |
| G-3 | 183.00 | 58,361 | 60,000 | 64,000 | 72,000.00 |
| G-4 | 169.00 | 15,000 | 33,200 | 33,806 | 44,411.00 |
| G-8 | 866.00 | 1,69,282 | 1,90,700 | 1,92,620 | 2,41,000.00 |
| G-9 | 320.00 | 15,545 | 1,17,715 | 1,16,390 | 1,22,390.00 |
| G-10 | 535.00 | 61,000 | 1,02,780 | 1,01,910 | 1,50,750.00 |
| G-13 | 681.00 | 1,62,615 | 1,14,540 | 2,09,475 | 2,16,475.00 |
| G-14 | 507.00 | 1,77,666 | 1,67,515 | 1,71,414 | 1,80,414.00 |
| G-16 | 514.00 | 93,643 | 1,06,000 | 95,420 | 1,43,500.00 |
| G-17 | 1063.00 | 1,23,810 | 2,42,900 | 2,14,680 | 2,95,750.00 |
| Cantt. Area G-5, 6, 7, 11, 12 & 15 | 4934.00 | 94,393 | 1,00,000 | 1,08,000 | 1,06,600.00 |
| G-18 | 1475.00 | 46,237 | 40,220 | 2,00,210 | 2,00,210.00 |
| Total | 11,865.00 | 11,29,607 | 13,89,570 | 16,29,122 | 19,55,000 |

2.2	RESIDENTIAL DEVELOPMENT

A Redevelopment Strategy for accommodating proposed population as per MPD - 2021 is to be taken up by  efficient utilization of the existing urban land both in planned and un planned area, considering the infrastructure provisions.  The  additional population of about 3.2  lacs (1.6 lacs in G-18 area and 1.6 lacs in other sub-zones)  are to be accommodated  within the zone.

    - URBAN EXTENSION:

Sub Zone G-18 with an area of 1475 Hact. as a part of urban extension as envisaged in MPD-2021  is proposed to be developed as under :

( I )	Green Belt	-		845 Hact.

( ii )	Urbanisable  Areas	-	630 Hact.

About 283 hac.  of area is proposed to be developed under residential use zone  which will create approximately 3,200 dwelling units. Facility corridors along major roads comprising of non-residential activities such as commercial, recreational,  Public and Semi Public facilities and utilities are  proposed along major transport network for the remaining area.  The proposed Urban Extension will accommodate  population of about 1.6 lacs.

    - RESTRUCTURING AND UPGRADING/REDEVELOPMENT OF EXISTING AREAS:

To accommodate  additional  population of about 1.6 lacs  it is proposed that restructuring and upgradation of existing built up areas are to be taken up by the local body in different sub-zones,  as per norms prescribed in the Master Plan.

    - UNPLANNED AREAS :

VILLAGES :		There are 19 villages falling in Zone ‘G’ (excluding cantonment area) (Refer Annexure  ‘B’).  The socio-economic changes in these villages have been substantial.  The redevelopment plans should ensure that the permissibility of mixed use zoning at property or within the premise level is compatible to the predominant residential areas.  Village Redevelopment Schemes must be prepared for villages for development and upgradation.  The village abadi shall be treated as ‘Special Area’ where Special Area Regulations of MPD-2021 shall be applicable.

REHABILITATION COLONIES : 	There are several large rehabilitation 	colonies  which  provided  accommodation    for 	   the displaced person after the independence of the country.  These include Ramesh Nagar, Kirti Nagar, Moti Nagar, Punjabi Bagh East, Subhash Nagar, Rajouri Garden, Tilak Nagar etc.  These have become very congested, mainly due to non-residential activities in the residential premises.  It is therefore, necessary that for certain pockets where there is a heavy congestion urban renewal/redevelopment  schemes be prepared and required infrastructure be provided  by the local body.

UNAUTHORISED/ REGULARISED  COLONIES :  There 	are 1432 unauthorised colonies identified by Govt. of NCTD  which are under the process of regularization.  The list of the colonies falling in this zone are enclosed at (Annexure ‘C’ ).

In all unauthorized colonies whether on private or public land, regularization shall be done as per the government orders issued from time to time.  It must be ensured that for improvement of physical and social infrastructure, the minimum necessary/feasible level of services and community facilities are provided as per norms given in MPD-2021.

The unauthorized regularized colonies are proposed to be brought under planning  purview with proper development. It is proposed that these areas should be redeveloped ensuring modern services and amenities for a safe residential environment. The owners can jointly redevelop the areas based on the guidelines prescribed in the Master Plan.

RESETTLEMENT  COLONIES  :	The resettlement colonies falling in the zone is given at Annexure ‘D’.  These are to be redeveloped as stipulated in the  Master Plan.

    - LOCAL AREA PLANS  :

Local Area Plans are to be  prepared by the local body as  a part of   comprehensive scheme     and development  programme are to be taken  as per   approved   norms for  different use   zones.  Accordingly Services, community facilities, recreational and transport  network need to be strengthened   as per the redevelopment plan/norms.

2.2.5	MIXED USE:

Mixed use means the provision for non-residential activities in residential premises.  Mixed use streets/commercial streets and pedestrian streets

in the zone under mixed use regulations as envisaged in MPD-2021 have been notified by the GNCTD vide notification dated 15.09.06 for local commercial and mixed use activities, which  are  given in Annexure  ‘F’.

The extent of mixed use permissible in various categories of colonies and permissible/ non Permissible Uses on mixed use streets are clarified in MPD, which are   to be followed.

2.3	 WORK CENTERS

2.3.1	 INDUSTRY

The major extensive industrial areas located in this zone are Najafgarh road Industrial Area, North Industrial Area, Karam Pura Industrial area, Kirti Nagar Industrial Area, Lakkad Mandi, Mayapuri ( Rewari line ) Ph. I - II and Udyog Nagar, DSIDC Rohtak Road . These are the major employment centres of the division contributing employment around 2/5th of the total work force of the division.

It is proposed that Industries listed under prohibited category in the Master Plan shall not be permitted in the zone.

    - REDEVELOPMENT SCHEMES

The redevelopment schemes cover the following aspects:

Modernization and upgradation of Industrial planned areas and

redevelopment of areas which have become Industrialized over the period of Master Plans even though these are not designated as ‘Industrial’.

    - STRATEGY

i	It is proposed that polluting industries which are under prohibited category

need to shift themselves outside Delhi, within a period of three years after notification of the Zonal Plan. Lists of such industries need to be prepared by Industries Deptt. GNCTD.

ii	To maintain better environmental conditions the present industrial outlets require to adopt modern technologies which are not labour intensive.

iii	For  up gradation of industries in old industrial areas it is recommended that industrial units should take suitable measure and restrict pollution.

iv	Activities in Household industries are to be restricted as per activities prescribed under Group ‘A’ of MPD-2021which  should adhere to pollution control norms.

v	Besides planned industrial scheme, it is recommended that other industrial units functioning the zone must adhere to norms stipulated in the MPD.

vi	Redevelopment programme for upliftment of industrial units should be prepared by the local body in consultation with the Association.

vii	It is recommended that periodic feasibility check up/review should be undertaken by the Association regularly and improvement of industrial units be monitored to maintain pollution free environment.

    - REDEVELOPMENT OF UNPLANNED AREAS

Besides planned Industrial areas, the Govt. of Delhi has notified following non conforming clusters of Industrial concentration having more than 70% plots in the clusters within Industrial activities for redevelopment. These are:

    - Peeragarhi Village.

    - Khyala.

iii   Basai Dara Pur

The redevelopment process involves preparation of re-development plans for widening of roads, laying of services development of open spaces and parking etc. the re-development of above areas needs to be based on clearly defined parameters in terms of types of Industries which may be permitted. The redevelopment plans are to be prepared as per norms prescribed in the master plan.

The following community facility / PSP use  is permissible under sub-clause 8(2) of MPD-2021 in  Residential use zone as part of approval of Layout Plan  and as special permission from the Authority :

- Hospital (up to 100 beds)

- Primary Health Centre/Family Welfare Centre/Maternity Home/dispensary etc.

- Nursing Home/poly clinic/clinic/clinical laboratory etc.

- Dispensary for pet and animals

- Primary School/Middle School/Sr. Sec. School

- School for Mentally/Physically Challenged

- Technical Training Centre (ITI/Polytechnic/Vocational/Training Institute/Management Institute/Teacher Training Institute, etc.

- Facilities – Bus terminal, taxi stand, mil / vegetable booths, religious premises, vending booth, petrol/ CNG filling pump, recreational club, police post, police station, fire station, post office, & telegraph office and telephone exchange

2.3.5	TRADE AND COMMERCE

With the Mixed use strupulation of MPD – 2021, the shopping and commercial areas are going to increase substantially in the Zone . It is proposed that scattered district centres and other commercial areas of the zone are to be linked properly by the  MRTS corridor.

DISTRICT CENTRES

The District Centres are meant to serve as the apex of the  hierarchy of the Commercial centres which are  conceived as major work & commercial  centres, while serving the community with a reasonable variety of shopping,  services and  social, cultural activity.

The District Centres already developed or in advance stages of

development in the zone are as follows:-

- Peeragarhi (Rohtak Road G-17)

- Paschim Vihar (G-17)

- Janak Puri (G-13)

- Raja Garden (G-9)

SUB DISTRICT CENTRES

One sub-district centre at Hari Nagar is proposed. These centres were developed on the basis of integrated schemes and needs upgradation in terms of infrastructure, parking spaces, hawking plaza and other physical infrastructure facilities.

The implementing agency/land owning agency can undertake review for overall upgradation of these centres. The maintenance of these centres should be periodically checked and reviewed by the local body for better environmental condition. .

    - COMMUNITY CENTRES

The Community Centres are conceived as shopping and business centre catering to the needs of the population at community level. These centres may have retail shopping., commercial and Govt. officers, local body/public sector undertaking Offices, Cinema/Cineplexus, Hotels, Restaurant, Banquet Halls and Guest House, Nursing Home, Dispensaries, clinical Laboratories as indicated in the Master Plan. Presently 18  community centres are existing and 7  are proposed in the zone.  The area  under G-18 forming part of West of NH-8 area shall have separate requirement of commercial activities which are indicated in the proceeding paras. The existing built up Community Centres may be upgraded  as per norms stipulated in the Master Plan,  in consultation with the owners/association for upgradation of the commercial unit.  Details of Community Centres  in the zone are as follows :

Table 2.3.6

| Community No. | Required | Position in Zonal Plan ----------------------------------------------------- Eixst. Prop. Total |  |  |
| --- | --- | --- | --- | --- |
| 1 | 1 | 2 | - | 2 |
| 2 | 1 | 3 | - | 3 |
| 3&4 | 2 | 6 | - | 6 |
| 5 | 1 | - | 1 | 1 |
| 6 | 1 | 1 | - | 1 |
| 7&8 | 2 | 3 | - | 3 |
| 9 | 2 | 1 | 2 | 3 |
| 10 | 1 | 1 | 2 | 3 |
| 11&12 | 2 | 1 | 2 | 3 |
| Total | 13 | 18 | 7 | 25 |

    - LOCAL SHOPPING CENTRES (LSCs)/CONVENIENCE SHOPPING CENTRES (CSCs)

The LSCs and CSCs will cater to the day to day needs of the local population. The area developed prior to 1962 may continue subject to conditions prescribed in the Master Plan. The existing built up LSCs/CSCs may be redeveloped if need be with enhanced FAR subject to payment of appropriate levies.

LOCAL SHOPPING   						Table 2.3.7

| Community NO. | Required |  | Position in Zonal Plan |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Exist. Prop. Total |  |  |  |  |  |
| 1. |  | 4 | 1 |  | - |  | 1 |  |
| 2. |  | 8 | 11 |  | - |  | 11 |  |
| 3 & 4 |  | 12 | 1 |  | - |  | 1 |  |
| 5. |  | 7 | 2 |  | - |  | 2 |  |
| 6. |  | 6 | 3 |  | - |  | 3 |  |
| 7 & 8 |  | 13 | 6 |  | - |  | 6 |  |
| 9. |  | 11 | - |  | - |  | - |  |
| 10. |  | 5 | 5 |  | - |  | 5 |  |
| 11 & 12. |  | 13 | 17 |  | - |  | 17 |  |

    - SERVICE MARKETS

Special attention is required for low turn over and day to day needs  shops for fruits and vegetables, service and repairs, junk and scrap material (Kabari), building materials, automobile workshop etc. the grouping of such activities with planned retail markets leads to conversion of shops into high profit commercial activity. To avoid such situation, it is proposed to develop service market on unutilized LSC/CSC sites.

    - INFORMAL TRADE

Informal Trade in various use zones and activities such as informal shops, weekly markets, organized eating places, Handicraft Bazar etc. may be taken up. The Provision of informal trade as per norms in the MPD should be ensured. Utilities, Public conveniences shall also be provided as per  the requirement.

    - WHOLESALE MARKET AND WARE HOUSING

Regional wholesale markets along with warehousing and other facilities are proposed to be developed on the major entry routes to Delhi. These are proposed near Mayapuri Industrial area and near Janakpuri area & Third Dilli Haatt project in an area of 3.92 hact (8 acres) Near Janakpuri area (warehousing area) is proposed to be developed by Delhi Tourism and

Transportation  Development Corporation Ltd.

    - GOVERNMENT OFFICES:

The Govt. areas  are  mainly Cantonment area comprising of sub-zone G-5, 6,7,11 and 12, Covering an area of about 2674  Hac  The Govt. of India Press is located in Mayapuri Indl. Area ( G-8).  Other central Govt. offices and offices of NCTD, are  located in different parts of the zone.  For optimum utilization of Govt. Land, it is recommended that redevelopment plan may be prepared and approved by the local body as per norms stipulated in the Master Plan.

  - SOCIAL INFRASTRUCTURE

Social infrastructure can be looked at, in terms of  facilities  indicated  at  the city level and community facilities which are indicated at the Layout level in various use zones. Social infrastructure includes provision of health, education, sports facilities, communication, security and safety, disaster management, distributive and other facilities.

    - HEALTH

Presently 2  General Hospitals and 2 Intermediate Hospitals are existing in the Zone. There  is a considerable amount of deficient hospital beds in the zone. A list has been prepared by the Directorate of Health Services, Govt. of NCT as per provisions made in MPD-2021, where new hospitals and health centres are proposed. The proposals/ land requirement shall be taken up as and  when implemented by GNCTD/ the local body in a Phased member.

TABLE  2.4.1

HOSPITALS

| Community No. | Required |  | Position in Zonal Plan |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | EXIST. PROD. TOTAL Gen. Inter. Gen. Inter Gen. Inter |  |  |  |  |  |
|  | Gen. Inter. |  |  |  |  |  |  |  |
| 1. | - | 1 | - | - | - | - | - | - |
| 2. | 2 | 2 | 1 | - | - | - | 1 | - |
| 3 & 4 | - | 4 | 1 | - | - | - | 1 | - |
| 5. | - | 2 | - | - | - | - | - | - |
| 6. | - | 2 |  | 1 | - | 1 | - | 2 |
| 7 & 8 | 2 | 4 | - | 1 | 1 | - | 1 | 1 |
| 9. | - | 3 | - | - | 1 | - | 1 | - |
| 10. | - | 2 | - | - | - | - | - | - |
| 11 & 12. | 1 | 4 | - | - | 1 | 4 | 1 | 4 |
| Total | 5 | 24 | 2 | 2 | 3 | 5 | 5 | 7 |

    - EDUCATION

COLLEGES:

In the zone presently 6 No. of colleges are existing. As per UGC norms, 5 lac of population may have one general college/ professional college. The details of existing and proposed colleges are given below.

Other/ educational facilities such as vocational training centers, ITI, Polytechnic, training institute, management institute, ITI  are proposed to be developed as per norms/requirement from time to time.

COLLEGES							TABLE 2.4.2(1)

| Community No. | Required as per MPD-2021 | Position in Zonal Plan |  |  |
| --- | --- | --- | --- | --- |
|  |  | Exist. Prop. Total |  |  |
| 1 | 1 | 1 | 1 | 2 |
| 2 | 1 | 1 | - | 1 |
| 3 & 4 | 2 | 1 | 1 | 2 |
| 5 | 1 | 1 | - | 1 |
| 6 | 1 | 1 | - | 1 |
| 7 | - | 1 | 1 | 2 |
| 8 | 2 | - | - | - |
| 9 | 2 | - | - | - |
| 10 | 1 | - | - | - |
| 11 & 12 | 2 | - | 1 | 1 |
| TOTAL | 13 | 6 | 4 | 10 |

SR. SEC.  SCHOOLS

At present there are 93 Sr. Sec. Schools functioning in the zone. Many of Sr. Sec. Schools are running in double shifts. New Schools as per Master Plan norms are proposed.

TABLE  2.4.2.(2)

SR. SECONDARY SCHOOL

| Community No. | Required | Position in Zonal Plan. |  |  |
| --- | --- | --- | --- | --- |
|  |  | exist. Prop. Total |  |  |
| 1 | 9 | 6 | 2 | 6 |
| 2 | 17 | 8 | 3 | 11 |
| 3 & 4 | 24 | 19 | 1 | 20 |
| 5 | 14 | 5 | 2 | 7 |
| 6 | 12 | 5 | 1 | 6 |
| 7 & 8 | 26 | 16 | - | 16 |
| 9 | 22 | 6 | 1 | 7 |
| 10 | 11 | 9 | - | 9 |
| 11 & 12 | 27 | 19 | - | 19 |
| TOTAL | 162 | 93 | 10 | 103 |

    - SPORTS FACILITIES:

The requirement of sports activities have been considered with the aim of development of sports and play areas for all age groups at appropriate levels.

- It is proposed that sports facilities shall be developed according to proper layout plan.

- The existing sports infrastructure shall be up-graded and efficiently re-planned to provide better facilities.

- New play field shall be preferably provided/developed in the vicinity of educational institutions and landscape areas.

- The sports facilities areas should be accessible by a network of pedestrian and cycle tracks wherever feasible.

    - COMMUNICATIONS

Besides existing facilities in different sub-zones,  communication facilities  including Post and Telegraph and Telephone Services,   have been proposed to be graded  in different sub-zones, to meet the requirement of projected population.

    - SECURITY & SAFETY

POLICE:

The main agency looking after security aspect in the city is Delhi Police. At present there are 6 Police Stations and about 10 Police Posts are located in different part of the zone.

Distt. Jail known as Tihar Jail is situated in sub-zone G-13. It is proposed that redevelopment programme of the Jail should be taken up for improving  existing conditions.

Existing CRPF Camp in sub-zone G-14 on Outer ring Road and at Vikas Puri in sub-zone G-16 may be developed as per modified norms, if required.

FIRE:

Presently, about 4 fire stations. and one fire training centre (G-3) are functioning in the zone. One fire station is proposed in the disaster management centre proposed at Sub-Zone G-10

    - DISASTER MANAGEMENT

According to the Indian Seismic Zone map, Delhi is placed in Seismic Zone iv, which means high damage risk zone. Such natural and man made disasters neither can be prevented nor predicted. However, with the technological advancement to some extent mechanism can be developed to mitigate the after effects of the disaster.

Shakur basti Oil Depot in sub zone G-10 has been shifted to a site between Ghevra and Tikri kalan, which will be a major storage site for oil. Part of the land thus vacated is proposed  for Disaster Management Centre and parks for recreational activities. As per MPD 2021 norms Disaster management training centre, hospital, open grounds for practice and relief camps, communication centre, hostels, staff residence, fire station, watch and ward residence (up to 20 sq. m.) is permitted in the Disaster Management Centre.

    - DISTRIBUTIVE FACILITIES

Distributive Facilities include Milk Booths, Fruit & Veg. and LPG Services as basic requirements of day to day life.

1.	Milk Booths are proposed within 1Km. of distance for a population of 	5000.

2.	Fruit & Veg. Booths are proposed in the Traditional Centres, weekly Markets and other areas as per requirements and on mixed land use streets as per norms.

3.	LPG Goodowns:	As against the requirement of 30 sites of LPG Godowns, 47 sites are existing in the zones. Therefore, no additional sites have been proposed. The new depots required for the energy requirement of increased population shall be developed in the NCR region and supply to Delhi should be through pipelines.

2.4.8	OTHER COMMUNITY FACILITIES:

Other community facilities include :

i Development of old age home, care centre for physically, mentally challenged, working Women hostels, adult education center, orphanage and children centers,  ii  Religious facilities,  iii  Anganwari  etc.  These are proposed  to be developed as per norms/requirement.

    - CREMATION GROUND / BURIAL GROUND & CEMETERY

At present there are 6 no. of Such facilities  existing in the zone,  New  sites may be developed subject to availability of land.

    - SOCIO - CULTURAL FACILITIES

Socio cultural facilities includes Banquet Hall, Multi/ purpose Community Hall, Community/recreational Club, Auditorium, Exhibition Ground and Science Center etc. These  are to be developed as per feasibility, requirements and availability of land and as per planning norms and standards prescribed in the Master Plan.

  - TRANSPORTATION

2.5.1.	ROADS

The zone is well connected by roads and major roads of the zone are Rohtak Road/ NH-18, Ring Road, Other Ring Road, Jail Road and Pankha Road etc, NH-18 is passing through the Southern Portion of the zone. Express way is proposed to link Mathura Road with G.T. Road which will also pass through sub-zone G-18. The list of other major roads is  given in Annexure ‘A’.

    - RAIL

The zone is well connected with following railway lines.

      - Rohtak Road Railway Lines.

      - Ring Road Railway lines.

      - Rewari/ Jaipur Railway lines.

The Railway Lines are broad gauge to Rohtak Railway Line and meter gauge to Rewari/ Jaipur Line. It is proposed that proper up-gradation with modern technology and facilities be undertaken for facilitating safe and speedy movement of the passengers.

2.5.3	AIR

The domestic airport at Palam and Indira Gandhi International Airport are located in the zone (sub-zone 15). Both the Airports have been linked to the other part of the city and urban extension through the transport network to facilitate fast movement.  Presently the modernization and improvement/  up gradation of  Airport Authority has been taken by Delhi International Airport Ltd. (DIAL) In order to meet the requirements of   growing air traffic at International standard it is proposed that concerned land owning authority  may    initiate development progamme in a  phased manner. The airport may be connected by an expanded NH-8 as well as MRTS providing railway stations close to the Passengers terminal to shorter journey time. Augmentation of physical   infrastructure facilities as per requirement / norms are to be taken up simultaneously with the concerned local body.

2.5.4	EXISTING / PROPOSED GRADE SEPARATORS AND EXISTING BRIDGES.

It is proposed that major road intersections may be provided with grade separators to ease out the traffic.  The emphasis has also been given to grade separators for pedestrian and cyclist wherever required.  In the zone 6 Nos. of grade separators are existing.

      - Patel Nagar (R.O.B.)

      - Mayapuri R.O.B. (Ring Road)

      - Delhi Cantt. R.O.B.

      - Near Punjabi Bagh (Transport Centre) R.O.B.

      - Mangolpuri R.O.B. (Outer Ring Road)

      - Daya Basti/Zakhira R.U.B.

And five existing bridges on Najafgarh drain are at Khayala, Raghubir Nagar, Basai Darapur, Patel road Extn. and the other one is near East Punjabi Bagh Club, Rohtak Road.

Looking in to the overall future development of National capital Union Territory of Delhi, and specifically Western part of Delhi, it is proposed to have 8 nos. more grade separators to improve the traffic flow.  These are proposed in the following locations :

- Punjabi Bagh (On Outer Ring Road and Rohtak Road)

- Peera Garhi Chowk (on Outer Ring Road and Rohtak Road)

- Lajwanti Garden (On Jail Road and Mayapauri Road)

- Raja Garden (On Ring Road and Najafgarh Road)

- Pankha Road (R.O.B on Delhi Rewari Railway Line)

- Tilak Nagar Grade Separator

- Grade Separators, (Distt. Centre Janakpuri)

- Grade Separators Raja Garden (on Najafgarh Road and Ring Road)

The existing / proposed grade separators and bridges are shown on the plan.  	Other proposals are to be taken up   in connection with proposed transport 	route in the zone.

IMPORTANT INTERSECTIONS :

New road intersections in the zone are to be planned and the details have to be worked out for the individual intersections as per the requirement of the area and adequate pedestrian, cycle tracks may be incorporated for safety of the commuters.

“All  the road owning agencies shall get Road Development Plan prepared for 30 mtr. & above R/W roads shown in MPD-2021 alongwith interchange details of junction/intersection, falling in the alignment showing location of bus stop, metro station & the pedestrian linkage for smooth changeover from one mode to another.”

2.5.5	MRTS CORRIDOR

The Metro Railway system is the most important component of a Rapid Transport System  in the City. Following MRTS Corridors are Proposed in the zone. These are marked on the Zonal development Plan.

1.	Inderprastha to Dwarka Corridor Passing  through Najafgarh Road.

2.	Mundka to  Inderlok passing through Rohtak Road.

3.	Mundka to Patel  Road via Rama Road passing through Ashok  Park Corridor

4.	Linking Patel Road  a n d Rohtak Road.

5.	Central Sectt. Corridor  linking Indira Gandhi International Airport.

Further, the extension o f MRTS may be taken up to other  parts of the  zone    as   per plan and programme.

2.5.6	INFLUENCE ZONE

It is proposed that while preparing local Area plan maximum up to 500 m wide belt on both the sides of the Central Line of MRTS/ major transport Corridor be identified in consultation with  the local body at the stage of preparation of local area plan.. The development of this area  shall be taken up as per feasibility/availability of land and applicability of development control norms shall be as per use zone and norms.

    - BRTS CORRIDOR

BRTS Corridor is proposed in the second phase in zone –‘G’ has incorporated the road network along Ring Road, Moti Ngar, Jaidev Park and Rohtak Road, Punjabi Bagh, Paschim Vihar, Khyalla, Nagloi Sayed, Vikas Puri and Najafgarh Road. It is proposed that the network be laid at the earliest for efficient movement of BRTS, to facilitate daily passengers and decongest traffic movement.

2.5.8	BUS DEPOT

Bus Depots located at Mayapuri, Hari Nagar, Peera garhi, Kesho pur, Nangloi  are sufficient for   the existing population. Refer Table  2.5.9.

2.5.9	BUS DEPOTS & TERMINALS

It is envisaged in the MPD – 2021, that one Bus terminal be provided for 1 lacs population. Hence the required number of Bus Terminals for the entire zone is 18 at present 8 No of Bus terminals are existing in the zone  around keshopur, Uttam Nagar, ITI Hari Nagar, Tagour Garden, Raja Garden, Punjabi Bagh and Moti Nagar.

One Bus Terminal site is proposed in G-10 area in the proposed Community centre.  2 No. are proposed in G-18 Area and  3 No are proposed  in other community  centers as per requirement. New Bus terminals need to be planned , and developed in strategic location to make the use of BRTS and Metro stations

| BUS DEPOTS/TERMINALS ZONE – ‘G’ |  |  |  |  |  | Table 2.5.9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S.No. | Description | Reqd. as per MPD – 2001 | Position in zonal plan |  |  | Net deficiency /Remarks |  |
|  |  |  | Existing |  | Proposed | Total | Surplus |
| 1. | Bus Depots | 5 | 5 | … |  | 5 | Nil |
| 2. | Bus Terminals | 13 | 12 | 6 |  | 18 | Nil |

2.5.10	FUEL STATIONS ;

The main fuel types /being used include Petrol, Diesel and CNG.  These fuels are being made available from Petrol Pumps and CNG stations.  With the advancement of technology some new types of clean fuels may also be used in future.  CNG  stations may be permitted in all use zones except in Regional park/Ridge, developed District Parks, Petrol Pumps are permissible in all use zones except in Recreational use zones.

    - PETROL PUMPS :

24 Sites of Petrol Pumps are existing in the zone.  New sites are proposed mainly to be located in district/community centres and in other use zones as per norms/requirement.

2.5.12	BICYCLE/CYCLE RICKSHAW

Bicycle / Cycle Rickshaw are important modes of travel, particularly with reference to short and medium trip lengths. It is proposed that on all arterial roads fully segregated cycle tracks should be provided with provision for safe parking in part and ride lots. Rickshaw as a non authorized mode of transport should be concisely planned along with  pedestrianisation

2.5.13	RAILWAY STATIONS/YARDS

Presently there are two railway Stations/ yards existing in this zone. These are:

- Kirti Nagar

- Delhi Cantt.

Other 4 railway stations located on the periphery of this zone are as follows :

- Mangol puri

- Shakur Pur

- Daya Basti

- Patel Nagar / Prem Nagar

The railway station shall be taken up for upgradation and modernization by the Railways.

2.5.14	PARKING :

The use of DTC terminals and depots for development of public parking along with parking of DTC buses, private buses, and chartered buses, should be explored and specific projects developed. Based on the site feasibility, parking facility can be created under the open space without disturbing the green area on the surface and surrounding environment. Public parking and under ground parking are to be developed by to local body where ever feasible, as per master plan norms.

2.5.15	PROPOSED SURFACE PARKING

Out of 100 new surface parking proposed in different parts of Delhi , following  sites have been proposed in the zone :

i.	Marble market Maya purl chock to Raja Garden chock (G-2),

ii.	Revital gym to khanna jewelers, Najafgarh, Rajouri garden (G-8),

iii.	Kukreja Hospital on Najafgarh Road, Rajouri Garden   ( G-8 ),

iv.	UTI Bank parking ( G- 16 ),

v.	near BSES Hari Nagar, MTNL telephone exchange Hari Nagar (G-8),

vi.	House tax office (West Zone), Ashok Nagar (G-8),

vii.	Near Gate no. 4, Tihar Jail (G-13),

viii.	Lane outside HIM Motors, Paschim Vihar, Peera garhi Industrial area service road , From Baswin Motors to D-11 Udyog Nagar (G-17).

2.5.16   NIGHT PARKING

The parking of buses during night time be restricted to approve general parking area of District Centres, Community Centres, Local Shopping Centres, Facility Centres, Service Centres etc. and policy be framed by concerned lands deptt.  for enforcement agencies

2.5.17.   MULTI LEVEL PARKING

Multilevel parking facility should preferably be developed in the designated parking space or in the residential, public – semi- public facilities, commercial, transport node, DTC depot, etc. Some Multi-Level Parking Sites are proposed in the zone, like – Rajouri garden, Sant Nagar, Ravi Bagh Market, etc. In addition , the concerned agencies shall finalise other sites from time to time.

2.5.18      OTHER PARKING AREAS

In all new commercial / Business/ Industrial centers, adequate parking on the surface as well as below and above the ground must be provided. The use of basement wherever provided for parking, must be strictly adhered to Master Plan norms. All encroachments on land earmarked for public parking should be removed. However, Public parking areas may be used for second hand car Bazaar on payment basis, only during holidays subject to meeting requirement / condition of the concerned authorities.

2.5.19	PARK AND RIDE

To encourage use of Public transport, such facilities should be provided to reduce traffic congestion on the main roads.

  - ENVIRONMENT

    - The essential aspects to improve the urban liv-in conditions should essentially be seen in terms of urban management and service management.

The Urban Management includes area in terms of noise, in terms of water bodies and land w.r.t. open space and green areas.

The Service Management includes proper management of water supply, sewerage solid waste disposal and transport network. It is proposed that these components should be managed by the local body in such a way that natural resources and natural features are utilized to its optimum capacity. Considering these aspects following are the recommendations:

- Existing drainage basin Such as Najafgarh and other drains as identified on the Plan shall be made self-sustainable and proper water management should be made by integrating water sewerage and drainage system.

- Development of parks and green corridor along major drains Nallahas should incorporate conservation of water and water bodies.

- To increase sub surface soil water through seepage of rain water, proper paving tills should be used in the pavements and parking areas.

- Provision for rain water harvesting be made in all new developments.

- STP/CEPT be developed as per norms.

- For Pollution Control, level of noise pollution, air pollution be monitored periodically and suitable measures like proper land use planning along major transport arteries and industrial bet with green belt buffer be created.

2.6.2	RECREATIONAL USE :

2.6.3	RECREATIONAL  AREA :

The  total  Recreational  area proposed at zonal level works out to about 	595.10 	Hec. 	out of which  519.70 Hec. is under district parks, 	stadium and 	children parks and the remaining 75.4 Hec. at zonal level which 	is 	mainly located in the residential areas.

RECREATIONAL AREA				                                   Table 2.6.3

| Sub-zones | District Park | Area in Hac. Children Park | NHP | Play Ground/ Station | Total |
| --- | --- | --- | --- | --- | --- |
| G-1 | 3.50 |  |  |  | 3.50 |
| G-2 |  |  | 2.50 |  | 2.50 |
| G-3 | 4.00 |  | 0.75 |  | 4.75 |
| G-4 |  |  | 6.00 |  | 6.00 |
| G-8 | 101.70 |  | 17.50 | 8.0 | 127.20 |
| G-9 | 29.50 |  | 1.0 |  | 30.50 |
| G-10 | 72.20 | 4.0 | 3.40 | 25.20 | 104.60 |
| G-13 | 30.10 |  | 12.50 |  | 42.50 |
| G-14 | 36.00 |  |  |  | 36.00 |
| G-16 | 32.80 |  | 7.75 |  | 40.55 |
| G-17 | 158.00 |  | 24.00 | 15.0 | 197.00 |
|  | 467.70 | 4.0 | 75.4 | 48.0 | 595.10 |

The total green area proposed at zonal level works out to about 595.10 Hec., out of which 519.70 Hac. is under district parks, stadium and children parks and the remaining 75.4 Hac. is mainly located in the residential areas. The area will increase after surrender of land from Industries at    as per Supreme Court orders, which is to be utilized  for Recreational use.

2.6.4	DISTRICT PARKS AND SPORTS STADIUM

Besides retaining the Master Plan green areas, a number of sports station and children parks have been proposed, including existing facilities in sub-zone G-10, one children park and a sports stadium have been proposed in sub-zone G-17, one sport stadium has already been built by DDA and the other has been planned near Nangloi mainly for catering the rural population. A sports complex has already been built by DDA in Hari Nagar and it is proposed to develop another one in the south of Maya Puri Indl. Area Ph.-II.

2.6.5	NEIGHBOURHOOD PARKS / PLAY AREAS (NHP)

The large green areas in the form of neighborhood parks works out to apprx. 75.4 Hec. and are distributed in most of the residential colonies of the zone. These calculations for the green areas have not been done for sub-zones falling in the Cantt.Area, Airport and rural sub-zone G-18.

    - POLLUTION  CONTROL

The present industrial belt i.e. Kirti Nagar, Mayapuri and Udyog Nagar at Rohtak Road forms most polluted  ares in the zone. This needs to be developed or redeveloped as per norms stipulated in the Master Plan.

    - MULTI PURPOSE GROUNDS :

A network of the multipurpose grounds, which are basically of parks and open spaces are categorie3sed in MPD-2021 at three levels viz. City Multipurpose Ground.  District Multipurpose Ground and community Multipurpose Ground.

These multipurpose grounds are at the community level shall be upto 2 ha. At the District level of 4 ha. and the city  (Zone) level shall be upto 8 hac.  Already approved parks on 60’.0’ road for holding marriages by the local body/MCD are designated as multi-purpose grounds.

  - URBAN DESIGN.

For assemblage of built up areas and process of arranging these elements functioning and beautifully as a part of urban design are to be made for different use zones within the prescribed guidelines of the Master Plan.

  - CONSERVATION OF BUILT HERITAGE/ARCHAEOLOGICAL 	SITES IN THE ZONE

There are  five nos. of heritage/archaeological sites in the zone.  These are 	falling in sub-zone G-15 as per the statement given below :

- Sarai Sohel	-	Near Indira Gandhi International Airport

- Tomb of Sohel	-	Inside Jet Engine Overhaul Complex, IGI 					Airport

- Sarai Mehram Nagar -	Near Palam Airport Hotel, Mehram Nagar

- Katra 		-	Near Airport Hotel, Mehram Nagar

- Garden of Mehram	North of Mehram Nagar, Near Palam Airport

Khan

Heritage sites are to be maintained as per prescribed norms.

  - PHYSICAL INFRASTRUCTURE

    - WATER SUPPLY

Based on the norms the total requirement of water supply for the entire zone will be around 131.30 MGD (Million Gallons per day) with the break up as follows:

MGD

i	Domestic @ 225 litters per capacity per day			- 77.00

ii	Industrial, Commercial and community 			- 16.80

@ 4500 liters/hect./day)

i.e. @ 17 liters per capita per day

iii	Fire Protection

@ 1% of total demand i.e.

@ 4 litres  per capita per day					  - 2.5

iv	Garden @ 67,0001 es/hect. Day i.e. 35 liters/ capita/day	  - 15.00

v	For floating population and special uses

like emphasis, big hotels etc. @ 52 liters/capita/day		- 20.00

131.30

2.9.2	SEWERAGE

- Assuming that 80% of the total water requirement will go into the sewerage, the total sewerage for the entire zone would be around 85.MGD In the zone a large sewerage treatment plant Keshopur is already existing. The existing capacity of the plant is 32 MGD, which is proposed to be augmented to 170 MGD.

    - DRAINAGE

Najafgarh drain is the major drainage system in the zone. It is proposed the drainage falling in the zone partly be covered and available area of Dilli Hatt be proposed and implemented subject to the approval of the Tech. Committee.  for other drains in the zone, covering of Nallas can be taken up after ascertaining their feasibility.

    - ELECTRICITY

Keeping in view the norms, the total power requirement for the entire zone would be of the order of around 825 MW. The location of existing sub-stations including 220 KV sub-station are indicated on the zonal Plan. Proposed Power Stations in the Urban Extensions to be implemented as per proposed plan.

2.9.5	SOLID WASTE

Keeping in view the norm of 0.68 per kg. Per capita per day , the total solid waste disposal of the entire zone works out to around 2500 Tones per day. A large part of this solid waste could be accommodated in the sanitary land fill sites identified as per MPD-2021.

  - PROPOSED LAND USE

The land use plan for Zonal Dev. Plan, ‘G’ Zone is prepared on the basis of :

- Approved Zonal Development Plan ‘G’ Zone and land use modifications thereupon.

- Information received from different organizations.

- Considering the ground realities which have been, a part of approved Plans.

- Requirement of additional populations as per projections made in MPD-2021.

The proposal of Land use distribution are as follows:

LANDUSE DISTRIBUTION FOR ZONE ‘G’ (WEST DELHI – I )

Table 2.10

| S. NO. | LANDUES ANALYSIS | EXISTING |  | PROPOSED |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Area in Hec. | % age | Area in Hec. | % age |
| 1. | Residential | 3149.56 | 57.72 | 2810.80 | 51.53 |
| 2. | Commercial | 141.29 | 2.58 | 175.00 | 3.20 |
| 3. | Manufacturing | 374.47 | 6.86 | 175.00 | 3.20 |
| 4. | Recreational | 586.80 | 10.75 | 650.88 | 11.92 |
| 5. | Transportation | 593.00 | 10.87 | 475.60 | 8.71 |
| 6. | Utility | 82.70 | 1.53 | 708.00 | 12.99 |
| 7. | Government | 75.81 | 1.39 | 54.56 | 00.99 |
| 8. | Public/ Semi-Public | 325.89 | 5.98 | 406.48 | 7.46 |
| 9. | Vacant | 126.80 | 2.32 |  |  |
|  | Total | 5456.32 | 100.00 | 5456.32 | 100.00 |

DISTRIBUTION OF LANDUSE FOR SUB ZONE G-18

( Proposal of urban Extension )

Table 2.10(1)

| S. NO. | LANDUSE | AREA IN HEC. | % AGE |
| --- | --- | --- | --- |
| 1. | Residential | 283.00 | 44.93 |
| 2. | Commercial | 25.00 | 3.97 |
| 3. | Industrial | 25.00 | 3.97 |
| 4. | Green/ Recreational | 126.00 | 20.00 |
| 5. | Public / Semi Public | 50.00 | 7.93 |
| 6. | Circulation | 63.00 | 10.00 |
| 7. | Utility | 58.00 | 9.20 |
|  | Total | 630.00 | 100.00 |

    - CHANGE OF LANDUSE:

Change of landuse are to be proposed U/s 11 (A) DD Act. The list of the land use modifications in the Zonal Plan is given below (Table 2.11.1)

TABLE  2.11.1

CASES INVOLVING CHANGE OF LANDUSE IN ZONE ‘G’ (Already 	notified)

| S.No. | Sub-Zone | Area Hect. | Change of Land use |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | From | To |  |
| 1. | G-10 | 97.7 Ac. | Commercial (Oil Depot) | PSP Facilities | Part of modifications of MPD-2021 vide notification dt.7.2.07 |

2.11.2	PHASING & IMPLEMENTATION :

For effective implementation, the proposal in the Zonal Dev. Plan is to be monitored periodically.

Considering the time constraints of plan preparation and implementation the requirement of housing units infrastructure requirements and other facilities with reference to holding capacity on different sub-zone are worked out. It is proposed that Redevelopment Plan be prepared  by the local body as per norms.

  - PLAN REVIEW & MONITORING

Most of the areas under planning zone ‘G’ are under built up residential, industrial, recreational etc. it is proposed that plan be implemented by the local body, as per scheduled programme.

DISTRIBUTION OF FACILITIES OF ZONAL PLAN

TABLE – 2.12.1

I

|  | SUB ZONE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. No. | Facilities | 1 | 2 | 3 | 4 | 8 | 9 | 10 | 13 | 14 | 16 | 17 | 18 |
| 1. | Hospital ‘C’ (101 beds to 200 beds) | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 2. | Hospital ‘D’(upto 100 beds) | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 3. | Maternity Home | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 4. | Nursing Home/Polyclinic | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 5. | Multipurpose Community Hall (Provision for marriages small public gathering function eating joint and library gym, etc.) | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 6. | Electric sub station 66 KV | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 7. | Community Centre | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 8. | Informal Bazar | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 9. | Community Park - Park - Multipurpose Park/ground | 1 1 | 1 1 | 1 1 | 1 1 | 2 2 | 1 1 | 1 1 | 2 2 | 2 2 | 1 1 | 2 2 | 1 1 |
| 10. | Bus Terminal | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 11. | Waste water treatment facility | AS PER REQUIREMENT |  |  |  |  |  |  |  |  |  |  |  |
| 12. | Sewerage pumping station | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |
| 13. | Parking space for parking of buses LMVs IPTs, etc. | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 2 | 1 | 2 | 1 |

REQUIREMENT OF FACILITIES AS PER DISTRICT LEVEL IN ZONE ‘G’

TABLE – 2.12.2

II

| Sl. No. | Facilities | Requirement |
| --- | --- | --- |
| 1 | Hospital ‘A’ (501 beds & above) | 3 |
| 2. | Hospital ‘B’ (201 beds to 500 beds) | 6 |
| 3. | Veterinary Hospital for pet animals & birds | 3 |
| 4. | Vocational Training Centre: (ITI/Polytechnic/Vocational Training Institute/Management Institute/ Teacher Training Institute etc.) Research and Development Centre | 3 |
| 5. | Professional College (Engineering) | 3 |
| 6. | Police Station | 6 |
| 7. | Fire Station (5 to 7 Km. radius) | 9 |
| 8. | Care Centre for Physically/Mentally challenged | 3 |
| 9. | Working women-men hostel | 3 |
| 10. | Orphanage/Children’s Centre (one each) | 3 |
| 11. | Electric sub station 220 KV | 3 |
| 12. | District Centre | 3 |
| 13. | Service Market | 3 |
| 14. | Bus Terminal | 3 |
| 15. | Bus Depot | 3 |
| 16. | District Park a. Park Multipurpose ground/Park | 3 |
| 17. | District Sports Centre | 3 |
| 18. | Recreational club | 3 |
| 19. | Cremation ground | 3 |

REQUIREMENT OF FACILITIES AS PER ZONAL/SUB CITY LEVEL IN ZONE ‘G’

TABLE – 2.12.3

III

| S. No. | Facilities | Requirement |
| --- | --- | --- |
| 1 | Medical College (As per Medical Council of India Regulatory Body) | 1 |
| 2. | Telephone Exchange | 1 |
| 3. | Sub City Wholesale Market | 1 |
| 4. | Bus Depot | 2 |
| 5. | Head Post Office & Administration office | 2 |
| 6. | Sewage Pumping Station | 2 |
| 7. | Municipal Office for water and sewerage | 2 |
| 8. | Sewerage Treatment Plant (180) mld) | 2 |
| 9. | City Park a. Park b. Multipurpose | 2 2 |
| 10. | Old Age Home | 4 |
| 11. | Divisional Sports Centre | 2 |
| 12. | Burial ground/Cemetery | 2 |
| 13. | Religious Centre | 2 |
| 14. | Science Centre | 2 |

NOTE :	It is to mention that the basic requirement of these facilities are worked out in totality.  Facilities which in excess are to continue and facilities which are deficient are to be supplemented to maintain a balance distribution.

ANNEXURE   ‘A’

MAJOR ROADS

- Rohtak road / NH-10				60.0 Mtr. R/W

- NH-8 ( Swarna Jayanti marg )		90.0 Mtr. R/W

- Ring Road (Mahatma Gandhi marg )		60.0 Mtr. R/W

- Outer Ring Road (Dr. Hedgewar marg )	60.0 Mtr. R/W

- Jail Road					45.0 Mtr. R/W

- Najafgarh Road / Shivaji marg		60.0 Mtr. R/W

- Pankha Road					45.0 Mtr. R/W

- Patel Road					60.0 Mtr. R/W

- Rama Road					30.0 Mtr. R/W

- Bharthal Road ( Express way  )		100 Mtr. R/W

- Dholi Piao Road				24 Mtr. R/W

- Lal sen Mandir Marg				30 Mtr. R/W

- Vikas puri Road				24 Mtr. R/W

- NS Road					24 Mtr. R/W

Annexure  ‘B’

URBAN VILLAGES FALLING IN ZONE ‘G’ (EXCLUDING CANTONMENT AREA)

| S. No. | Name of Village | Sub-Zone |
| --- | --- | --- |
| 1. | Basai Darapur | G-3 |
| 2. | Tihar | G-8 |
| 3. | Tatar pur | G-9 |
| 4. | Madi pur | G-10 |
| 5. | Nangal Raya | G-13 |
| 6. | Posangi pur | G-13 |
| 7. | Aslat pur khadar | G-13 |
| 8. | Nangli jaleeb | G-13 |
| 9. | Khyala | G-14 |
| 10. | Chaukhandi | G-14 |
| 11. | Shahbad Mohammad pur | G-15 |
| 12. | Kesho pur | G-16 |
| 13. | Bodella | G-16 |
| 14. | Nangloi Sayyad | G-17 |
| 15. | Jwala Heri | G-17 |
| 16. | Peera Garhi | G-17 |
| 17. | Kapas hera | G-18 |
| 18 | Bijwasan | G-18 |
| 19 | Samalka | G-18 |

Annexure ‘C’

Tentative List of unathorised/ regularized colonies in Zone G

| S. No. | Name of Colony | Sub Zone |
| --- | --- | --- |
| 1. | Amaltas Avenue, Samalka Colony | G-18 |
| 2. | Kapas hera Extn Block-A | G-18 |
| 3. | Kapas hera Extn Block-B | G-18 |
| 4. | Kapas hera Extn Block-C | G-18 |
| 5. | Mank Vihar Extn. | G-18 |
| 6. | Samalka Extn. Old Gurgaon | G-18 |
| 7. | Samalka Extn. East | G-18 |
| 8. | Samalka Extn. Kapar Hera Chowk | G-18 |
| 9. | Samalka Extn. Bansi Nagar | G-18 |
| 10. | Village Shahbad Mohammad Pur Extn. Area | G-15 |
| 11. | Vill Basai Dara Pur | G-3 |
| 12. | WZ 106, Area Rajouri Garden | G-8 |

Annexure ‘D’

List of resettlement colonies in Zone –G

| 1. | Khyala Phase-I | G-14 |
| --- | --- | --- |
| 2. | Khyala Phase-II | G-14 |
| 3. | Khyala Phase-III | G-14 |
| 4. | Chaukhandi | G-14 |
| 5. | Raghubir Singh | G-14 |
| 6. | Jwala Puri Phase-I | G-17 |
| 7. | Jwala Puri Phase-II | G-17 |

Rehabilitation Colonies in Zone-G

Annexure ‘E’

| 1. | Ramesh Nagar | G-2 |
| --- | --- | --- |
| 2. | Kirti Nagar | G-2 |
| 3. | Moti Nagar | G-3 |
| 4. | Punjabi Bagh | G-4 |
| 5. | Subhash Nagar | G-8 |
| 6. | Rajouri Garden | G-8 |
| 7. | Tilak Nagar | G-14 |

ANNEXURE  ‘F’

Identification of Commercial Streets in Zone -G (West Zone) as per 2021

(As per the list received from M.C.D)

| S.No. | Name of Road/Street | Stretch of Road | ROW (in mtr) |
| --- | --- | --- | --- |
| 1. | Chowkhandi Road to Subhash Nagar Drain WZ-72, Mukherjee Park to Chowkhandi | NG Road to Chowkhandi Chowk | 9.00 |
| 2. | Road adjacent to Ganesh Nagar M.C. Pry. School | NG Road to Krishna Park Gali No. 8 | 9.00 |
| 3. | Main Market Road Sudershan Park | Shiv Setu to D-Block Sudershan Park | 9.00 |
| 4. | Mall Road Tilak Nagar | Chowkhandi Road to 1-Block Tilak Nagar | 11.00 |
| 5. | Tanga Stand Road, Moti Nagar | B-8 Moti Nagar to B-35 Moti Nagar | 12.00 |
| 6. | Road in front of Block No. 6 & 23 Moti Nagar | 2/24 Moti Nagar to 23/24 Moti Nagar | 12.00 |
| 7. | Dividing Road of Meenakshi Garden & Ajay Enclave | Meenakshi Garden Red Light on NG Road to D-80 Ajay Enclave | 12.00 |
| 8. | Dividing Road 17-Block & 24-Block Tilak Nagar | Post Office Tilak Nagar to Mall Road Tilak Nagar | 12.00 |
| 9. | Dividing Road B-Block Ganesh Nagar & S-4 Block Old Mahavir Nagar | NG Road to Mangla Hospital C-127 | 18.00 |
| 10. | Vedic Marg (Towards Ashok Nagar) | 3-Block Ashok Nagar to Jail Road | 24.00 |
| 11. | Jagdish Sachdeva Marg Moti Nagar | Natraj Cinema to Arya Samaj Mandir | 24.00 |
| 12. | Road in Sudershan Park | Balmiki Mandir to A-Block Gurudwara | 24.00 |
| 13. | Pauri Wala Gurudwara Road Old Mahavir Nagar | S-1/1 Old Mahavir Nagar to Gali No.-17 Krishna Puri | 9.00 |
| 14. | Budhela Road, Krishna Puri | Ganesh Nagar Road near A-Block Ganesh Nagar to Outer Ring Road near L-11 Block Krishna Puri | 9.00 |
| 15. | Ugrsen Market Road to Meenakshi Garden | NG Road Near Tilak Nagar Police Station to 36-Block Ashok Nagar | 9.00 |
| 16. | Old Market Road Tilak Nagar | Tilak Nagar Hospital to 25/15 Tilak Nagar | 11.00 |
| 17. | Ganesh Dass Khatri Marg | NG Road Red Light to Outer Ring Road | 13.50 |
| 18. | NG Road to Sabji Mandi (Prem Nagar to Jail Road) | NG Road To Jail Road | 18.00 |
| 19. | Old Sabji Mandi Road, Moti Nagar | 3/17 Moti Nagar to 1/56 Moti Nagar | 12.00 |
| 20. | Durga Market Road, Moti Nagar | B-8 Moti Nagar to 4/1 Moti Nagar | 12.00 |
| 21. | C-Block Old Market Road Hari Nagar | Hari Nagar Clock Tower to Gandhi Park | 9.00 |
| 22. | A-Block Market Road Hari Nagar | C-Block Market Road to A-Block Market Road | 9.00 |
| 23. | S-Block, Main Road Khyala | Old Khyala Road to new Khyala Road | 9.00 |
| 24. | Post Office Road, Karam Pura | C-1 Karam Pura to B-35 Karam Pura | 12.00 |
| 25. | Main Road H, 1 Block Karam Pura | Shivdass Puri Marg to Milan Cinema Road (Both Sides) New Moti Nagar 1-47 to H-1 Karam Pura | 18.00 |
| 26. | Ambedkar Market Road Raghubir Nagar | PWD Road No.-29 to 12.5 Sq. Yard Road Raghubir Nagar | 24.00 |
| 27. | Asalat Pur Village | Phirni Road | 9.00 |
| 28. | Mandir Waali Gali New Mahavir Nagar on NG Road | NG Road to Budhela Road | 9.00 |
| 29. | Milan Cinema Road | Shivaji Marg to Najafgarh Drain | 9.00 |
| 30. | Main Road Back of H.I.L. Colony | Shivdass Puri Marg to Milan Cinema Road (One Side) New Moti Nagar | 18.00 |
| 31. | Shiv Dass Puri Marg | Moti Nagar Crossing/Flyover to Najafgarh Drain | 60.00 |
| 32. | State Bank Road, A-Block, R/C Khyala | Police Chowki to Maddi Wali Gali | 6.00 |
| 33. | Mangal Bazar Road Vishnu Garden | Aggarwal Sweets to G-6 Vishnu Garden | 6.00 |
| 34. | Maddi Waali Gali Vishnu Garden | 283/368 to Old Khyala Road | 7.50 |
| 35. | NA + NE Block, Vishnu Garden | N-9 to Mandir Gurudwara New Khyala | 7.50 |
| 36. | Bhatta Sahib Gurudwara Road, Vishnu Garden | III-A/102 to A-6, Vishnu Garden | 9.00 |
| 37. | B-1 Sunil Dairy Road Vishnu Garden | B-1/100 to B-1/9 Vishnu Garden | 9.00 |
| 38. | Main Market Road R/C Madipur | PWD Road No. 29 Vishnu Mandir Marg | 13.50 |
| 39. | Dispensary Road R/C Madipur | PWD Road No. 29 to Vishnu Mandir Marg | 13.50 |
| 40. | Som Bazar Road R/C Madipur | H.No. F-1 R/C Madipur Ring Road to PWD Road No. 29 | 13.50 |
| 41. | Chand Nagar Road | Plot No. 10 to Plot No. 399 Chand Nagar | 13.50 |
| 42. | Vishnu Mandir Marg R/C Madipur | Rohtak Road to B-46 New Slum Qtrs. | 18.00 |
| 43. | Old Pankha Road | Pankha Road to NG Road | 24.00 |
| 44. | Phirni Road Basai Dara Pur | WZ-408 to WZ-58A | 9.00 |
| 45. | 100’ Road to RZ-Block, Major Bhupander Singh Nagar (30’ R.O.W.) | 100’ Road to RZ-Block Major | 9.00 |
| 46. | E-Block, R/C Khyala Road (30’ R.O.W.) | DESU Office to Ashram | 9.00 |
| 47. | Phirni Road Basai Dara Pur | Sheela Mandir Chowk to WZ-59A | 13.50 |
| 48. | Chakki Wala Road-1, (20’ R.O.W.) Guru Nanak Nagar | WZ-9/3 to WZ-324 | 6.00 |
| 49. | Chakki Wala Road-II, (20’ R.O.W.) Guru Nanak Nagar | WZ-7/1A to WZ-100A | 6.00 |
| 50. | WZ-494 to NG Road Basai Darapur | WZ-494 to NG Road Basai Darapur | 12.00 |
| 51. | A-Block, R/C Khyala Main Road | Saleti Building to Patel Chowk | 12.00 |
| 52. | Chand Nagar Punjab Market | 80’ Road to BSES office Mukherjee Park | 18.00 |
| 53. | Khyala Cement Store to NW-Chowk (60’ R.O.W.) | Cement Store to NW-Chowk | 18.00 |
| 54. | New Khyala Road 100’ upto T.C. Camp Chowk & 45’ upto NW Chowk | Subhash Nagar More to NW Chowk | 30.00 |
| 55. | Phirni Road Madipur Village | WZ-156/8 to WZ-660 | 6.00 |
| 56. | Outer Ring Road to Wooden Bridge (20’ R.O.W.) | Outer Ring Road to Wooden Bridge | 6.00 |
| 57. | Tyagi Market Basai Darapur | WZ-480A to F-259A | 7.50 |
| 58. | Possengi Pur Village | Phirni Road | 9.00 |
| 59. | Phirni Road Jwala Heri | PWD Road No. 30 to Krishan Chand Complex | 9.00 |
| 60. | Jwala Heri Market Road | 52-B Jwala Heri to Kumar Sweets | 9.00 |
| 61. | Jwala Heri Market Road | 19-B Jwala Heri to Sharbati Extn. Shop No. 14 | 9.00 |
| 62. | Main Khyala Road 45’ Vishnu Garden | B-123 Vishnu Garden to (Aggarwal Sweets), Sham Nagar | 13.50 |
| 63. | Phirni Road Khyala Village (60’ & 45’) | B-1/100 to NW Chowk | 18.00 |
| 64. | 100’ Road – Khyala (One Side | Patel Chowk to Park Hospital | 30.00 |
| 65. | Rohtak Road (One Side) | Zakhira to Outer Ring Road | 60.00 |
| 66. | Janak Puri Marg (Lal Sai Mandir Marg) | C-1 Pankha Road to Hari Nagar Depot Jail Road | 30.00 |
| 67. | Outer Ring Road One Side Towards New Mahavir Nagar | NG Road to PWD Road No. 30 Paschim Vihar | 60.00 |
| 68. | Najafgarh Road | Outer Ring Road to Dwarka More | 60.00 |
| 69. | PWD Road No. 28 Raghubir Nagar | Ring Road to PWD Road No. 29 | 30.00 |
| 70. | Ring Road | Rajdhani College Raja Garden to Rama Road Crossing | 60.00 |

Sources : GNCTD

Mixed Landuse Streets notified  as

Commercial Streets

WEST ZONE

| S. No. | Name of the Roads/ Streets | Stretch of the Road | R.O.W (In Mtr.) |
| --- | --- | --- | --- |
| 1 | Main Road Bali Nagar | H- Block, Bali Nagar Najatgarh Road to B- Block, Bali Nagar | 24 |
| 2 | Satguru Ram Singh Road | Between Kriti Nagar Maya Puri Chowk to Kirti Nagar, Patel Nagar Chowk | 30 |
| 3 | Main Najafgarh Road | Laxman Sylvania, New Moti Nagar Chowk to Outer Ring road crossing i.e. Ganesh nagar | 60 |
| 4 | Jail Road / Station Road | Lajwanti Garden Chowk to nangal Raya | 45 |
| 5 | Subhash Nagar Road | Subhas Nagar to Deen Dayal Upadhyaya Hospital | 24 |
| 6 | Jail Road | Tilak Nagar Chowk to nangal Raya Flyover i.e jail Road | 45 |

Identification of Mixed Use Streets in Zone-G (West Zone)

| S. No. | Name of Road/Street | Stretch of Road | ROW (in mtr.) |
| --- | --- | --- | --- |
| 1. | A-326 to A-566 JJ Colony Chowkhandi | A-326 to 566 JJ Colony Chowkhandi | 9.00 |
| 2. | Dividing Road of 16-Block & 11-Block Tilak Nagar along Tilak Nagar Hospital | Tilak Nagar Market Chowk to 14/5 Tilak Nagar | 11.00 |
| 3. | Dividing Road of Ashok Nagar & Ajay Enclave | Radha Swami Satsang Ghar Ajay Enclave to 36/1 Ashok Nagar | 11.00 |
| 4. | Puri Mandir, Gurudwara Road Moti Nagar | B-137, New Moti Nagar to B-207 New Moti Nagar | 12.00 |
| 5. | Tota Ram Ahuja Marg adjacent to House Tax Office MCD Ashok Nagar | Katyal Mandir Ashok Nagar to 68-Block Ashok Nagar | 12.00 |
| 6. | Old Market Road 16-Block Tilak Nagar | 16/1, Tilak, Nagar near Tilak Nagar Social Centre to 14/22 Tilak Nagar | 12.00 |
| 7. | A-Block Sudershan Park | A-1 to Pumping Station | 12.00 |
| 8. | Gurudwara to A-306, JJ Colony, Chowkhandi | Chand Nagar to A-306, Chowkhandi | 13.50 |
| 9. | B-265 to B-566, JJ Colony, Chowkhandi | B-265 to 566, JJ Colony, Chowkhandi | 13.50 |
| 10. | B-786 to Camp Central School Tagore Garden. | New Khayala Road to Central School | 15.00 |
| 11. | A-306 to A-545, JJ Colony, Chowkhandi | A-306 to A-545, JJ Colony Chowkhandi | 18.00 |
| 12. | 100 ft. Chowkhandi Road. | NG Road to Round About Tilak Nagar | 30.00 |
| 13. | C-Block Market Road Hari Nagar. | Maya Puri Road to Mangal Pandey Marg | 9.00 |
| 14. | Shiv Mandir Road in Shiv Nagar | Jail Road, Shiv Nagar to Janak Puri | 9.00 |
| 15. | Gurudwara Road in Shiv Nagar | Jail Road to H. No. WZ-530 Shiv Nagar | 9.00 |
| 16. | MS-Block Hari Nagar | H. No. 14 to WZ-431 | 9.00 |
| 17. | MS-Block Hari Nagar | H. No. MS-11 to WZ-441 | 9.00 |
| 18. | MS-Block Hari Nagar | H. No. MS-114 to WZ-443 | 9.00 |
| 19. | H-101 to H-1 Sant Nagar Extn. | H-101 Sant Nagar Extn. To Subhash Nagar Drain | 12.00 |
| 20. | Police Chowki Road, Raghubir Nagar | Bus Route Road to R-Block Raghubir Nagar | 12.00 |
| 21. | 17-Block Subhash Nagar | Subhash Nagar Bus Route to 14-Block Road | 13.50 |
| 22. | Main Road A & B Block, New Moti Nagar | 1/5, T.C. Block to C-1 Moti Nagar | 18.00 |
| 23 | Subhash Nagar Bus Route | NG Road to Maya Puri Road | 18.00 |
| 24. | Between C & D Block 25 Sq. yards Raghubir Nagar | Police Chowki Road to 12.5 Sq. Yard Road. | 18.00 |
| 25. | Between D & R Block Raghubir Nagar | Police Chowki Road to 12.5 Sq. Yard Road | 18.00 |
| 26. | N-Block 1-Nagar Raghubir Nagar | Bus Route Road to P-Block Raghubir Nagar | 18.00 |
| 27. | Road Between L & K Block | PWD Raod No. 28 to Ambedkar Market Road Raghubir Nagar | 18. |
| 28. | Road Between A & B Block 25 Sq. Yards Raghubir Nagar | PWD Road No. 28 to Bank Road. | 18.00 |
| 29. | Maharishi Road R/C Madipur | PWD Road No. 29 to Shiv Mandir Road | 18.00 |
| 30. | Cambridge Road (Subhash Nagar) | Subhash Nagar Bus Route to Cambridge School | 24.00 |
| 31. | Maha Kavi Tulsi Dass Marg, Subhash Nagar | Maya Puri Road to Subhash Nagar Bus Route | 24.00 |
| 32. | Bus Route Road Raghubir Nagar | PWD Road No. 28 to 12.5 Sq. Yards Road | 24.00 |
| 33. | 12.5 Sq. Yards Raghubir Nagar | PWD Road No. 29 to R-Block | 24.00 |
| 34. | Shaheed Mangal Pandey Marg. | Road No. 32 to Maya Puri Road | 24.00 |
| 35. | PWD Road No. 29 Raghubir Nagar | PWD Road No. 28 to NG Drain | 30.00 |
| 36. | Central Road Budhela | Bank of India No. 28 to NG Drain | 6.00 |
| 37. | Om Vihar Ram Nagar Road. | N.G. Road to Anil House | 9.00 |
| 38. | Sant Garh Road (30’ R.O.W.) | Tilak Vihar School to WZ-1/1, Sant Garh | 9.00 |
| 39. | 100’ Road to D-Block, M.c. Pry. School, Major Bhpinder Singh Nagar (30’ R.O.W.) | 100’ Road to D-Block, M.C. Pry. School Major Bhpinder Singh Nagar, R.C. Khyla. | 9.00 |
| 40. | Main Road, JJ Colony, Bindapur | A-1, JJ Colony, Bindapur to B-111 | 10.00 |
| 41. | Central School Road Hostel | Emplyment Exchange to Hostel Phimi Road. | 12.00 |
| 42. | Shaheed Chander Marg, Uttam Nagar | Mandir Marg to N.G. Road. | 12.00 |
| 43. | Tanwar Marg Basai Dara Pur | N.G. Road to Mandir | 15.00 |
| 44. | Bhirwa Road, Tagore Garden | N.G. Road to PWD Road No.-28 (D-525 to PWD Road No. 28) One Side | 30.00 |
| 45. | PWD Road No. 30 Paschim Vihar | PWD Road No. 29 to Outer Ring Road. | 30.00 |

IDENTIFICATION OF MIXED USE STREETS IN WEST ZONE

| S. No. | Name of Road/ Street | Stretch of Road | ROW (in mtr) |
| --- | --- | --- | --- |
| 1 | Road between Pkt I & III of Paschim Puri slum & JJ | PWD Road No. 29 to Baba Nivriti Marg | 9.00 |
| 2 | Main road Pkt-III Paschim Puri Slum &JJ | Gate of Pragati Apartment to H. No. 101 Pkt-III Paschim Puri | 9.00 |
| 3 | Post office road, Ramesh Nagar | Water Fall to 2- B Block single Storey, Ramesh Nagar | 13.50 |
| 4 | Naksha Road, Ramesh Nagar | 2-A Block (N.G. Road) to Chacha Neharu Park, Ramesh Nagar | 15.00 |
| 5 | Shaheed Rajguru Marg Vikas Puri | Guru Virjanand Marg to shankar Chowk N.G.-l Vikas Puri | 18.00 |
| 6 | F- Block Main road Vikas Puri | Guru Virjanand Marg to H- Block Vikas Puri | 18.00 |
| 7 | Pankj batra Marg, Ramesh Nagar | Bali Nagar Crossing to Sarvodya Vidyalaya Mansarovar Garden | 18.00 |
| 8 | Fir Station to C- 4F Janak Puri | Janak Puri Marg to C-4F Block Janak Puri | 18.00 |
| 9 | Vidya Marg to C-4A Janak Puri (Shani Bazar Marg) | Vidya Marg to Dabri Road | 18.00 |
| 10 | C-4E market to C-4B Block Janak Puri | C-4E to C-4B Block Janak Puri | 18.00 |
| 11 | Devki Nandan Marg Tagore Garden | PWD Road No 28 to Devki Nandan Chowk | 18.00 |
| 12 | BG-6, Peripheral Road Paschim Vihar | PWD Road No 29 to BG-8,DDA Market | 18.00 |
| 13 | Ramesh Nagar Road | B-1Rajouri Garden to Block 7, Sr. Sec. School, Ramesh Nagar | 18.00 |
| 14 | Major Suresh Gadhok Marg, Rajaur | N.G. Road to Major Sudesh Marg | 18.00 |
| 15 | Mansarovar Garden Road | Ring Road M.S Garden D-1 to D-7 upto MCD Pry. School M.S Garden and E-103 to E-107 Ring Road M.S Garden | 18.00 |
| 16 | Guru Virjanand Marg Vikas Puri | N.G. Road to Outer Ring Road | 24.00 |
| 17 | 80’ Road, sharda Puri U-Ring Road | Ring Road to Ring Road Mansarover grden | 24.00 |
| 18 | Dabri road C-Block Janak Puri | Pankha road C-2D to Janak Puri Marg C-2B | 24.00 |
| 19 | Ram Mandir Marg Janak Puri | Vidya Marg C-5B to Dabri road C-3 | 24.00 |
| 20 | S. S. Mota Singh Marg Janak Puri A- Block | Major Deepak Tyagi Marg to Lal Sai Marg | 24.00 |
| 21 | Major sudesh Kumar Marg Rajouri Garden | Ring Road to NG Road | 24.00 |
| 22 | Road in Rajouri Garden | NG Road to J-19 Rajouri Garden | 24.00 |
| 23 | Vishal Cinema Road, Vishal Enclave | N.G. Road to PWD Road No. 28 | 24.00 |
| 24 | Ch. Balbir Singh Marg | PWD Road No. 30 to Rohtak Road | 24.00 |
| 25 | Vikas Puri Main Road from H-3 Block, to J-Block (80’ R.O.W) | H-3 Block to J- Block | 24.00 |
| 26 | Mall Road between C-5C& C-6B Janak Puri | Vidya Marg Janak Puri to Pankha Road | 24.00 |
| 27 | B-1 Marg (80’) Janak Puri | Najafgarh Road to C-2B Janak Puri Marg | 24.00 |
| 28 | B-2 Marg (80’) Janak Puri | Najafgarh Road to Jank Puri Marg (Bharti College) | 24.00 |

IDENTIFICATION OF PEDESTRIAN SHOPPING STREETS IN WEST ZONE

| S. No | Name of Road/ Street | Stretch of Road | ROW (in mtr) |
| --- | --- | --- | --- |
| 1 | Dal Mill road Uttam Nagar | N. G. Road to Holi Chowk | 4.50 |
| 2. | Lane in Gulab Bagh | N.G. Road to Plot No. 191 | 4.50 |

IDENTIFICATION OF MIXED USE STREETS IN WEST ZONE

| S. No. | Name of Road/ Street | Stretch of Road | ROW (in mtr) |
| --- | --- | --- | --- |
| 1 | Road between Pkt I & III of Paschim Puri slum & JJ | PWD Road No. 29 to Baba Nivriti Marg | 9.00 |
| 2 | Main road Pkt-III Paschim Puri Slum &JJ | Gate of Pragati Apartment to H. No. 101 Pkt-III Paschim Puri | 9.00 |
| 3 | Post office road, Ramesh Nagar | Water Fall to 2- B Block single Storey, Ramesh Nagar | 13.50 |
| 4 | Naksha Road, Ramesh Nagar | 2-A Block (N.G. Road) to Chacha Neharu Park, Ramesh Nagar | 15.00 |
| 5 | Shaheed Rajguru Marg Vikas Puri | Guru Virjanand Marg to shankar Chowk N.G.-l Vikas Puri | 18.00 |
| 6 | F- Block Main road Vikas Puri | Guru Virjanand Marg to H- Block Vikas Puri | 18.00 |
| 7 | Pankj batra Marg, Ramesh Nagar | Bali Nagar Crossing to Sarvodya Vidyalaya Mansarovar Garden | 18.00 |
| 8 | Fir Station to C- 4F Janak Puri | Janak Puri Marg to C-4F Block Janak Puri | 18.00 |
| 9 | Vidya Marg to C-4A Janak Puri (Shani Bazar Marg) | Vidya Marg to Dabri Road | 18.00 |
| 10 | C-4E market to C-4B Block Janak Puri | C-4E to C-4B Block Janak Puri | 18.00 |
| 11 | Devki Nandan Marg Tagore Garden | PWD Road No 28 to Devki Nandan Chowk | 18.00 |
| 12 | BG-6, Peripheral Road Paschim Vihar | PWD Road No 29 to BG-8,DDA Market | 18.00 |
| 13 | Ramesh Nagar Road | B-1Rajouri Garden to Block 7, Sr. Sec. School, Ramesh Nagar | 18.00 |
| 14 | Major Suresh Gadhok Marg, Rajaur | N.G. Road to Major Sudesh Marg | 18.00 |
| 15 | Mansarovar Garden Road | Ring Road M.S Garden D-1 to D-7 upto MCD Pry. School M.S Garden and E-103 to E-107 Ring Road M.S Garden | 18.00 |
| 16 | Guru Virjanand Marg Vikas Puri | N.G. Road to Outer Ring Road | 24.00 |
| 17 | 80’ Road, sharda Puri U-Ring Road | Ring Road to Ring Road Mansarover Grden | 24.00 |
| 18 | Dabri road C-Block Janak Puri | Pankha road C-2D to Janak Puri Marg C-2B | 24.00 |
| 19 | Ram Mandir Marg Janak Puri | Vidya Marg C-5B to Dabri road C-3 | 24.00 |
| 20 | S. S. Mota Singh Marg Janak Puri A- Block | Major Deepak Tyagi Marg to Lal Sai Marg | 24.00 |
| 21 | Major sudesh Kumar Marg Rajouri Garden | Ring Road to NG Road | 24.00 |
| 22 | Road in Rajouri Garden | NG Road to J-19 Rajouri Garden | 24.00 |
| 23 | Vishal Cinema Road, Vishal Enclave | N.G. Road to PWD Road No. 28 | 24.00 |
| 24 | Ch. Balbir Singh Marg | PWD Road No. 30 to Rohtak Road | 24.00 |
| 25 | Vikas Puri Main Road from H-3 Block, to J-Block (80’ R.O.W) | H-3 Block to J- Block | 24.00 |
| 26 | Mall Road between C-5C& C-6B Janak Puri | Vidya Marg Janak Puri to Pankha Road | 24.00 |
| 27 | B-1 Marg (80’) Janak Puri | Najafgarh Road to C-2B Janak Puri Marg | 24.00 |
| 28 | B-2 Marg (80’) Janak Puri | Najafgarh Road to Jank Puri Marg (Bharti College) | 24.00 |