DELHI DEVELOPMENT AUTHORITY

ELECTRICAL DIVISION NO. – 4 / ROHINI, DELHI.

##### SCHEDULE OF WORK

Name of Work 	:  -	M/o completed schemes of Nazul A/c-II under SWZ.

SH. 	 : - 	Repairing of Musical Fountain at District park, Janak Puri.

| S.No. | Description of items | Qty. | Unit | # Rate | ### Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Supplying and replacement of Mechanical Actuator and Gear Box of the Musical Fountain including swing mechanism, testing etc. as required | 5 Nos. | Each |  |  |
| 2 | Overhauling / Repairs of Solenoid Valves of size 40mm/65mm including supplying and replacement of solenoid coil, trigger pins, gaskets, 'O' rings etc. complete as required. | 12 Nos. | Each |  |  |
| 3. | Overhauling / Repairs of Solenoid Valves of size 40mm/65mm including supplying and replacement of trigger pins, gaskets, '0' rings etc. complete as required. | 25 Nos. | Each |  |  |
| 4. | Fabrication and installation of Manifold of existIng Circular Fountain, from pump to pressure tank of size 150 mm dia. x 1000mm long and from pressure tank to fountain including supplying and replacement of' B' Class G.I piping of 80mm dia., 50 mm dia., 40mm dia. of suitable lengths duly epoxy coated on all surfaces, G.I bends, flanges, reducers, 50mm dia. G.M Gate Valves, Heliflex pipe, clamps, etc. as required. | 2 Jobs | P/Job |  |  |

| 5. | Fabrication and installation of Manifold of existing linear Fountain, from pump to main pressure tank of size 250mmdia.x 1600mm long and from pressure tank to fountain frame including supplying and replacement of 150mm dia. and 2.4mtr.long distributor, G.I 'B' Class piping of 150 mm dia., 50mmdia., 40mm dia. of suitable lengths duly epoxy coated on all surfaces G.I Bends, flanges, reducers, 40mm dia. ofG.M Gate Valves, Heliflex pipe etc. as required. | 1 Job | Job |  |  |
| --- | --- | --- | --- | --- | --- |
| 6. | Supplying ands replacement of Brass Nozzles of following sizes /Type etc. as required:- - 6 mm Orfice dia. - 4mm Orfice dia. - Mist Nozzle | 50 Nos. 50 Nos. 30 Nos. | Each Each Each |  |  |
| 7 | Supplying and replacement of following lengths of Bottom Supports of main frame made of 50mm dia. 'B' Class G.I Pipe etc. as required. - 900mm long - 1200mm long | 4 Nos. 3 Nos. | Each Each |  |  |
| 8 | Supplying and replacement of 50 mmx50mm x 6mm size of M.S angle duly galvanized & painted of suitable lengths for water proof lights including making holes and making arrangement for fixing of lights etc. as required. | 24 mtr. | Mtr. |  |  |
| 9 | Supplying and replacement of 32mmx 4mm G.I Strip for swing effect including making holes and arrangement for connector assembly etc. as required. | 37 Mtr. | Mtr. |  |  |
| 10. | Repairing of existing Central Computerized 48 channels one no. Electronic controller, 2 nos. x 250 Watt 'AHUJA' make 'SSA - 250W' model Power Amplifiers, one no. 'SONY' make 'TC- FX-170'model Cassette Tape recorder including supplying and replacement of suitable LC's, relays, contactors etc. as required. | 1 Job | Job |  |  |
| 11. | Supplying and replacement of75 watt (minimum) 'AHUJA' make High Power Column speaker suitable for the existing musical system of the fountain complete with weather proof corrosion resistant fiber reinforced plastic cabinet including connections, testing etc. as required. | 4 Nos. | Each |  |  |
| ## Total |  |  |  |  |  |

###### EXECUTIVE ENGINEER

Electrical Division NO. IV / DDA

Rohini, Delhi – 110085