OFFICE OF THE  EXECUTI VE ENGINEER

SOUTH EASTERN DIVISION NO.4

SARITA VIHAR NEW DELHI:-76

…….

NIQ No.01/EE/SED-4/DDA/08-09

“ NOTICE INVITING QUOTATION  “

Sealed item rate quotations are invited for the work of “D/o land at Jasola. SH: “Making good undulation in road 36 M Row in pocket, 10-B. The quotation shall be received upto 3.00 P.M. on 08.09.08 and shall be opened at 3.30 P.M. on the same day in the office of EE/SED-4/DDA office complex, Near Fly over, Mathura Road,Sarita Vihar,New Delhi-76, in the presence of contractor/their authorized representative.

Schedule of quantity can be have on any working day from the date of issue of this notice to the date of opening between 10.00 A.M. TO 3.00 p.m. from the office of EE/SED-4/DDA.

-sd-

Executive Engineer

South Eastern Division No.-4

No.F.5(6)A/SED-4/DDA/08-09/1069                                              Dtd.01.09.08

SCHEDULE OF QUANTITY

Name of work:-  M/O Various completed scheme under East Zone.

SH:- Repair and maintenance of flat No.71-T in Sector-8 at Jasola.

| Sl. No. | Description of items. | Qty. | Unit. |
| --- | --- | --- | --- |
| 1. | Providing and applying tack coat using hot straight run bitumen of grade 80/100 including heating the bitumen, spraying the bitumen with mechanically operated spray unit fitted on bitumen boiler, cleaning and preparing the existing road surface as per specifications. - On W.B.M. @ 0.75 kg/sqm. - On bituminous surface @ 0.50 kg/sqm. | 400.00 M2 1650.00 M2 | Sqm. Sqm. |
| 2. | Providing and laying bitumen penetration macadam with hard stone aggregate of quality, size and grading as specified, with bitumen of suitable penetration grade including required key aggregate as specified, spreading course aggregate with the help of self propelled/tipped tail mounted aggregate spreader and applying bitumen by a pressure distributor and then spreading key aggregate with the help of aggregate spreader complete including consolidation with road roller of minimum 8 to 10 tonne capacity to achieve specified values of compaction and surface accuracy. a) For reqd. compacted thickness using coarse aggregate of size 50-20mm graded @ 0.60 cum per 0.50 cum. Key aggregate of size 12.5mm graded @ 0.15 cum. per 0.50 cum. with paving asphalt 80/100 @ 0.50kg./0.50 cum. | 20.00 M3 | Cum. |
| 3. | 2.5cm premix carpet surfacing with 2.25 cum and 1.12 cum. of stone chipping of 13.2mm and 11.2mm size respectively per 100 sqm. and 52kg. and 56kg. of hot bitumen per cum of stone chippings of 13.2mm and 11.2mm size respectively including a tack coat with hot straight run bitumen including consolidation with road roller of 6 to 9 tonne capacity etc. complete. ( tack coat to be paid for separately). a) With paving asphalt 80/100 heated and then mixed with solvent at the rate of 70 grams per kg. of asphalt. | 1650.00 M2 | Sqm. |

Sd/-                                          Sd/-                                            Sd/-

JE(T)                                 AE(P)/SED-4                         EE/SED-4/DDA