Schedule of Quantity

N.O.W:- M/o RSKP , Pitampura.

SH:- Repair of glazed tiles and maintenance of water line & sanitary system & whitewashing etc. in swimming pool  at RSKP.

| S.No. | Description of Items | Qty. | Unit | Rate (Rs.) | Amount (Rs.) |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling tile work in floors & roofs laid in cement mortor i/c stacking material within 50meters lead. (a)For thickness of tiles 10mm to 25mm. | 32.10 | Sq.mtr. |  |  |
| 2. | P/F 1st quality ceramic glazed wall tiles confirming toI.S.15622(Thickness to be specified by the manufacturer of approved make in all colors, shades except burgundy, bottle green , black of any size as approved by engineer-in-charge in skirting, risers of steps and dados over 12mm thick bed of cement mortor 1:3(1 cement : 3coarse sand) & jointing with grey cement slurry @3.3kg/sqm. i/c. pointing in white cement mixed with pigment of matching shade complete. | 9.60 | Sq.mtr. |  |  |
| 3. | P/L ceramic glazed floor tiles 300 x 300mm (Thickness to be specified by the manufacturer of 1st quality confirming to IS-15622 of approved make in colours such as White, Ivory, Grey, Fume, Red Brown, laid on 20mm. thick cement mortar 1:4 (1 cement : 4coarse sand i/c. pointing the joints with white cement and matching pigment etc. complete. | 22.50 | Sq.mtr. |  |  |
| 4. | P/F C.P. brass shower rose with 15or 20mm in let. (a) 150 mm diameter | 15nos. | Each |  |  |
| 5. | P/F C.P brass long nose bib cock of approved quality confirming to IS standard & weighing not less than 810gms-15mm nominal bore | 4nos. | Each |  |  |
| 6. | P/F C.P. brass stop cock(concealed) of standard design and of approved make confirming to IS-8931. | 4nos | Each |  |  |
| 7. | P/F solid plastic seat with lid for pidestar type w.c. pan complete. (a) white solid plastic seat with lid. | 10nos. | Each |  |  |
| 8. | P/F PTMT Towel rail complete with brackets fixed to wooden cleats with C.P. brass screws with concealed fitting arrangement of approved quality & color. (a) 600 mm long towel rail with total length to 645mm with 78mm and effective length of 88mm weighting not less 190gms. | 6nos. | Each |  |  |
| 9. | P/F PVC cover i/c. necessary locking arrangement on over head water tank. | 10nos. | Each |  |  |
| 10. | Cleaning of swimming pool and toddler pool and deck area i/c. all material; required for cleaning and labour complete. | 1job | Each job |  |  |
| 11. | Cleaning of filteration plant at terrace, supply and filling of gravel etc. for filtration complete. | 1job | Each job |  |  |
| 12. | Flushing and cleaning of G.I. pipe line coming from terrace(OHT) to change rooms by air jetting & chemical treatment etc. complete. | 1job | Each job |  |  |
| 13. | Removing dry or oil bound distemper water proofing cement paint and the like by scrapping, sand papering and papering the surface smooth i/c. necessary repairs to scratches etc. complete. | 1637 .00 Sq.Mtr. | Sq. Mtr |  |  |
| 14. | Distempering with oil bound washable distemper of approved brand and manufacturer to give an even shade. - Old work( one or more coats) | 1100.00 Sq.Mtr | Sq. Mtr. |  |  |
| 15. | Finishing walls with textured exterior paint of required shade. - New work( Two or more coats applied @3.28 ltr/10 sqm) over an including base coat of water proofing cement paint applied @ 2.20Kg/10sqm. | 537.00 Sq.Mtr | Sq.Mtr |  |  |
| 16. | Painting with synthetic enamel pain of approved brand and manufacturer of required color to give an even shade. - One or more coats on old work | 71.00 Sq. Mtr | Sq.Mtr |  |  |
| 17. | Providing and fixing curtain rods of 125mm thick brass plates with two brass brackets fixed with brass screws and wooden plugs etc. wherever necessary complete. - 25 mm diameter. | 42.30 Mtr. | Mtr. |  |  |
| 18. | Providing & fixing curtain(plastic sheet) of required shade and shape with necessary rings etc. complete as per direction of Secy/RSKP | 89.00 Sq.Mtr | Sq. Mtr |  |  |
| 19. | P/F wash basin with C.I. brackets, 15mm C.P. brass pillar taps 32mm C.P.brass waste of standard pattern i/c. painting of fittings and brackets cutting and making good the walls wherever reqd. - White vitreous china wash basin size 630x450 mm with a pair of 15mm C.P.brass pillar taps. | 1 No. | Each |  |  |
| 20. | P/F P.V.C. waste pipe for sink or wash basin i/c. P.V.C. waste fittings complete. - 40 mm dia | 15 nos. | Each |  |  |
| 21. | P/F on wall face unplasticised Rigid P.V.C. rain water pipe confirming to I.S.13592 type A i/c. jointing with seal ring confirming to I.S. 5382 leaving 10mm gap for thermal expansion single socketed pipes. - 110 mm dia meter | 10.60 Mtr | Mtr. |  |  |
| 22. | P/F P.V.C. low level flushing cistern with manually controlled device( handle lever) confirming to IS.7231, with all fittings and fixtures complete. - 10 ltr. Capacity- white | 10 nos. | Each |  |  |
| 23. | Washed stone grit plaster on exterior walls of height upto 10M. above level in two layers, under layer 12mm cement plaster 1:4 (1 cement : 4 coarse sand) furrowing the under layer with scratching tool, applying cement slurry on the under layer @ 2Kg of cement per square meter, top layer 15mm cement plaster 1:1/2:2 ( 1 cement : ½ coarse sand : 2 stone chipping 10mm nominal size) in panels with groove all around as per approved pattern i/c. scrubbing and washing the top layer with brushes and water to expose the stone chippings, complete as per specification and direction of Engineer-in-charge (Payment for providing grooves shall be made separately) | 4.25 Sq.Mtr | Sq. Mtr. |  |  |
| 24. | Stone work (machine cut edges) for wall lining etc. (veneer work) backing filled with a grout of 12mm thick cement mortar 1:3 (1 cement : 3 coarse san) i/c. pointing in white cement mortar 1:2 ( 1 white cement : 2 stone dust) with an admixture of pigment matching the stone shade : (To be secured to the backing by means of cramps which shall be paid for separately) - 40 mm thick | 5.00 Sq.Mtr. | Sq. Mtr. |  |  |
| Total |  |  |  |  |  |