Schedule of Quantity

NOW:- M/O RSKP, Pitampura.

SH:- Supply & Fixing of M.S. bracket i/c. wiring of light poles of Swimming Pool at RSKP, Pitampura

| S.No. | Description of item | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Supply & making of M.S. bracket of size 100x75 cm rectangular shape made with 100x100x6mm thick M.S square tube i/c cutting in shape and size. Welding and making of provision for fixing of light fittings & cable entry etc. as complete as reqd. | 7 nos. | Each |  |  |
| 2. | Dismantling of existing damaged M.S angle bracket and 400watt metal halide fittings after cutting of nut bolt and fixing of New M.S. rectangular bracket of Metal halide fittings i/c. connection testing etc. as complete as reqd. | 7 nos. | Each |  |  |
| 3. | Supply & drawing of 3x2.5sq.mm copper conductor wire in the existing M.S tubular poles of height 10m long i/c. connection testing as complete as reqd. | 180 mtr. | Mtr. |  |  |
| 4. | Supply & testing of additional 4x24 watt energy efficiency compact street light fitting with acrylic cover deep drawn aluminium housing with 50m dia G.I. bracket and 24watt T-5 lamp 4 nos i/c. connection, testing etc. as complete as reqd. | 7 nos. | Each |  |  |
| 5. | Supply & replacement of 6-32 MPR SPMCB in the existing M.S. box i/c. thimbling, connection, testing etc. as complete as reqd. | 7 nos. | Each |  |  |
| 6. | Supply & replacement of following items in 400watt metal hailed fitting etc. as complete as reqd. - 400 watt metal halide lamp - 400 watt choke for HPIT lamp - 33 MFD Capacitor - Iginitor for 400 watt HPIT lamp | 4 nos. 4 nos. 6 nos. 8 nos. | Each Each Each Each |  |  |
| 7. | Painting of M.s, tubular poles of height 10Mtr. Long with two or more coats of superior quality enamel paint i/c. rubbing, cleaning etc. complete as reqd. | 8 nos. | Each |  |  |