Schedule of Quantity

NOW:- Up-gradation of RSKP, Pitampura

SH:- Repair of Swimming Pool and deck area at RSKP.

| S.No. | Description of Items | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Providing and fixing 1st quality ceramic glazed wall tiles conforming to IS: 15622 (thickness to be specified by the manufacturer), of approved make, in all colours, shades except burgundy, bottle green, black of any size as approved by Engineer-in-Charge, in skirting, risers of steps and dados, over 12 mm thick bed of cement mortar 1:3 (1 cement : 3 coarse sand) and jointing with grey cement slurry @ 3.3kg per sqm, including pointing in white cement mixed with pigment of matching shade complete. | 48.00 sqm | Sqm |  |  |
| 2. | 40 mm thick fine dressed stone flooring over 20 mm (average) thick base of cement mortar 1:5 (1 cement : 5 coarse sand), including pointing with cement mortar 1:2 (1 cement : 2 stone dust) with an admixture of pigment to match the shade of stone. - Red Sand Stone | 9.10 sqm | Sqm |  |  |
| 3. | Filling the joints with neat cement slury mixed with pigment in tiles & stone work as per direction of Engineer-in-charge. | 507.00 sqm | Sqm |  |  |
| 4. | Providing and fixing glazed paper tiles on walls in cement mortar 1:3 (1 cement: 3 coarse sand) as per direction of Engineer-in-charge. | 10.15 sqm | Sqm |  |  |
| 5. | Regrading terracing of mud phaska covered with tiles or brick, in cement by dismantling tiles or bricks, removing mud plaster, preparing the surface of mud phaska to proper slope, relaying mud plaster gobri leaping and tiles or bricks, grouted in cement mortar 1:3 (1 cement : 3 fine sand), including replacing unserviceable tiles or bricks with new ones and disposal of unserviceable material to the dumping ground (the cost of the new tiles or brick excluded) within 50 metres lead. | 280.00 sqm | Sqm |  |  |
| 6. | Supply of F.P.S. brick tiles class degination ‘100’ at site. | 4000 nos. | 1000 nos. |  |  |
| 7. | Flushing of G.I. pipe line coming from O.H.T. to toilet & bath by air pressure pump as per direction of Secy./RSKP | 1 job | Each |  |  |
| 8. | Providing & fixing instruction board in swimming pool made of flex board size 2.50mt x 0.50mt as per direction of Secy./RSKP | 1 no. | Each |  |  |
| 9. | Providing & fixing P.V.C. chair on life guard stand. | 2 nos. | Each |  |  |
| Total |  |  |  |  |  |