SCHEDULE OF QUANTITIES

Name of work:-	D/o Park  Siri Fort Ph-I at Panchsheel (Raisalwala Park)

SH:-	Renovation of boundary wall and making Bajri  Path.

# S.No.		Description of Item							Qty.		Unit	   Rate 	          Amount_____

- Earth work in excavation in foundation trenches or drains (not exceeding

1.5m in width or 10 Sqm. on plan) i/c dressing of sides and ramming  of

bottoms lift upto 1.5m i/c getting out the excavated soil as directed within

a 50 m lead.									136.0     	Cum.															 Cum.

- Providing and fixing precast machine made cement concrete kerb stone

of Size 300x300x100mm with or without cavity on one side of kerb stone

manufactures using   15 grade concrete using GRP moulds by vibro

compactions process so as to achieve shutter finish as per the sample

approved by the Engineer-in-charge i/c fixing with 10mm thick cement

mortar 1:3( 1 cement: 3 coarse sand) at bottoms and 10mm thick for

pointing all roundwith the cement mortar 1:3 (1 cement : 3 fine sand)

i/c drainage opening wherever required.					2354Mtr.	Metre

- Making bajri path including preparation of sub-grade supplying and laying

brick aggregate 40mm nominal size 10cm deep with blinding material

consisting of 12mm red bajri for blinding and 12mm red bajri on top

consolidated with hand roller.							2942.50	Sqm.

Sqm.

- Providing and filling dry brick ballast 50mm nominal size in pits (0.60x0.60

X0.30) as per direction of Engineer-in-charge.				4.00		Cum.

Cum

-: 2:-

## S.No.		Description of Itme							Qty.		Unit	    Rate		 Amount____

- Demolishing cement concrete i/c stacking of serviceable material and

disposal of unserviceable material within 50m lead.				5.60		Cum.

Cum.

- Demolishing brick work i/c stacking of serviceable material and

disposal of unserviceable material within 50m lead.			          76.00		Cum.

Cum.

- Removing mortar from bricks and cleaning bricks i/c stacking with in

a  lead of 50m (stacks of cleaned bricks shall be measured).

a) From brick work in cement mortar.				       12500.00	        1000 Nos.

Nos.

- Brick work with FPS bricks of class designation 75 in foundation and

plinth in cement mortar 1:6 (1 cement : 6 coarse sand).

(Bricks shall be issued free of cost).					          25.00		Cum.

Cum.

- Brick work with cement fly ash FPS bricks of class designation 75 with

mix. 1:3:6 (1 cement : 3 coarse sand : 6 graded fly ash) in foundation &

plinth. (a) In cement mortar 1:6 (1 Cement : 6 coarse sand).		          5.00		Cum.

Cum.

- Providing and laying in position cement concrete of specified grade

Excluding the cost of centring and shuttering – all work upto plinth

Level. (a) 1:2:4 (1cement : 2 coarse sand : 4 graded stone aggregate

20mm nominal size).							          3.50		Cum.

Cum.

-:3:-

______________________________________________________________________________________________________S.No.		Description of Item						Qty.		Unit	Rate		Amount

b) 1:5:10  (1cement : 2 fine sand : 10 graded stone aggregate

40mm nominal size).						27.00		Cum.

Cum.

11.       Centring and shuttering including strutting, propping etc. and

removal of form for foundation, footings, bases of columns etc.	29.00		Sqm.

Sqm.

- Steel work welded in built up section/framed work including

cutting hoisting, fixing in position and applying a priming coat

of approved steel, primer using structural steel etc. as required,

- In grating frames, guard bars, ladders, railings, brackets,

gates & similar works.					          6211.00	             Kg.

Kg.

- Painting with synthetic enamel paint of approved brand and

manufacture to give an even shade.

a) Two or more coats on New Work.				           513.00		 Sqm.

Sqm.

- Stone slate work of required pattern for wall lining upto 10 Mtr.

Height etc. (Verneer work) over 12mm thick bed of cement

Mortar 1:3 (1 Cement : 3 coarse sand) and jointed with grey

Cement slurry @ 3.3Kg./Sqm. as per direction of Engineer-in-

Charge. a) 8 to 10mm thick stone slate in wall lining.		289.00		Sqm.

Sqm.

-:4:-

______________________________________________________________________________________________________

S.No.		Description of Item						Qty.	             Unit	 Rate		Amount_____

- 25mm thick rubbed stone coping over 12mm (average) thick

base joints finished in cement mortar 1:4 ( 1 Cement : 4 Coarse

sand). A)  Red sand stone.					         128.00		Sqm.

Sqm.

- Extra for providing edge moulding in round shape at the top of

edge of 25mlm thick red sand stone slab coping i/c machine

grinding  to edge to give smooth finish.			          641.00		Mtr.

Mtr.

- Removal of mulba by mechanical transport including loading,

unloading stacking as per direction of Engineer-in-charge

(For all leads and lifts).					          57.00		Cum.

Cum.

STIPULATED MATERIAL:-

- Cement  : 12.95 MT @ Rs.4600/- per MT.

EE/SWD-5/DDA.