TERMS AND CONDITIONS

The work comprise of running and operation of pump sets, D.G. set/ submersible pump set/ sewage pump set and general lighting as per the inventory attached.

2.	The contractor shall depute the following staff under the contractor.

a.	Operator – 2 Nos. one in each shift i.e. from 6.00 AM to 2.00 PM and from 2.00 PM to 10PM. for water supply

b.	 Operator – 3 Nos. (1 No. in each shift) from 6 AM to 2 PM,        2 PM to 10 PM and 10 PM to 6 AM for sewage pump set.

c.	Khallasi – 1 No. from 10 PM to 6 PM for clear water supply.

d.	Wireman  Grade-I shall attend the site of work between 9.00 AM to 5 PM at least once in fortnight and as and when required for similarly for sewerage pump.

3.	The contractor shall employ trained and competent staff to the satisfaction of Engineer-in-charge having two years experience on operation of pump/D.G. set.

4.	The contractor shall supply the list of staff along with  experience certificates to be engaged on the installation within 7 days of award of work. Engineer-in-charge shall have the right to reject any staff employed by the contractor.

5.	The staff employed by the contractor shall wear uniform and wear badges displaying their name and designation. The staff shall also be given photo identity cards by the contractor displaying their names, father’s name, address, date of birth, name of the agency with address and telephone no., site for which employed and period of validity. A copy of the identity cards shall be given to the Engineer-in-charge within 15 days of award of work.

6.	The contractor shall employ staff throughout the period of contract including Saturdays, Sundays, gazetted holidays and National holidays.

7.	Following records are to be kept at site of work.

- Printed log book for recording operation.

- Attendance register for staff deployed.

- Complaint/Inspection registers.

Note: The contractor shall get the registers authenticated from Engineer-in-charge before use at site.

8.	The following minor maintenance including preventive maintenance shall be carried out by the contractor. The details of work carried out shall be recorded in the log book:-

- Servicing and greasing of pumps/motors once in 3 months.

- Replacement of rubber packing of valves and worn out nut and bolts etc.

- Replacement of gland dori as when required.

- Replacement of HRC fuses as and when required.

- Replacement of FI Tubes, incandescent lamps and starters.

- Adjustment, alignment/tightening of nuts and bolts as and when required.

- Replacement of burnt out thimbles, wire terminals.

- Top up of battery water.

- Daily cleaning of equipments pump house.

- Replacement of diesel filter, oil filter & air filters of D.G. set.

Note: The material required for carrying out above jobs shall be supplied by the contractor within quoted rates.

9.	The D.G. set shall be operated in case of electrically failure or as and when desired by the Engineer-in-charge. However, P.O.L. shall be supplied by the department.

10.	Co-ordination and follow up action with electrically supply agency shall be done by the contractor in case of failure/fault in supply/fluctuation in voltage.

11.	The contractor shall intimate the Engineer-in-Charge about the major breakdown or failure of electric supply in time.

12.	Watch and ward shall be the responsibility of the contractor. Any loss due to theft or pilferage etc. shall be borne by the contractor.

13.	The payment to the contractor shall be made bi-monthly or at other suitable intervals desired by the contractor but not less than one calendar month.

14.	Proper handing/taking over of all the E&M installations shall be carried out by JE(E) in charge of the work and the contractor in writing immediately after award of work/before start of work.

15.	The Engineer-in-charge, if so desire, may extend upto 2 months or terminate the contract at any time after giving 15 days notice.

16.	If any staff employed by the contractor is found absent on a particular time and date, an amount proportionate to per shift shall be recovered from the contractor’s bill.

17.	All allied accessories/ installations shall be kept in working order and in case any damages/loss is done to the pumping installation  due to the negligence of the operational staff deployed by the contractor, the contractor shall have to make good the damages/loss at his own cost. In case the contractor fails to rectify the damages/loss within reasonable time, the same shall be got rectified/replaced by the department at the risk and cost of the contractor.

18.	The contractor shall keep all the necessary spares and T&P at site required for operation and maintenance to be carried under his contract.

19.	In case of any disobedience or misbehavior by the operational staff of the contractor, the Engineer-in-charge may ask the contractor to dispense with the services of such staff with immediate effect. The contractor shall provide suitable substitute simultaneously.

20.	The rate quoted by the contractor shall be inclusive of all taxes, duties and Govt. levies etc.

21.	Pumping installation shall be painted (excluding control panel) once during the contract period.

22.	Any injury to the operational staff, in case of any mishap, during duty hours will be the liability of contractor.

23.	HPSV lamp/Ballast/Ignitor shall be of Philips/Crompton make only.

Executive Engineer (Elect.)

Elect. Division No…..

NIT No. 11/EE/ED-8/DDA/2008-09			   Estimated cost :- 3,84,655/-

Earnest money :- 7693/-

Date of Sale : 13.06.2008

Date of opening 17.06.2008

SCHEDULE OF WORK

Name of work: M/o Pump set at various colonies under North Zone (Nozul AC-II)

SH – ARMO clear water booster pump set submersible pump set D.G. Set, sewage pump set at 560 SFS houses

cat II in Block C&D at Shalimar Bagh.

| S.No. | Description of item |  | Qty. | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Operation of pumps/D.G. set/submersible and sewage pump set as per the inventory and terms and conditions attached. |  | 1 Job for 12 months |  | Per Job/ Per month |  |
| 2. | Supply and replacement of the following items:- |  |  |  |  |  |
|  | (a) | HPSV lamp 70 Watt | 7 Nos |  | Each |  |
|  | (b) | Copper wound polyster filled ballast 70 Watt | 7 Nos |  | Each |  |
|  | (c) | Itnitor 70 W | 7 Nos. |  | Each |  |