Estimated Cost: 47,337/-

Earnest Money :- 947/-

Date of Sale :- 27.06.2008

Date of opening :- 01.07.2008

DELHI DEVELOPMENT AUTHORITY

ELECTRICAL DIVISION NO. 8

SCHEDULE OF WORK

| Name of work | : | M/O Cycle market at Jhandewalan Ph. I & II |
| --- | --- | --- |
| SH | : | Running, maintenance and operation of storm water pump set and Diesel Engine pump set at Jhandewalan Ph-I for rainy season |

| S.NO. | DESCRIPTION OF ITEMS | QTY. | UNIT | RATE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Operation of storm / sewage pump sets and Diesel Engine pump sets as per terms & conditions attached. | 1 Job For 3 months | Per Job/ P. Months |  |  |
|  |  |  |  | Total |  |

TERMS & CONDITIONS

1. 	The work comprises of running and operation of pump sets, Diesel Engine pump set and general lighting as per the inventory attached.

2. 	The contractor shall depute the following staff under the contract.

a) 	Operator Gd.-I 3 nos. one in each shift i.e. from 6:00 AM to 2:00 PM and from 2:00 PM to 10:00PM. & 10:00PM to 6:00AM.

b) 	Wireman Grade-I shall attend the site of work between 9:00 AM to 5:00 PM at least once in a fortnight and as an when required.

3. 	The contractor shall employ trained and competent staff to the satisfaction of Engineer-in-charge having two years experience on operation of pump sets.

4. 	The contractor shall supply the list of staff along with experience certificates to be engaged on the installation within 7 days of award of work. Engineer-in—charge shall have the right to reject any staff employed by the contractor.

5. 	The staff employed by the contractor shall wear proper uniform and wear badges displaying their name and designation. The staff shall also be given photo identity cards by the contractor displaying their name father’s name, address. Date of birth name of the agency with address and telephone No., site for which employed and period of validity. A photocopy of the identity cards shall be given to the Engineer-in-charge within 15 days of award of work.

6. 	The contractor shall employ staff throughout the period of contract including Saturdays, Gazetted holidays and National Holidays.

7. 	Following records are to be kept at site of work.

a. 	Printed log book for recoding operation.

b. 	Attendance register for staff deployed.

c. 	Complaint / Inspection register/ Maintenance Register

Note: The contractor shall get the registers authenticated from Engineer-in-charge before use at site.

8. 	The following minor maintenance including preventive maintenance shall be carried out by the contractor. The details of work carried out shall be recorded in the log book /  Maintenance Register

a.	 Servicing and greasing of pumps / motors once in 3 months.

b. 	Replacement of rubber packings of valves and worn out nuts and bolts as and when required.

c. 	Replacement of gland dori as and when required.

d. 	Replacement of HRC fuses as and when required

e. 	Replacement of Fl. Tubes, Incandescent lamps and starters for Fl. tube.

f. 	Adjustment/ alignment / tightening of nuts and bolts as and when required.

g. 	Replacement of burnt out thimbles, wire, terminals etc.

h. 	Daily cleaning of equipments / pump house.

Note: The material required for carrying out above jobs shall be supplied by the contractor within quoted rates.

9. 	Diesel Engine pump set shall be operated in case of electricity failure or as and when desired by the Engineer-in-Charge. However P.O.L. shall be supplied by the department.

10. 	Co-ordination and follow up action with electric supply agency shall be done by the contractor in case of failure / fault in supply / fluctuation in voltage etc.

11. 	The contractor shall intimate the Engineer-in-charge about the major breakdown or failure of electric supply in time.

12. 	Watch and ward shall be the responsibility of the contractor. Any loss due to theft or pilferage etc. shall be borne by the contractor.

13. 	The payment to the contractor shall be made bi-monthly or at other suitable intervals desired by the contractor but not less than one calendar month.

14. 	Proper handing / taking over of all the E&M installations shall carried out by JE(E)-in-charge of the work and the contractor in writing immediately after award of work / before start of work.

15. 	Contract shall be for period of 3 months and shall come into force from the date of start of work.  The contract may terminated at any time after giving 15 days notice.

16.	If any staff employed by the contractor is found absent on a particular time and date an amount proportionate to per shift shall be recovered from the contractor’s bill.

17. 	All allied accessories / installations shall be kept in working order and in case any damages I loss is done to the pumping installations due to the negligence of the operational staff deployed by the contractor, the contractor shall have to make good the damages / loss at his own cost. In case the contractor fails to rectify the damages / loss within reasonable time, the same shall be got rectified / replaced by the department at the risk and cost of the contractor.

18. 	The contractor shall keep all the necessary spares and T&P at site required for operation and maintenance to be carried under his contractor.

19. 	In case of any disobedience or misbehavior by the operational stall of the contractor the Engineer-in-charge may ask the contractor to dispense with the services of such staff with immediate effect. The contractor shall provide suitable substitute simultaneously.

20. 	The rates quoted by the contractor shall be inclusive of all taxes, duties and govt. levies etc.

21.	 Pumping installation shall be painted panel once during the contract period.

22. 	Any injury to the operational staff in case of any mishp during, duty hours will be the liability of contractor.

23. 	During the operation period, if any additional pump set installed by the department, the operation of pump shall be done by the agency within quoted rates.

INVENTORY

| PART-A |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (i) | Sewaged Pumps. |  |  |  |  |  |  |
| (a) | 5.00 HP Centrifugal pump set |  |  |  |  |  | 4 Nos. |
| (b) | Control panel comprising of |  |  | 30 Amp. TP&N |  | - | 4 Nos. |
|  |  |  |  | D.O.L. Starter |  | - | 4 Nos. |
|  |  |  |  | Voltmeter |  | - | 4 Nos. |
|  |  |  |  | Amp Meter |  | - | 4 Nos. |
|  |  |  |  | SPP |  | - | 4 Nos. |
| PART-B |  |  |  |  |  |  |  |
| (i) | Diesel Engine Pump set |  |  |  |  |  |  |
| (a) |  | 5 H.P. |  |  |  |  | 1 Nos. |
| (b) |  | 10 H.P. |  |  |  | - | 1 No. |
| PART-C |  |  |  |  |  |  |  |
| (a) | Internal Electrical installation Incandescent Lamp 100 watt |  |  |  |  | - | 8 Nos. |
| (b) | External Electrical |  |  |  |  | - | NIL |