NOTICE NO. 2/EE/ED7/ DDA/0708

NIT.NO.4

DELHI DEVELOPMENT AUTHORITY

ELECTRICAL DIVISION NO.7

Name of work :- M/o office Building under North Zone.

SH : Repairing and Rewinding of Sub. P/Set at Ashok Vihar office complex.

SCHEDULE OF WORK

| S.NO. | DESCRIPTION OF ITEM | QTY | UNIT | RATE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | Repairing and Rewinding of sub .P/set of 5 H.P stage “KSB” make i/c replacement of worn out part i/c necessary reassembling painting, commissioning testing on load etc. as reqd. - Rewinding of 5 H.P sub motor -01Job - Sub Motor parts . a)G. M. Bearing Bush b) water seal c)Motor Base ] d)Thrust Bearing e)Thrust plate f)Sand Guard C)Pump Side a) G.M Impeller b)Guide Bush c) Distance Bush d)G.M. Neck Ring e)NRV Body f) Stage Bowl D)Labour charges for recessembling painting on load etc.as reqd. | 05Nos 02Nos 01Nos 01Nos 01Nos 01Nos 03Nos 05Nos 05Nos 05Nos 01Nos 03Nos 01Job | 1Job | Job |  |
|  | Supplying and replacement of G.I.P pipe mediumclass of 50mm.flangedtype i/c fitting and nut –bolts etc as reqd. | 12mtr. | Mtr. |  |  |
|  | Providing and fixing 32 AMP ,metal clad TP& N suitable fuse unit 415/500 volts, with fuses i/c drilling holes on the board connecting earthing the body etc. reqd. | 1no | Each |  |  |
|  | Supplying and fixing of 3X 1.5 sq.mm. copper cond.sub. cable i/c cable clips jointing and testing etc. as reqd. | 30mtr. | Mtr. |  |  |
|  | Supplying and fixing of AMP meter (0-50) 96 mm.sq. type on the existing panel board i/c testing etc. complete as reqd. | 1No | Each |  |  |
|  | Supplying and fixing of volt meter (0-500v) 96 mm square type on the existing panel board i/c testing etc complete as reqd. | 1No. | Each |  |  |
|  | Supplying and fixing of DOL starter MK -1 type on the existing panel board i/c testing etc. complete as reqd. | 1No | Each |  |  |
|  | Lowering of the sub p/set in the existing bore well up to the reqd. depth along with G.I pipe and its accessories i/c as reqd. | 1Job | Job |  |  |

DELHI DEVELOPMENT AUTHORITY

ELECTRICAL DIVISION NO. 7

ADDITIONAL CONDITIONS & SPECTELCATIONS

- The work as indicated in the schedule of work attached have with i/c any modification /alternations ordered  subsequently shall be carried out as per the specification indicated below:-

  - C.P.W.D specification for electrical works (part-I) Internal 1994 part II External 1994 with correction; slips upto the date of tender.

  - Relevant BIS as modified upto the date wherever BIS code do not exist the British standard should be followed .

- The rate quote by the contractor shall be firm and should be inclusive of all central /state Govt. taxes duties and levies. Concessional sales tax from C/D.

- Nothing extra shall be paid on account of cartage

- No TA& P shall be issued to the agency by the Deptt. For the ------------- of work

- Water and electricity for the execution of work shall be arranged by the firm through their own arrangement and changes if any shall be borne by them

- Testing of the installation shall be done by the contractor within the quoted rates. The arrangement of labour /material required for testing shall be arranged by the firm within the quoted rates.

- Time allowed for carrying out of the work shall be 1(one )month.

- termination of cable in TP&N switch /NCB 1 isolator shall be done by the contraction with in the quoted rates

- The contractor shall stand guarantee for a period of 12 months for material supplied /installed against manufacturing defects / satisfactory performance with the date of completion /handing over which ever is later on a non judicial stamp paper of Rs.50

- The cleanse 10 Cm 10 CC shall not be applicable in this contract. No tube well bore shall be left open s & when sub. /p/sets are taken out for repair /replacement

- No tube well bore shall be left open as & when sub. P/sets are taken out for repair / replacement.

- The worn- out parts of sub. p/set tube replaced shall be f the same make as that of the pumpset.

- The contractor has return the dismentalled materials to deptt. After replacing the defective items at site /rebate for the dismentalled material may be offered (if reqd.)

- The following makes are acceptable for the followings:-

  - DOL Starter –LT /LK Siemens, Bharti Cutler Hammer

  - GI pipe as per IS 1239 – jindal (Hissar ), Surya, TATA

  - PVC sheathed ,PVC insulted flat submersible copper cable –As per IS –Plaza , finolex  Safex.

  - Ammeter & Vol timer as per IS :1248- A.E. /Havell’S

  - PVC insulated copper wire as per

IS:694				SKY tone ,”National /ECKO/Havell’s

  - TP N Main switch –LT/LK, standard ,siemens.

Executive Engineer

Electrical Division

DDA