DELHI DEVELOPMENT AUTHORITY

ELECTRICAL DIVISION NO.7

KESHAV PURAM :DELHI 110035

PRESS TENDER NOTICE NO. 5 /EE/ ED7 /DDA /2007-08

Sealed Item Rate Tender are invited for the for the following works by the Ex.Engineer (Elect) Divn.No. 7 DDA Keshav Puram, contractor of appropriate class of DDA CPWD MES P& T & Railway which shall be opened on the same day at 3.30 P.M at ground  floor vikas minar DDA office I.P Estate New Delhi in the presence of intending purchaser of their authorized representative.

The tender document can be had on any working day from the office of CAJ / North Zone / DDA Ashok vihar sales counter / vikas Minar between the 11.00 A.M. to 3.00PM on payment of amount of mentioned against each work ( non – refundable ) on production of enlistment registration certificate and valid registration with work control cell of sales tax deptt. of GUCID and tax clearance certificate issued by the works contract cell in From XI as well as the amount of earnest money the shape of cash receipt treasury challan / deposit as cell receipt  of schedule bank / fixed deposit receipt of a schedule bank / demand draft of schedule bank  in favour of Rs. A.O / CAJ N. Zone / DDA Sales of tender forms will be stopped four days ( including the date of opening ) before the date fixed for the opening of tender documents.

The manner of submission of tenders opening of the same and entry of contractor’s / their authorized representative to the place of opening shall be as provided in clause 3(a), (b), (c) PWD -6 forming part of tender

| S.No. | Name of wkr. | Nit | Estimated cost Earnest money | Tender cost Time allowed | Dt. Of sale Dt. Of opening |
| --- | --- | --- | --- | --- | --- |
| 1 | M./o park plantation trucks and other equipment in Hort.wing under N.A I I SH: A/R/O of pump set path / park light at various parks in shalimar Bagh AA Block, Thakur Dass wala Bagh B.M / B.Q Block & Ram Bagh | 5 | Rs: 221892.00 Rs:4438.00 | Rs: 500.00 12(twelve month | 1.6.07 5.6.07 |
| 2. | M/o park plantation trucks and other equipment inHort. Wing under N.A I I SH: S/I of submersible sewage pump set with accessories in park / green at Pitampura in Hort. Divn. No. I I DDA | 8 | Rs: 481720.00 Rs;9634.00 | Rs: 500.00 02(two months ) | Rs: 1.6.2007 5.6.2007 |

Executive Engineer(E)

Electrical Division  No. 7

Dated :24/5/.7

PRESS TENDER NOTICE NO 5/EE/ED7/DDA/0708

N.I.T. NO. 5

SCHEDULE OF WORK

NAME OF WORK:-M/o Park plantation trucks & other equipment in Hort. Wing N.A. II (SH:A/R/O Pump set path / park light at various parks in Shalimar Bagh AA Block , AP Block Thakur Dass wala bagh, BM /BQ block & Ram Bagh.

| S.No. | Description of items | Qty | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Annual running & operation of pump set path light park light at various park in Shalimar Bagh as per the inventory and terms & conditions attached | 12 months | p/ month |  |  |

INVENTORY

PART – A

- Booster pumps

- ….. HP Centrifugal pumpset 					NIL Nos.

- ….HP Vacuum pump for priming of above pump set 	NIL Nos

- Control panel comprising of ……..

- Datails of valves ………

PART – B

(i) D.G. Set 			NIL

(a) Capacity ……KVA

(b) Control panel comprising of …….

PART-C

- Submersible pumps

- 5 HP submersible pump set 			9Nos

- Control panel comprising of Starter ammeter , voltmeter MCB Bus shunt capacitor etc. 					9 Nos.

PART – D

- Water level indicator

PART – E

- Internal Electrical installation 			107 Nos.

- External Electrical installation

(compound lighting etc.

TERMS AND CONDITION

- The work comprises of running and operation of pump sets. And general lighting as per the inventory attached .

- The contractor shall depute the following staff under the contract

(a) Operator 	2Nos.

(b) Khallasi	2 No.

(c) Wireman Grade – I shall attend the site of work between 9.00AM to 5.00PM at least once a week and as and when required

- The contractor  shall employ trained and competent staff to the satisfaction of Engineer in charge having two years experience on operation ;of pump

- The contractor shall supplyv the list of staff along with experience certificates to be engaged on theinstallation within 7 days of awardof work. Engineer in charge shall have the right to reject any staff employed by the contractor

- The staff employed by the contractor shallwear proper uniform and wear badges displaying their name and designation the staff shall also be given photo identity cards by the contractor displaying their name , father’s name address date of birth name of the agency with address and telephone No. site for which employed and period of validity. A copy of the identity cards shall be given to the Engineer in charge within 15 days of award of work .

- The contractor shall employ staff throughout the  period of contract including Saturday Sunday Gazetted holidays and National; holidays

- Following records are to be kept at site of work.

  - Printed log bok for recording operation

  - Attendance register for staff deployed

  - Complaint / Inspection register

NOTE:-The contractor shall get the registers authenticated from Engineer in charge use at site.

- The following minor maintenance including preventive maintenance shall be carried out by the contractor. The details of work carried out shall be recorded in the log book.

    - Replacement of rubber packing of valves and worn out nuts and bolts etc.

    - Replacement of HARAC fuses as and when required

    - Adjustment alignment/ tightening of nuts and bolts as and when required.

    - Replacement of burnt out thimbles wire terminals etc.

    - Daily cleaning of equipments

Note :-The material; required for carrying out above jobs shall be supplied by the contractor within quoted rates

- To keep all st. lighting points and all allied installations in working order during the timings / fixed by the Engineer in charge

- Co- ordination and follow up action with electric supply agency shall be done by the contractor in case of failure / fault in supply / fluctuation in voltage.

- The contractor shall intimate the Engineer in charge about the major breakdown or failure of electrical supply in time

- All electric connections shall be checked at least once in a month & set right , if required

- The payment of the contractor shall be made bi- monthly or at other suitable intervals desired by the contractor but not less than one calendar month .

- Proper handing / taking over of all E& M installations shall be carried out by JE(E)  in charge of the work and the contractor in writing immediately after award of work / before start of work.

- The Engineer in charge id so desire may extend the contract up to 2 month or terminate the contract at any time after giving 15 days notice.

- If any staff employed by the contractor is found absent on a particular time and date an amount proportionate to per shift shall be recovered from the contractor’s bill.

- All allied accessories / installation shall be kept in working order and in case any damages / loss is done to the pumping installations due to the negligence of the operational staff deployed by the contractor, the contractor shall have to make good the damages / loss at his own cost. In case the contractor fails to rectify the damages loss within reasonable time the same shall be got rectified / replaced by the department at the risk and cost of the contractor.

- The contractor shall keep all the necessary spares and T& P at site required for operation and maintenance to be carried under his contract .

- In case of any disobedience or misbehavior  by the operational staff of the contractor the Engineer in charge may ask the contractor to dispense with the services  of such staff with immediate effect. The contractor shall provided suitable substitute simultaneously.

- The rates quoted by the contractor shall be inclusive of all taxes duties and Govt. levies etc.

- Any injury to the operational staff in case of any mishap, during  duty hours will be the liability of contractor .

- The clause 10C or 10CC shall not be applicable in this contract .

- The time allowed for carrying out of the work shall be 12 months.

- Testing of earth continuity and earth elect code shall be carried out once in a year by the agency in the presence of authorized representative of the Engineer in charge

EE / ED-7

SCHEDULE OF WORK

NAME OF WORK :- M/o Park plantation trucks & other equipment in Hort. Wing 			under N.A II

SUB HEAD	:- 	S/I of submersible sewage pump set with accessories in park / 			green at Pitampura in HORT. DIVISION NO. II D.D.A

| S.No. | Description of items | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Supplying and installation of submersible sewage pump set (unclog pumps) single phase motor 160 / 230 V 50 HZ oil filled vertical i/c control panel for single phase motor of following rating suitable housing chain in two sides joining of cable connections delivery etc. line connection testing and commissioning etc. complete etc. as reqd. - Solid handing -40 mm - Head 16.5 mtr - Discharge 550lmp - R PM 2900 - cutlet dia 50mm | 6set | Set |  |  |
|  | Supplying and fixing of copper cond submersible cable of size 1 x 3x 2.5mm i/c connection and testing etc. complete as reqd | 1000Mt | Mtr |  |  |
|  | Supplying of 50mm dia PVC suction hose pipe (green ) heavy duty i/c cartage and testing etc. as reqd. | 1200 M | Mtr. |  |  |

MAKES OF MATERIAL AND GOVERNING SPECIFICATIONS

| 1. | Sewage submersible pumpset | - | KSB/ DARKING /MODY |
| --- | --- | --- | --- |
| 2. | PVC sheathed /PVC insulated flat submersible copper cable | IS-694 | Plaza/ Finolex/ safex |
| 3. | PVC Hose pipe | - | Deep Jyoti / dufflex / Kissan(Heavy duty) |

ADDITIONAL CONDITIONS SPECIFICATIONS

- The contractor must get acquinted with proposed sitefir tge wirj abd stydt soecufucatuibns abd cibdutuibs carefully before tendering. The work shall be executed as per programme approved by the Engineer in charge. The progress of the building work. If the site for electrical work is not available in parat or in full for any erason whatsoever, the time for the execution of the electrical work shall be correspondingly extended. No claim for any idle labour of compensation (except as provided under clause 10C /10CC as applicable in the pinted conditions of contract shall be entertained. )

- The contractor shall submit completion certificate as per appendixle of the general  specifications for electrical work (part I ) internal within one month after actual date of completion, railing which an amount @1% of the tendered cost subject tomaximum of Rs. 5000/- shall be deducted form any amount due to the contractor from DDA.

- The sewage submersible pump set of model No. WV15L/40P of the make of  M/s Darling pump set or similar superior makes the control unit/ panel for the above pumpset shall be supplied or installed within the quoted cost.

- Testing of installation:-

  - The entire system shall be tested to the satisfaction of Engineer in charge .

  - Tests shall be performed in the presence of Engineer in charge of his representative

  - The contractor shall provide all labour equipments and material required for performance of the tests:

- The rates quoted by the contractor shall be firm and should be inclusive of all central / state Govt. taxes duties and levies etc.

- concessional sales Tax From C/D shall not be issued by the deptt.

- The installation shall be covered by the conditions that the above installation or any part therefore found defective within the Guarantee period should be replaced by the contractor free of charge and reasonable time. The guarantee shall cover the following s:-

- Quality strength and performance of material and used.

- Safe electrical and mechanical strength of all parts of the equipments  under all specified conditions of operations satisfactory performance.

- Safe performance figure and other values as specified in the schedule of Guarantee  / Technical particulars.

- Prompt service during guarantee period

- attending to consequential damages in the equipments supplied and installed by the tender.

- Guarantee period of 12 months shall be counted from the date of handing over of installation to the deptt. against manufacturing defects on a non judicial stamp paper of Rs. 50/- only

- Contractor shall provided the manufacturer test certificate in respect of all

major item.

- the contractor shall be fully responsible for watch and word of the material 	brought /work executed at site till the same is complete and taken over by the

deptt.

SPECIFICATIONS:-

The work as indicted in the schedule of quantities attached here with including any modifications additions alterations ordered subsequently shall be carried out as per specifications given below:-

10

- CPWD specifications for Electrical works at Delhi Part I (internal )2005 and Part II (External ) 1994 with amendments upto the date of tenders

- Indian Electricity Rules 1956as amended upto date.

- A table indicating the makes governing specifications and other details in  respect of some of the important materials to be used in the work is attached. These specifications shall have preference over those indicated in 14 (i) above.

- Only material bearing IS/BIS certifications mark shall be used in the work. Where articles of different designs / make bearing ISI /BIS certification  mark are available the decision of Engineer in charge about the design / make to be used in the work shall be final and binding on the contractor.

- Where material bearing ISI / BIS certification marks are not available material conforming to relevant BIS / ISI shall be used with prior approval of Engineer in charge.

- If the specifications of any item are not available then the decision of the Engineer in charge regarding quality shall be final and binding on the contractor.

11. In case of any discrepancy in the description of any item given with schedule of quantities and the specifications the former shall prevail if the specifications of any item are not available the decision of Engineer in charge shall be final

12  After the award of work the contractor shall get all the material approved from the Engineer in charge before use in the work. The contractor shall also produce test certificates for conformity with relevant IS : specification if required by the Engineer in charge.

ADDITIONAL SPECIFICATIONS

13 Additional specifications are indicated in the following clauses. These additional specifications given below shall have precedence over those indicted in para  above in case of deviations between the two

14  The metal boxes to be used in the work shall be made of M.S sheet and galvanized .

15 Connectors to be used  anywhere in the installations shall be of porcelain type.

16 No tool and plants will be issued for execution of the work or for testing of the installation. These shall be arranged by the contractor within the quoted cost.

17 The contract would have to make his own arrangement for cartage and transportation of the materials and nothing extra would be payable on this account.

18  The following information to be furnished by the tender

- Type make model and HP of the pump set offered

- Discharge in liters per minutes and head  in meters

- Revolution per minutes

- Time allowed for carrying of work within two months.

EXECUTIVE ENGG.

ELECTRICAL DIVISION - 7