DELHI DEVELOPMENT AUTHORITY

RASHTRIYA SWABHIMAAN KHEL PARISAR

PITAM PURA, DELHI-110088

NOTICE INVITING QUOTATION  No.32 /2008-09

Sealed quotations are invited by the Secretary, RSKP, on behalf of the Delhi Development Authority (Sports Wing) for the following work from the eligible contractors / agencies. The quotations should reach in the office of the undersigned on or before 27.11.08 upto 3.00 PM and the same shall be opened on the same day at 3:30 PM in the office of the Secretary of the Complex in the presence of quotationers or their authorized representatives. Sche

Name of work: M/O RSKP PitamPura

SH:. Const. of Water Cooler Shade near Cricket Ground.

| S.No. | Item |  | Qty. | Unit |  | Rate |  | Amount |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Earth work in excavation by mechanical means (Hydraulic excavator) manual means in foundation trenches or drains (not exceeding 1.5 m in width of 10 Sqm on plan) i/c dressing of sides & ramming of bottoms lift upto 1.5m i/c getting out the excavated soil and disposal of surplus soil as directed within a lead of 50 metres. a) all kinds of soil. |  | 4.20 | Cum |  |  |  |  |  |  |  |
| 2. | Providing and laying in position cement concrete of specified grade e/x the cost of centering and shuttering All work upto plinth level 1:5:10 (1 Cement : 5 Coarse Sand : 10 Graded stone agg. 40mm nominal size) |  | 1.00 | Cum |  |  |  |  |  |  |  |
| 3. | P/L cement concrete in retaining walls, return walls, walls (any thickness) i/c attached pilasters, columns piers, abutement, pillars, post, struts, butteresses, string or lacing courses, parapets, coping, bed blocks, anchor blocks, plain window sills, fillets etc. upto floor five level e/x the cost of centring, shuttering & finishing. 1:2:4 (1 cement:2 C.sand: 4 graded stone agg.20mm N.S. |  | 0.25 | Cum |  |  |  |  |  |  |  |
| 4. | RCC work in beams, suspended floors, roofs having slope upto 15” landings, balconies, shelves, chajja, lintel beams, plain window sills, staircase & spiral staircase upto floor five level e/x the cost of centring, shuttering, finishing and reinforcement with 1:2:4 (1 cement : 2 C.Sand : 4 greaded stone agg. 20mm N.S. |  | 0.50 | Cum |  |  |  |  |  |  |  |
| 5. | Centring and shuttering, i/c strutting propping etc. and removal of form for suspended floors, roofs, landings, balconies & access platform. |  | 3.15 | Sqm |  |  |  |  |  |  |  |
| 6. | Reinforcement for RCC work i/c straightening, cutting, bending, placing in position and binding all complete Thermo-mechanically treated bars. |  | 75.00 | Kg. |  |  |  |  |  |  |  |
| 7. | Brick work with FPS bricks of class designation 75 in foundation and plinth in Cement Mortar 1:6 ( 1 cement: 6 coarse sand) |  | 3.50 | Cum |  |  |  |  |  |  |  |
| 8. | Brick work with FPS bricks of class designation 75 in superstructure above plinth level upto floor V level in all shapes and size in Cement Mortar 1:6 (1 cement: 6 Coarse sand) |  | 3.00 | Cum |  |  |  |  |  |  |  |
| 9. | P/F T-iron frames for doors, windows & vantilaters of mild steel Tee Sections, joints mitred and welded with 15X3mm lugs 10 cm long embedded in cement concrete blocks 15X10X10Cm of 1:3:6(1 cement:3 C.Sand: 6 graded stone agg. 20mm N.S.) with wooden plug and screw or rawl plug and screws or with dash fastner or with fixing clips or with bolts & nuts as required i/c fixing of necessary butt hinges and screws & applying a primary coat of approved steel primer | 85.00 |  |  | Kg |  |  |  |  |  |  |
| 10. | Kota Stone slab flooring over 20m (average) thick base laid over and jointed with grey cement slurry mixed with pigment to match the shade of slab i/c rubbing and polishing complete with base of cement mortar 1:4 (1 cement : 4 coarse sand) 25 mm thick | 8.00 |  |  | Sqm |  |  |  |  |  |  |
| 11. | P/F 1st quality ceramic glazed wall tiles confirming to IS:15622 (thickness to be specified by the manufacturer) of approved make in all colours shade except burgundy, battle green, block of any size as approved by Engineer-in-charge in skirting, risers of steps and dado over 12mm thick bed of cement mortar 1:3 (1 cement : 3 C.Sand) and jointing with grey cement slurry @ 3.30 Kg/Sqm i/c painting in white cement mixed with pigment of matching shade complete. | 11.50 |  |  | Sqm |  |  |  |  |  |  |
| 12. | 12mm cement plaster of mix 1:6 (1 cement: 6 Fine sand) | 15.50 |  |  | Sqm |  |  |  |  |  |  |
| 13. | 15 mm cement plaster on the rough side of single or half brick wall of mix 1:6 (1 cement: 6 Fine sand) | 1.50 |  |  | Sqm. |  |  |  |  |  |  |
| 14. | 6 mm cement plaster of mix 1:3 (1cement:6 F.Sand) | 1.00 |  |  | Sqm |  |  |  |  |  |  |
| 15. | Finishing walls with water proofing cement paint of required shade New Work (Two or more coats applied @ 3.84 Kg/10Sqm. | 18.00 |  |  | Sqm |  |  |  |  |  |  |
| 16. | Painting with synthetic enamel paint of approved brand and manufacture to give an even shade. Two or more coats on new work. | 3.00 |  |  | Sqm |  |  |  |  |  |  |

Terms and Conditions –

- The works shall be carried out as per CPWD specifications amended upto date & to the entire satisfaction of Secy. / J.E.(Civil) / RSKP.

- All material used at Site shall be got approved before fixing at site from Secretary (RSKP)/JE(Civil)

- Prices quoted shall be firm and inclusive of all Taxes.

- The payment will be made after entire satisfaction of J.E (Civil) / Secretary / RSKP, DDA

- No T & P will be issued by the Department.

- No advance payment shall be made to the Agency.

- Nothing extra will be paid to the Agency

- No minor labour shall be engaged at site

- Director (Sports) reserves the right to accept or reject any bid/quotation without assigning any reason

- Conditional quotation will not be accepted

- All statutory deductions like Income Tax, Service Tax, or any other such taxes at the rates applicable at the time of payment shall be made from the Agency.

- The tenderer / quotationer should be registered with the Department of Trade & Taxes, Govt. of NCT Delhi and supposed to furnish a copy of VAT registration with the quotation. The quotationer / tenderer should also submit an undertaking or proof of filling the return/proof of last payment made to the Deptt of Trade and Taxes. The quotationer / tenderer will be solely responsible for making any outstanding payment payable to the Deptt of Trade & Taxes and DDA shall not be responsible

- If there are two or more agencies quoting the same rate preference would be given to the agency which is already serving the subject to its satisfactory performance.

If even then there is a tie between two or more such agencies then the work

would be awarded to the agency which has the highest gross annual turnover for

the last three years.

- Debarred agency / or individual shall not be permitted to participate in the tendering process.

- Any individual who has been debarred is not permitted to take part in any negotiations or represent any agency if he holds a power of attorney on the agencies behalf. Any agency engaging debarred persons is also liable to debarring

- The experience of agencies should be taken into account rather than individuals incorporated with the agency, which does not include proprietorship.

- Time allowed is one month for completion of work.

18. Validity of quotations is 60 days.

Sd/-

(S.C. Gupta)

Secretary / RSKP

No. F   5(61)/2008 /RSKP/DDA/1011		         Dated:20.11.08

Copy to:

- Director(System)-through E-mail

- Director(Sports), DDA

- Sr. AO(Sports)/DDA

- All Secretaries of the DDA Sports Complexes i/c Secretary(Coordn.).

- The Secretary, DDA Contractor’s Association, Vikas Kutir, New Delhi.

- The Secretary, DDA Contractor’s Welfare Association, E-18, Vikas Kutir, New Delhi.

- AAO/RSKP, DDA

- J.E.(Civil) ,RSKP, DDA

- Notice Board

Sd/-

Secretary/RSKP