P-123

SCHEDULE OF QUANTITIES

Name of work	: Dev.of Service Centre Sector-9, Dwarka, Ph.I.

SH:	  Repair of road in Service Centre behind R.D.Rajpal Public School.

| S.No. | Description of item. | Qty. | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Providing and applying tack coat using hot straight run bitumen of grade 60/70 including heating the bitumen, spraying the bitumen with mechanically operated spray unit fitted on bitumen boiler, cleaning and preparing the existing road surface as per specifications. a) On bituminous surface @ 0.50 kg./sqm. | 3425.00 Sqm. | Sqm. |  |  |
| 2. | 2.5cm premix carpet surfacing with 2.25cu.m and 1.12 cu.m. of stone chippings of 13.2mm and 11.2mm size respectively per 100 sq.m. and 52 kg. and 56 kg. of hot bitumen per cu.m. of stone chippings of 13.2mm and 11.2mm size respectively including a tack coat with hot straight run bitumen including consolidation with road roller of 6 to 9 tonne capacaity etc. complete. (Tack coat to be paid for seperately) a) With paving Asphalt 60/70 with no solvent. | 3425.00 Sqm. | Sqm. |  |  |
| 3. | Filling pot-holes and patch repair with bituminous concrete using crushed stone aggregate 20mm nominal size and 13.2mm nominal size as specified in CPWD specification and bitumen of grade 60/70 @ 56 kg. bitumen per cum of total mix including hot mixing of stone aggregate and bitumen in hot mix plant transporting mixed material and also including removal of all failed material trimming of completed excavation to provide firm vertical faces cleaning of surfaces and back filling with mixed material bituminous concrete over a tack coat and compacting with road roller of 8 to 10 tonne capacity, trimming and finishing the surface to form a smooth continous surface as per the direction of Engineer in-charge. (tack coat shall be paid separately). | 24.00 Cum | Cum. |  |  |
| 4. | Surface dressing of the ground including removing vegetation and in-equalities not exceeding 15cm deep and disposal of rubbish, lead upto 50 M and lift upto 1.5m. a) All kinds of soil. | 3607.00 Sqm. | Sqm. |  |  |
| 5. | Removal of malba/building rubbish by mechanical transport including loading & unloading. a) Lead upto 1.00 Km. | 541.00 Cum | Cum |  |  |

J.E.(T)			A.E.(P)/WD-10		E.E./WD-10/DDA.