SCHEDULE OF QUANTITY

NAME OF WORK :        C/o Commonwealth Games Village for common wealth games-2010 Off Road NH-24, Near Akhshardham.

SUB HEAD           : 	Contingencies upgradation of office complex of E.E./C.G.D.-2/D.D.A. at Seed Bed Park, Shakarpur .

| S.No. | Description of Item | Qty. | Unit | Rate | Amount | Remark |
| --- | --- | --- | --- | --- | --- | --- |
|  | Earth work in rough excavation, banking excavated earth in layers not exceeding 20 cm in depth, breaking clods watering, rolling each layer with 1/2 tonne roller or steel rammers, and rolling every 3rd and top-most layer with power roller of minimum 8 tonnes and dressing up in embankments of roads, floods banks, marginal banks and guide banks or filling up ground depressions, lead up to 50 m and lift up to 1.5 m: - All kinds of soil | 6.50 | Cum |  |  |  |
|  | Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering - All work up to plinth level: a) 1:4:8 (1 cement : 4 fine sand : 8 graded stone aggregate 40 mm nominal size): b)1:2:4:( 1cement :2 coarse sand :4 graded stone aggregate | 2.00 1.00 | Cum cum |  |  |  |
|  | Providing and laying damp- proof course 40mm thick with cement concrete 1:2:4 (1cement : 2 coarse sand : 4 graded stone aggregate 12.5mm nominal size). | 4.14 | Sqm |  |  |  |
|  | Appling a coat of residual petroleum bitumen of penetration 80/100 of approved quality using 1.7kg per square metre on damp proof course after cleaning the surface with brushes and finally with a piece of cloth lightly soaked in kerosene oil. | 4.14 | Sqm |  |  |  |
|  | Brick work with F.P.S. bricks of class designation 75 in foundation and plinth in: cement mortar 1:6 (1 cement : 6 coarse sand) | 2.80 | Cum |  |  |  |
|  | Brick work in super structure with cement mortar 1:6 (1 cement : 6 coarse sand) | 12.50 | Cum |  |  |  |
|  | Removing dry or oil bound distemper ,water proofing cement paint and the like by scrapping , sand papering and preparing the surface smooth including necessary repair to scratches etc . complete. | 475.00 | Sqm |  |  |  |
|  | Making plinth protection 50mm thick of cement concrete 1:3:6 (1 cement : 3 coarse sand : 6 graded stone aggregate 20mm nominal size) over 75mm bed by dry brick ballast 40mm nominal size well rammed and consolidated and grouted with sand including finishing the top smooth. | 15.50 | Sqm |  |  |  |
|  | Reinforced cement concrete work in beams, suspended floors, roofs having slop up to 15 landings, balconies, shelves, chajjas, lintels, bands, plain window sills, staircases and spiral stair cases up to floor five level excluding the cost of centering, shuttering, finishing and reinforcement with 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20mm nominal size) | 0.60 | Cum |  |  |  |
|  | Reinforcement for R.C.C works including straightening, cutting, bending, placing in position and binding all complete. T.M.T. bars. | 60.00 | Kilogram |  |  |  |
|  | Centering and shuttering including strutting propping etc. and removal of form for: - Lintels, beams, plinth beams, girders, bressummers and cantilevers. | 7.00 | Sqm |  |  |  |
|  | 12 mm cement plaster of mix 1:6 (1 cement :6 fine sand) | 106.21 | Sqm |  |  |  |
|  | 15 mm cement plaster of mix 1:6 (1 cement :6 fine sand) | 97.00 | Sqm |  |  |  |
|  | Demolishing C.C. work manually/by mechanical means including stacking of steel bars and disposal of unserviceable material within 50 meters lead as per direction of Engineer-in-charge. 1:3:6 or richer mix | 1.62 | Cum |  |  |  |
|  | Demolishing brick work manually/by mechanical means including stacking of serviceable material and disposal of unserviceable material within 50 meters lead as per direction of Engineer-in-charge. a) In cement mortar | 14.43 | Cum |  |  |  |
|  | Removing mortar from bricks and cleaning bricks including stacking within a lead of 50 m (stacks of cleaned bricks shall be measured): - From brick work in cement mortar | 3500 | 1000 Nos |  |  |  |
|  | Taking out doors, windows and clerestory window shutters (steel or wood) including stacking within 50 meters lead. - Of area 3 sq. metres | 10 | Each |  |  |  |
|  | Distempering with oil bound washable distemper of approved brand and manufacture (one or more coats) and of required shade on old work ( one or more coat ). | 26.00 | Sqm |  |  |  |
|  | Providing and fixing aluminium sliding door bolts ISI marked anodized ( anodic coating not less than AC 10 as per IS: 1868) transport or dyed to required colour or shade with necessary screws etc. complete. 300X16mm | 16.00 | Each |  |  |  |
|  | Providing and fixing aluminium tower bolts ISI marked anodized (anodic coating not less than grade A C 10 as per IS: 1868) transparent or dyed to required colour or shade with necessary screws etc . Complete. 300 X 10 mm | 26.00 | Each |  |  |  |
|  | Providing and fixing aluminium handles ISI marked anodized (anodic coating not less than grade AC 10 as per IS: 1968) transparent or dyed to required colour or shade with necessary screws etc. complete. 125 mm. | 33.00 | Each |  |  |  |
|  | Providing and fixing aluminium hanging floor door stopper ISI marked anodized (anodic coating not less than grade AC 10 as per IS: 1968) transparent or dyed to required colour or shade with necessary screws etc. complete. - Single rubber stopper | 12.00 | Each |  |  |  |
|  | Steel work welded in built up sections / framed work including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer using structural steel etc. as required. - Hot finished welded type tubes. | 300.00 | Kilogram |  |  |  |
|  | Steel work welded in built up sections / framed work including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer using structural steel etc. as required. In gratings, frames, guard bar, ladder, brackets, gates and similar works. | 50.00 | Kilogram |  |  |  |
|  | Kota stone slab flooring over 20mm (average) thick base laid over and jointed with grey cement slurry mixed with pigment to match the shade of the slab including rubbing and polishing complete with base if cement mortar 1:4 (1 cement : 4 coarse sand): - 25mm thick | 27.60 | Sqm |  |  |  |
|  | Providing and laying polished vitrified floor tiles in different sizes (thickness to be specified by the manufacture ) with water absorption’s less than 0.08% and conforming to IS : 15622 of approved make in all colours and shades , laid on 20mm thick cement mortar 1:4 (1 cement : 4 coarse sand): including grouting the joints with white cement and matching pigments etc., complete. - Size of Tile 60X60 cm | 195.00 | Sqm |  |  |  |
|  | Deduct for not using 20mm thick cement mortar 1:4 (1 cement: 4 coarse sand): bedding in laying of floor tiles. | 195.00 | Sqm |  |  |  |
|  | Fixing glazed/ ceramic/ vitrified floor tiles with cement based high polymer modified quick-set tile adhesive (Water based ) conforming to IS: 15477, using 5kg . adhesive per sqm of tile area, in average 3mm thickness. | 195.00 | Sqm |  |  |  |
|  | Providing and fixing on wall face unplasticised Rigid PVC rain water pipes conforming to IS: 13592 Type A including jointing with seal ring conforming to IS:5382 leaving 10mm gap for thermal expansion.(i) single socketed pipes 110mm diameter | 18.00 | Metre |  |  |  |
|  | Providing and fixing on wall face unplasticised –PVC moulded fittings/ accessories for unplasticised Rigid PVC rain water pipes conforming to IS: 13592 Type A including jointing with seal ring conforming to IS:5382 leaving 10mm gap for thermal expansion. - 110 mm Shoe (Bend 87.5 ) | 6.00 | Each |  |  |  |
|  | Providing and fixing on wall face unplasticised –PVC moulded fittings/ accessories for unplasticised Rigid PVC rain water pipes conforming to IS: 13592 Type A including jointing with seal ring conforming to IS:5382 leaving 10mm gap for thermal expansion. - 110 mm bend | 6.00 | Each |  |  |  |
|  | Providing and fixing False Ceiling Tiles of approved materials of size 595x595 mm in true horizontal level suspended on inter locking metal grid of hot dipped galvanized steel sections (galvanized @ 170 gsm/sqm.) consisting of main “T” runner suitably spaced at joints to get required length and of size 24x38mm made from 0.30mm thick (minimum) sheet spaced at 1200mm center to center and cross “T” of size 24x25mm made of 0.30mm thick (minimum) sheet, 1200mm long spaced between main “T” at 600mm center to center to form a grid of 1200x600mm and secondary cross “T” of lengh 600mm and size 24x25mm made of 0.30 mm thick (minimum) sheet to be interlocked at middle of the 1200x600mm panel to from of 600x600mm and laying false ceiling tiles of approved texture in the grid including, wherever required , cutting/making opening etc. Main “T” runners to be suspended from ceiling using GI slotted cleats fixed to ceiling with 6 mm dia and 50mm long dash fasteners, 4mm GI adjustable rods with galvanized level clips spaced at 1200m center to center along main T, bottom exposed width of 24mm of all T-sections shall be pre-painted with polyester paint, all complete at all heights as per specifications drawings and as directed by Engineer-in-Charge. (the rate is excluding cost of tiles which will be paid separately). | 190.00 | Sqm |  |  |  |
|  | Applying priming coat: 13.50.3 With ready mixed red oxide zinc chromate primer of approved brand and manufacture on steel galvanized iron/steel works. | 6.00 | Sqm |  |  |  |
|  | Wall painting with plastic emulsion paint of approved brand and manufacture of required colour to give an even shade: - Two or more coats on new work. | 475.70 | Sqm |  |  |  |
|  | Renewing glass panes, with putty and nails wherever necessary; - Float glass panes of thickness 4mm. | 4.32 | Sqm |  |  |  |
|  | Providing and fixing white vitreous china pedestal type water closet (European type W.C pan) with seat lid, 10 liter low level whiter P.V.C. flushing cistern with manually controlled device (handle lever), conforming to IS:7231, with all fittings and fixtures complete including cutting and making good the walls and floors wherever required: - W.C pan with ISI marked white solid plastic seat and lid. | 3.00 | Each |  |  |  |
|  | Providing and fixing white vitreous china flat bank or wall corner type lipped front urinal basin of 430x260x350mm and 430x410x265mm sizes respectively with automatic flushing cistern with standard flush pipe and C.P brass spreaders with brass unions and G.I clamps complete including cutting and making good the walls and floors wherever required: Range of two urinal basins with 5 liter white P.V.C. automatic flushing cistern. | 1.00 | Each |  |  |  |
|  | Providing and fixing P.V.C. waste pipe for sink or wash basin including P.V.C. waste fittings complete. - Semi Regid pipe 32 mm dia. | 4.00 | Each |  |  |  |
|  | Providing and fixing mirror of superior (of approved quality) and of required shape and size with plastic moulded frame of approved make and shade with 6 mm thick hard board backing: - Rectangular shape 1500x450mm - Rectangular shape 453x357mm | 2.00 2.00 | Each Each |  |  |  |
|  | Providing and fixing soil, waste and vent pipes: - 100mm dia Sand cast iron S&S pipe as per IS: 1729 | 13.00 | Metre |  |  |  |
|  | Providing and fixing bend of required degree with access door, insertion rubber washer 3 mm thick, bolts and nuts complete. - 100mm Sand cast iron S&S pipe as per IS: 1729 | 6.00 | Each |  |  |  |
|  | Providing and fixing plain bend of required degree. - 100mm Sand cast iron S&S pipe as per IS: 1729 | 4.00 | Each |  |  |  |
|  | Providing lead caulked joints to sand cast iron/centrifugally cast (spun) iron pipes and fittings of diameter. 100mm | 10.00 | Each |  |  |  |
|  | Providing and fixing G.I pipes complete with G.I fitting and clamps, including cutting and making good the walls etc. Internal work-Exposed on wall. - 15mm dia. Nominal bore. | 10.00 | Metre |  |  |  |
|  | Providing and fixing G.I pipes complete with G.I fittings including trenching and refilling etc. External work a)15mm dia. Nominal bore b)20mm dia. Nominal bore | 5.00 20.00 | Metre Metre |  |  |  |
|  | Painting G.I pipes and fitting with two coats of anti-corrosive bitumastic paint of approved quality: - 15mm diameter pipe - 20mm diameter pipe | 15.00 20.00 | Meter Meter |  |  |  |
|  | Providing and filling sand of grading zone V or coarser grade all round the G.I pipes in external work. - 15mm diameter pipe - 20mm diameter pipe | 5.00 20.00 | Metre Metre |  |  |  |
|  | Providing and fixing G.I Union in G.I pipe including cutting and threading the pipe and making long screws etc. complete (New work) - 15mm nominal bore - 20mm nominal bore | 4.00 2.00 | Each Each |  |  |  |
|  | Providing and placing on terrace (at all floor levels) polyethylene water storage tank ISI: 12701 marked with cover and suitable locking arrangement and making necessary holes for inlet, outlet and overflow pipes but without fittings and base support for tank. | 1000.00 | Per liter |  |  |  |
|  | Providing and fixing C.P brass long body bib cock of approved quality conforming to IS standards and weighing not less than 690 Gms. 15mm nominal bore | 6.00 | Each |  |  |  |
|  | Providing and fixing C.P brass stop cock (concealed) of standard design and of approved make conforming to IS:8931. | 8.00 | Each |  |  |  |
|  | Providing and fixing aluminium work for doors, windows ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conformaing to IS:733 and IS:1285, fixed with rawl plugs and screws or with fixing clips, or with expansion hold fasteners including necessary filling up of gaps at junctions, at top, bottom and sides with required PVC/neoprene felt etc. Aluminium sections shall be smooth, rust free, straight, mitered and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling C.P brass/ stainless steel screws, all complete as per architectural drawings and the direction of Engineer-in-charge. (Glazing and paneling to be paid for separately): a) For shutters of doors, windows and ventilators including providing and fixing hinges/pivots and making provision for fixing of fittings wherever required including the cost of PVC/neoprene gasket required(Fittings shall be paid for separately) i) Anodised aluminium (anodised transparent or dyed to required shade according to IS:1868, Minimum anodic coating of grade AC 15). | 100.00 | Kilogram |  |  |  |
|  | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with PVC/ neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge. (cost of aluminium snap beading shall be paid in basic item): With float glass of 4.0 mm thickness | 8.64 | Sqm |  |  |  |
|  | Providing and laying four courses water proofing treatment with bitumen felt over roofs consisting of first and third courses of blown bitumen 85/25 or 90/15 conforming to IS:702 applied hot @ 1.45Kg per square meter of area for each course, second course of roofing felt type 3 grade-1 (Hessian based self finished bitumen felt ) and fourth and final course of stone grit 6mm and down size or pea-sized gravel spread at 6 cubic diameter per square meter including preparation of surface but excluding grading complete with: Bitumen felt (Hessian base) type 3 grade I conforming to IS: 1322. | 319.65 | Sqm |  |  |  |
|  | Providing and fixing 6 mm dia. G.I. level adjusting hangers (upto 1200mm lengh ) fixed to roof slabs by means of ceiling cleats made out of G.I flat 40x3mm size 60mm long and expansion hold fasteners 12.5 mm dia. 40mm long complete as per direction of Engineer-in-charge. | 150.00 | Each |  |  |  |
|  | Finishing walls with Acrylic smooth exterior paint of required shade: - New work (Two or more coat applied @1.43 lit/10sqm. Over and including base coat of water proofing cement paint applied @2.20 kg/10 sqm.) | 319.00 | Sqm. |  |  |  |
|  | Providing and lying matt finish (anti skid) joint less ceramic floor tiles 400X400X thickness to be specified by the engineer in charge of first quality of Kajaria, Nitco, Orient make in color shade laid 20mm thick cement mortar 1:4 (1 cement: 4 coarse sand) including pointing the joint with white cement and matching pigment etc. all completer as per direction of engineering charge. | 51.00 | Sqm |  |  |  |
|  | Providing and Fixing 1st quality ceramic glazed wall tiles conforming to IS: 15622( thickness to be specified by the manufacture ) of approved make in all colours, shades except burgundy, bottle green, black of any size as approved by Engineer-in-Charge in skirting, risers of steps and dados over 12mm thick bed of cement Mortar 1:3(1 cement : 3 coarse sand ) and jointing with grey cement slurry @ 3.3Kg per sqm including pointing in white cement mixed with pigment of matching shade complete. | 56.00 | Sqm |  |  |  |
| 59. | Providing and fixing PVC coating gypsum ceiling tiles of approved quality size 595X595mm with aluminium backing with necessary nails screw etc. complete. (Frame work shall be paid separately). - White face insulating tile 7mm thick. | 190.00 | Sqm. |  |  |  |

| 60. | Deduct for cost of the work material got from dismantling at the time of execution of work like steel work, doors, windows, shutter, false ceiling, pipes, china ware, tiles etc.. | 1 Job | Each Job |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

NOTE: 1.  3500 Nos. of F.P.S. bricks shall be supplied by the Department @ Rs.1900/- per 1000

2.  Cement – 4.5 M.T. @ Rs.5,175/- supplied by the D.D.A. Store.

Sd/-					    Sd/-						  Sd/-

D/Man 				        A.E(P)/C.G.D.-2		                            E.E./C.G.D.-2/D.D.A.