Schedule of Quantity

NOW:- M/o RSKP, Pitampura.

SH:- Providing & fixing two nos. Shed near cricket ground at RSKP

| S.No. | Description of Items | Quantity | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavating holes upto 0.5cum i/c. getting out the excavated soil, then returning the soil as required in layers not exceeding 20c.m. in depth i/c. consolidating each deposited layer by ramming, watering etc depositing & surplus excavated soil as directed within a lead of 50mt. & lift upto 1.5mtr. - All kinds of soil | 10 nos. | Each |  |  |
| 2. | Providing and laying in position cement concrete of specified grade excluding the cost of centering & shuttering. All work upto plinth level. - 1:2:4 (1 cement : 2 c.sand : 4 graded stone agg 20mm n/s) | 1.25 M3 | M3 |  |  |
| 3. | Steel work in built up tabular trusses i/c, cutting hoisting fixing in position and applying a priming coat of approved steel primer, welded and bolted i/c. special shaped washers etc complete. - Hot finished seamless type tubes | 700.00 kg | Kg |  |  |
| 4. | Painting with synthetic enamel paint of approved brand and manufacturer to give an even shade. - Two or more coats on new works | 25.00 M2 | M2 |  |  |
| 5. | Supply & installation of precoated galvanized iron profile sheets( size shape and pitch of corrugative as approved by Engineer-in-charge) 0.50mm ± 5% total coated thickness(TCT) thick zinc coating 120gsm as per IS:277 in 240mpa sheet grade 5-7 microns epoxy primer on both side of the sheet and polyester top coat 15-18 microns sheet should have protective grade film of 25microns minimum to avoid scratches while transportation and should be supplied in single length up to 12mt. or as desired by Engineer-in-charge. The sheet shall be fixed using self drilling/ self topping screws of size (5.5 x55mm) with EPDM seal or with polymer coated J or L hooks, bolts and nuts 8mm diameter with bituman and G.I. limpet washers or with G.I. limpet washers filled with white lead complete upto any pitch in horizontal/ vertical or curved surface excluding the cost of purlins , rafters and trusses and i/c. cutting the size & shape wherever required. | 47.00 M2 | M2 |  |  |
| Total |  |  |  |  |  |